3GPP TSG-SA5 Meeting #160	S5-251696

Goteborg, Sweden, 07 - 11 April 2025

# Source:	Nokia

# Title:	Discussion paper on MSED components, requirements and naming

# Document for:	Endorsement

Agenda Item:	6.19.21.1

# 1	Decision/action requested

The group is asked to endorse the proposal.

# 2	References

[1] 3GPP TS 28.579, "Management services exposure to external consumers through CAPIF".

# 3	Rationale

## 3.1 Background

In clause 5.1.2 of TS 28.579[1], MSED has been defined as the API provider domain for exposing management services using CAPIF (see Figure 3.1-1). MSED consists of the API provider domain functions (i.e., the API Exposing Function (AEF), API management function (AMF) and the API publishing function (APF)).

Figure 3.1-1: MSED interactions with the CCF and the API invokers (i.e., the external MnS consumers).

Editor's Note: The possible deployment options for MSED are FFS

Editor's Note: The name MSED is still under discussion, and it is FFS whether it will be MSED/MSEDF or another term

The following requirements traceable to MSED have been defined in clause 5.3 of TS 28.579[1]:

REQ-MEXPO-FUN-01: MSED shall have the capability to provide the API provider domain functions as defined by CAPIF.

Editor’s Note: This requirement can be revised as the requirement shall be on 3GPP Management System.

REQ-MEXPO-FUN-02: 3GPP management system shall have the capability to provide the required mappings between the management service information and the CAPIF-defined information elements to enable the exposure of management services to external MnS consumers using CAPIF.

## 3.2 Observations

### 3.2.1. MSED Requirements/ Components

From Figure 3.1-1, the following observations can be made:

Observation 1: From the network operator’s perspective, when using CAPIF to expose APIs to internal or external API invokers, the following options are possible:

- For each network domain (e.g., Core, Management, RAN), they have dedicated API provider domain entities (e.g., MSED for OAM and NEF for core etc).

- Some of the API provider domain functions could be shared between the network domains (e.g., the AMF of MSED could be shared with other network domains). This is up to the network operator’s discretion.

- The AEF of MSED shall reside within the management system since this entity acts as a routing gateway between the external MnS consumers and the MnS producers and is responsible for the “actual” exposure of the management services (as service APIs using CAPIF) to external MnS consumers.

- The APF of MSED shall reside within the management system since this entity is responsible for the publishing and unpublishing of the management services to the CCF. This entity is also responsible for updating the published MnS information at the CCF. All this publishing information is only available within the 3GPP management system and nowhere else, hence this component shall be implemented by the management system.

Observation 2: If observation 1 is valid, then which of the following normative requirements accommodates all possible MSED deployment options:

Option-1

3GPP management system shall have the capability to provide the API provider domain functions (i.e., AEF, APF and AMF) as defined by CAPIF.

Option-2

3GPP management system shall have the capability to provide the AEF functionality

3GPP management system shall have the capability to provide the APF functionality

3GPP management system should have the capability to provide the AMF functionality

### 3.2.2 MSED vs MSEDF naming convention

The name MSED stands for Management Services Exposure Domain, which represents a logical grouping of the CAPIF API provider domain functions (i.e., AEF, AMF and APF) to enable the exposure of MnSs using CAPIF.

Accordingly, if MSED is composed of logical functions, then by deduction doesn’t this imply that MSED is not just a domain but a function?  So, should we rename it into Management Services Exposure Domain Function (MSEDF)?

# 4	Detailed proposal

The group is asked to do the following:

- Discuss the two options in clause 3.2.1.

- The group is asked to endorse either option-1 or option-2 in clause 3.2.1 to resolve the editor’s note on the MSED requirements (clause 5.3 of TS 28.579[1]) and set the pace for the definition of the normative requirements.

- Regarding clause 3.2.2, the group is asked to discuss and agree on a single name for MSED, i.e., either MSED or MSEDF to resolve the editor’s note in clause 5.1.2 of TS 28.579[1].