3GPP TSG-SA5 Meeting #160	S5-251698

Goteborg, Sweden, 07 - 11 April 2025

# Source:	Nokia

# Title:	Rel-19 pCR TS 28.579 Add use case solution for registration of the MSED into the CCF

# Document for:	Approval

Agenda Item:	6.19.21.1

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 3GPP TS 28.579, "Management services exposure to external consumers through CAPIF".

# 3	Rationale

This pCR proposes to add use case solution for registration of the MSED into the CCF.

# 4	Detailed proposal

It is proposed to add use case solution for registration of the MSED into the CCF to TS 28.579[1].

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]                       3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[Z]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; stage 3".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

Second Change

# 6 Solutions for exposing management services using CAPIF

## 6.Z Registration of the MSED into the CCF

### 6.Z.1 Introduction

In CAPIF, the registration of MSED as a recognized API provider domain in CAPIF is initiated by the AMF of the MSED. To register, unregister or update MSED, the AMF of MSED invokes the CAPIF_API_Provider_Management API service (see clause 5.11 of TS 29.222 [X]) over the CAPIF-5 interface. As part of the registration process (i.e., registration and update registration), the AMF of the MSED sends HTTP requests to the CCF whose request body includes the APIProviderEnrolmentDetails data type (see clause 8.9.4.2.2 of TS 29.222 [Z]).

Clause 6.Z.2 describes how to map MSED registration information to the APIProviderEnrolmentDetails data type and clause 6.Z.3 indicates the service operations that need to be supported by the AMF of the MSED to enable the registration procedures (i.e., registration, update registration and deregistration) of MSED.

### 6.Z.2 Mapping of MSED information into APIProviderEnrolmentDetails

Table 6.Z.2-1 presents the attributes of the APIProviderEnrolmentDetails data type (see clause 8.9.4.2.2 of TS 29.222 [Z]) and provides information on how this data type can be populated with the MSED registration information. Refer to Table 8.9.4.2.2-1 of TS 29.222 [Z] for detailed information on the attributes of APIProviderEnrolmentDetails data type (e.g., the attribute data type, presence indicator, cardinality, description and applicability information).

Table 6.Z.2-1: Mapping of MSED registration information to APIProviderEnrolmentDetails data type attributes

| Attribute name | Attribute additional information | Related MSED registration information/Comments |
| --- | --- | --- |
| apiProvDomId | The data type of this attribute is defined as "string" and presence qualifier is defined as "O". | Assigned by the CCF to the AMF of the MSED during the registration procedure of the MSED at the CCF (see clause 6.Z.3). |
| regSec | The data type of this attribute is defined as "string" and presence qualifier is defined as "M". | It can be used to store the security credentials of the MSED. |
| apiProvFuncs | The data type of this attribute is defined as "array (APIProviderFunctionDetails)" and presence qualifier is defined as "O". | See Table 6.Z.2-2. |
| apiProvDomInfo | The data type of this attribute is defined as "string" and presence qualifier is defined as "O". |  |
| suppFeat | The data type of this attribute is defined as "SupportedFeatures" and presence qualifier is defined as "C". | Only applicable when API provider is SCEF/NEF, and thus not applicable when the API provider is MSED. |
| failReason | The data type of this attribute is defined as "string" and presence qualifier is defined as "C". |  |
| apiProvName | The data type of this attribute is defined as "string" and presence qualifier is defined as "O". | Applicable in RNAA scenarios. |

Table 6.Z.2-2 presents the attributes of the APIProviderFunctionDetails data type (see clause 8.9.4.2.3 of TS 29.222 [Z]) and provides information on how this data type can be populated with the MSED related information. See table 8.9.4.2.3-1 of TS 29.222 [Z] for detailed information on the attributes of APIProviderFunctionDetails data type (e.g., the attribute data type, presence indicator, cardinality, description and applicability information).

Table 6.Z.2-2: Mapping MSED registration information to APIProviderFunctionDetails attributes

| Attribute name | Attribute additional information | Related MSED registration information/Comments |
| --- | --- | --- |
| apiProvFuncId | The data type of this attribute is defined as "string" and presence qualifier is defined as "C". | The API provider function id is assigned to each function composing the MSED (i.e., AEF, APF or the AMF) by the CCF as part of the MSED registration procedure. The assigned ids of the API provider domain functions are used in the use case clauses 5.2.1 and 5.2.2. |
| regInfo | The data type of this attribute is defined as "RegistrationInformation" and presence qualifier is defined as "M". | See Table 6.Z.2-3. |
| apiProvFuncRole | The data type of this attribute is defined as "ApiProviderFuncRole" and presence qualifier is defined as "M". | This data type serves to specify, for the MSED to be registered, which CAPIF API provider domain function(s) will be supported. Allowed values are AEF, AMF, APF. |
| apiProvFuncInfo | See clause 8.9.4.3.3 of TS 29.222 [Z] for the enumeration values and description of type ApiProviderFuncRole. | General information related to the specific MSED function (e.g., AEF, AMF or APF) |

Table 6.Z.2-3 presents the attributes of the RegistrationInformation data type (see clause 8.9.4.2.4 of TS 29.222 [Z]) and provides information on how this data type can be populated with the MSED related information. See table 8.9.4.2.4-1 of TS 29.222 [Z] for detailed information on the attributes of RegistrationInformation data type (e.g., the attribute data type, presence indicator, cardinality, description and applicability information).

Table 6.Z.2-3: Representing MSED registration information with RegistrationInformation attributes

| Attribute name | Attribute additional information | Related MSED registration information/Comments |
| --- | --- | --- |
| apiProvPubKey | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.9.4.2.4-1 of TS 29.222 [Z]). | It can be used to store the public key of the MSED. |
| apiProvCert | The data type of this attribute is defined as "string" and presence qualifier is defined as "O" (see table 8.9.4.2.4-1 of TS 29.222 [Z]). | It can be used to store the client certificate of the MSED, if existing. |

### 6.Z.3 Service operations supported by the AMF of the MSED to enable the MSED registration functionality

To register MSED as the API provider domain for management services at the CCF, the AMF of the MSED invokes the Register_API_Provider service operation as described in clause 5.11.2.2 of TS 29.222 [Z] via the CAPIF-5 interface. The body of the HTTP POST request shall include the API provider enrolment details as described in clause 6.Z.2.

To update the registration details of MSED at the CCF, the AMF of the MSED invokes the Update_API_Provider service operation as described in clause 5.11.2.3 of TS 29.222 [Z] via the CAPIF-5 interface. The body of the HTTP PUT request shall contain the updated API provider enrolment details as described in clause 6.Z.2.

To unregister MSED as the API provider domain for management services from the CCF, the AMF of the MSED invokes the Deregister_API_Provider service operation as described in clause 5.11.2.4 of TS 29.222 [Z] via the CAPIF-5 interface.

End of Changes