3GPP TSG-SA5 Meeting #160	S5-251699

Goteborg, Sweden, 07 - 11 April 2025

# Source:	Nokia

# Title:	Rel-19 pCR TS 28.579 Add use case requirements for publishing of management services to the CCF

# Document for:	Approval

Agenda Item:	6.19.21.1

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 3GPP TS 28.579, "Management services exposure to external consumers through CAPIF".

# 3	Rationale

This pCR proposes to add use case requirements for publishing management services to the CCF.

# 4	Detailed proposal

It is proposed to add the use case requirements for publishing management services to the CCF to TS 28.579[1].

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]                       3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[X]	3GPP TS 28.622: “Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)”.

[Y]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; stage 3".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

Second Change

### 5.2.1 Publishing of management services into the CCF

#### 5.2.1.1 Definition

This use case describes how to publish management services to the CAPIF core function (CCF) to be exposed to the external MnS consumers. Before publishing management services to the CCF, the operator shall determine, subject to the network operator policy, regulatory requirements or contractual obligations the following:

- The management services to be published

- The specific set of resources and corresponding operations of the management services to be published

- Whether the specific set of resources and corresponding operations of the management service will be published as one or more service APIs to the CCF.

- Optionally, the performance and fault data associated to the management service (referred to as MnS component C) to be published.

Traceability: REQ-MEXPO-PUB-01, REQ-MEXPO-PUB-02, REQ-MEXPO-PUB-03, REQ-MEXPO-PUB-04, REQ-MEXPO-PUB-05.

## 5.3 Requirements

REQ-MEXPO-FUN-01: MSED shall have the capability to provide the API provider domain functions as defined by CAPIF.

Editor’s Note: This requirement can be revised as the requirement shall be on 3GPP Management System.

REQ-MEXPO-FUN-02: 3GPP management system shall have the capability to provide the required mappings between the management service information and the CAPIF-defined information elements to enable the exposure of management services to external MnS consumers using CAPIF.

REQ-MEXPO-PUB-01: The 3GPP management system shall have the capability to map the management services (described by the MnSInfo IOC (see clause 4.3.42 of TS 28.622[X])) into the service APIs (described by the CAPIF ServiceAPIDescription information element (see clause 8.2.4.2 of TS 29.222[Y])).

REQ-MEXPO-PUB-02: The 3GPP management system shall have the capability to publish the service APIs to the CCF.

REQ-MEXPO-PUB-03: The 3GPP management system shall have the capability to retrieve the published service APIs at the CCF.

REQ-MEXPO-PUB-04: The 3GPP management system shall have the capability to update the published service APIs at the CCF.

REQ-MEXPO-PUB-05: The 3GPP management system shall have the capability to unpublish the service APIs from the CCF.

End of Changes