3GPP TSG-SA5 Meeting #160	S5-251704

Goteborg, Sweden, 07 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.156 | CR | 0108 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel16 CR TS 32.156 Enhancement of the description of the “isNotifyable” attribute property and correction of the default value for the “isReadable” attribute property |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2025-03-25 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The current description of the “isNotifyable” attribute property states that “It identifies if a notification shall be sent incase of a value change” which narrows the scope of this attribute property. - The default value for the “isReadable” attribute property is “False” |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Enhancement of the description of the “isNotifyable” attribute property to include all the scenarios in which this attribute property would be applicable. - Changing the default value for the “isReadable” attribute property to “True” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Leads to incoherrent specifications regarding the description of the “isNotifyable” attribute property vs it’s applicability - Inconsistencies in the default value for the “isReadable” attribute property |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 5.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Begin Change

### 5.2.1	Attribute

#### 5.2.1.1	Description

It is a typed element representing a property of a class. See 10.2.5 Property of [1].

An element that is typed implies that the element can only refer to a constrained set of values.

See 10.1.4 Type of [1] for more information on type.

See 5.3.4 and 5.4.3 for predefined data types and user-defined data types that can apply type information to an element.

The following table captures the properties of this modelled element.

Table 5.2.1.1-1: Attribute properties

| Property name | Description | Legal values |
| --- | --- | --- |
| documentation | Contains a textual description of the attribute. Should refer (to enable traceability) to the specific requirement. | Any |
| isOrdered | For a multi-valued multiplicity; this specifies if the values of this attribute instance are sequentially ordered. See subclause 7.3.44 and its Table 7.1 of [2]. If the property is present for attributes with a multiplicity of greater than “1”, it shall be set to either “True” or “False”. It shall not be set to “N/A”. | True, False (default) |
| isUnique | For a multi-valued multiplicity, this specifies if the values of this attribute instance are unique (i.e., no duplicate attribute values). See subclause 7.3.44 and its Table 7.1 of [2]. If the property is present for attributes with a multiplicity of greater than “1”, it shall be set to either “True” or “False”. It shall not be set to “N/A”. | True (default), False |
| isReadable | Specifies that this attribute can be read by the manager. | True (default), False (see NOTE 3) |
| isWritable | Specifies that this attribute can be written by the manager under the conditions specified in Annex B. | True, False (default) (see NOTE 4) |
| type | Refers to a predefined (see subclause 5.4.3) or user defined data type (see section 5.3.4). See also subclause 7.3.44 of [2], inherited from StructuralFeature. | NA |
| isInvariant | Attribute value is set at object creation time and cannot be changed under the conditions specified in Annex B. | True, False (default) |
| allowedValues | Identifies the values the attribute can have. | Dependent on type |
| isNotifyable | Identifies if the creation of the attribute value pair (which may or may not occur as part of an object creation), deletion of the attribute value pair (which may or may not occur as part of an object deletion) or attribute value change shall be notified (see Note 1, Note 2). | True (default), False |
| defaultValue | Identifies a value at specification time that is used at object creation time under conditions defined in Annex B. If there is no defined default value, the property shall be omitted from the attribute description or specified as ‘defaultValue: None.’. | No value (default) or a value that is dependent on allowedValues |
| multiplicity | Defines the number of values the attribute can simultaneously have. See subclause 7.3.44 of [2]; inherited from StructuralFeature. | See 5.2.8 Default is 1 |
| isNullable | Identifies if an attribute can carry no information. The implied meaning of carrying “no information” is context sensitive and is not defined in this Model Repertoire. | True, False (default) |
| supportQualifier | Identifies the required support of the attribute. See also subclause 6. | M, O (default), CM, CO, C |
| passedById | See Table 5.2.9.1-1: passedById property | True, False (default) |

NOTE 1: Whether a client/manager can receive the notification depends on a) if the client/manager has subscribed or registered for reception of such notification and b) if a notification mechanism is supported.

NOTE 2: If the attribute is a role-attribute and its property passedById is ‘False’, then changes in the navigable association target end instance do not trigger a notification.

NOTE 3: The value True is conditional on the presence of a mechanism for the MnS Producer to authenticate the MnS Consumer and verify whether the authenticated MnS Consumer is authorized to read.

NOTE 4: The value True is conditional on the presence of a mechanism for the MnS Producer to authenticate the MnS Consumer and verify whether the authenticated MnS Consumer is authorized to write.

#### 5.2.1.2	Example

This example shows three attributes, i.e., a, b and c, listed in the attribute (the second) compartment of the class Xyz.

Figure 5.2.1.2-1: Attribute notation

#### 5.2.1.3	Name style

An attribute name shall use the LCC style.

Well Known Abbreviation (WKA) is treated as a word if used in a name. However, WKA shall be used as is (its letter case cannot be changed) except when it is the first word of a name; and if so, its first letter must be in lower case.

End Change