3GPP TSG-SA5 Meeting #160	S5-251716

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.318 | CR | 0013 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests... |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.318 Fix affectedArea attribute for DSO service outage use case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson GmbH, Eurolab |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NSOEU |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | OutageAndRecoveryInfo IOC is defined to support the following NSOEU use case: “exposing performance and prediction information for high availability”. The IOC includes affectedArea attribute. Attribute description (clause 6.3.1.1, 6.4.1): affectedArea is used by the DSO to inform the MNO about the area/location of the service outage. The attribute allows defining the regions/locations where the DSO subsystems affected by the service outage are situated. Attribute solution (6.3.2): affectedArea is a datatype (EnergyServiceLocation <<dataType>>) consisting of the following: - CGI attribute (type: NrCellId), which provides the affected area by either a geographical area or a list of cell ids of the different RAN technologies. - energySupplyId attribute (type: String), which identifies the MNO facility that is using the DSO energy services. There is a mismatch between the description and the solution for affectedArea. The reason for this mismatch is the usage of CGI attribute in the solution, wherein various issues have identified: - The defined type, NrCellId (i.e. local identifier of a cell in NR ), does not match neither the definition of the CGI attribute in TS 28.318 (i.e. affected area by either a geographical area or a list of cells ids of different RAN technologies) nor the description of affectedArea (i.e. area/location of the service outage). The mistmatches are underlined. - The attribute name, CGI, does not match the description of affectedArea. CGI is used to identify a cell globally, and is constructed as follows: {PLMN ID, CI}, where CI is a local cell identifier (cell identity within the PLMN). This definition, extracted from TS 38.300 and TS 29.571, does not match the definition of CGI attribute in TS 28.318, causing confusing. Additionally, it also assumes that DSO must always know the CIs from the MNO, which might not be always true, as one of the objectives of the specification is to abstract (or hide) low-level (or sensitive) information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For affectedArea, it is proposed to align the solution (EnergyServiceLocation <<dataType>>) with the description, by doing the following changes on the CGI attribute: - Attribute name, changing to “outageArea”, to make it aligned with the semantics of affectedArea. - Attribute type, changing to “AreaOfInterest” to make it aligned with the semantics of affectedArea and knowledge expectations on DSO consumer side. Indeed, “AreaOfInterest” data type is already used in the other NSOEU use case (“exposed coordinated recovery requirements) for DSO to inform the MNO about areas/locations, so it makes sense to use here as well. - Attribute definition, incorporating TAI list as another possibility for DSO to specify areas/locations, according to “AreaOfInterest” specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mismatch between affectedArea description and solution, leading to inconsistency in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.1, 6.3.2.2, 6.4.1, Forge |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1679 at commit d3ba70debccd3f7d5f2d0892f34acab96762dd02 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 6.3.2	EnergyServiceLocation  <<datatype>>

#### 6.3.2.1	Definition

This <<datatype>> defines the area/location of the service outage.

This is defined as outageArea and/or energySupplyId. This defines the regions/locations where the DSO subsystems are situated.

The attribute outageArea is of type AreaOfInterest , which provides the affected area by either a geographical area or a list of TAIs or a list of cell ids of the different RAN technologies.

The energySupplyId identifies the MNO facility that is using the DSO energy services. The energySupplyId is known between both the parties using out-of-band mechanisms hence not subject to standardisation.

Thise energySupplyId, used extensively in this class definition is known to both the MNO and DSO and is the means by which information is organized, essentially the 'key identifier' used to exchange information about specific sites, which is essential for most Rapid Recovery actions (see Annex C.2) and Rapid Intervention actions (see Annex C.3).

#### 6.3.2.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| outageArea | M | T | T | F | T |
| energySupplyId | M | T | T | F | T |

#### 6.3.2.3	Attribute constraints

None.

#### 6.3.2.4	Notifications

The subclause 6.3.1.4 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

Next change

## 6.4	Attribute definitions

### 6.4.1	Attribute properties

The following table defines the properties of attributes specified in the present document in addition to those existing in clause 4.4.1 of [20].

| affectedArea | It defines the area/location of the DSO service outage. These are the regions/locations where DSO subsystems are situated. It is an aggregation of: • outageArea, which provides the affected area by either a geographical area or a list of TAIs or a list of cell ids. • energySupplyId, which identifies the MNO facility that is using DSO energy services. The energySupplyId is communicated between both the parties using out-of-band mechanisms hence not subject to standardisation. | Type: EnergyServiceLocation multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| --- | --- | --- |
| outageStartTime | It informs about the time when the outage of the DSO energy service has occurred at the location provided by the attribute affectedArea. Along with outage location and expected recovery time this would be beneficial for MNO to plan and prepare its UPS power backup at different site locations for its crucial communication services and also for assisting DSO for rapid intervention. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectedOutageEndTime | It informs about the time when the service is expected to be restored through manual dispatch and processes. In case the end of the outage cannot be estimated, the field is not filled. This is usually a longer time period that may not be acceptable for some crucial public services like hospitals. | type: DateTime multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| dsoRapidInterventionTime | It defines the time at which DSO restores its Energy feeder service. This is the time from when the DSO expects rapid intervention to start. From this time a DSO can use its automated smart energy services e.g SCADA to recover its distribution services much quicker in substations instead of manual processes. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dsoRapidInterventionDuration | Time interval for which the DSO requires rapid intervention services from the MNO to restore its energy distribution services rapidly. The time interval starts with dsoRapidInterventionTime. This time duration is defined in minutes. allowedValues: Integer with a minimum value of 1 | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isAffectedAreaPriority | It indicates whether the location of DSO substation needs recovery on priority by using smart energy services or not. This can be used by MNO to ascertain the gNBs at which the UPS power needs to be reserved as part of MNO Rapid Intervention. The default value is "False". allowedValues: True, False | Type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| mnoInterventionTime | It defines the time at which a MNO will actually be able to provide dedicated communication services to DSO (as rapid intervention) depending on its UPS backup status, other critical service venues etc. DSO needs this to use its smart energy services like SCADA for recovering its energy distribution services. | type: DateTime multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| mnoInterventionDuration | It defines the time duration for which a MNO will actually be able to provide dedicated communication services to DSO (as rapid intervention) depending on its UPS backup status, other critical service venues etc. The interval starts with mnoInterventiontime. This time duration is defined in minutes. allowedValues: Integer with a minimum value of 1 | Type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| actualOutageEndTime | It defines the time at which DSO finally restores its energy distribution services rapidly by using rapid intervention. This time is much smaller than expectedOutageEndTime. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| outageArea | This attribute provides the affected area by either a geographical area or a list of TAIs or a list of cell ids of the different RAN technologies. | type: AreaOfInterest multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| energySupplyId | energySupplyId identifies the MNO facility (e.g. base station) using DSO energy services. The energySupplyId is communicated between both the parties using out-of-band mechanisms hence not subject to standardisation. A meter with a unique energySupplyId is installed by DSO in the MNO site. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mnoMaxServiceDuration | It defines the remaining time duration for which MNO site can operate on its UPS backup at a given time i.e. capacity of UPS backup in terms of time duration (e.g. minutes) for which it can provide power supply to MNO site from a given time. It is defined in minutes. allowedValues: Integer with a minimum value of 1 [NOTE1] | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| targetThresholdLocation | This attribute specifies the location of the object instances that are to be monitored. | Type: AreaOfInterest multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NOTE 1: The value of mnoMaxServiceDuration can be taken from IETF RFC 1628 [11], upsEstimatedMinutesRemaining. ETSI ES 202 336-11 [12] defines a field "Estimated remaining battery autonomy (time) during discharge" as provided by the monitoring and control interface for battery system with integrated control and monitoring information model, however there is no corresponding stage 3 definition of this value. |  |  |

Next change

*** OpenAPI/TS28318_DsoNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: 3GPP NRM

version: 18.1.0

description: >-

OAS 3.0.1 specification of the OutageAndRecoveryInfo NRM

© 2025, 3GPP Organizational OutageAndRecoveryInfo Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.318; OutageAndRecoveryInfo NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.318/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

EnergyServiceLocation:

type: object

properties:

outageArea:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AreaOfInterest'

energySupplyId:

type: string

#-------- Definition of concrete IOCs --------------------------------------------

SubNetwork-ncO-DsoNrm:

type: object

properties:

OutageAndRecoveryInfo:

$ref: '#/components/schemas/OutageAndRecoveryInfo-Multiple'

DsoThresholdMonitor:

$ref: '#/components/schemas/DsoThresholdMonitor-Multiple'

OutageAndRecoveryInfo-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

outageStartTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

expectedOutageEndTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

dsoRapidInterventionTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

dsoRapidInterventionDuration:

type: integer

mnoInterventionTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

mnoInterventionDuration:

type: integer

mnoMaxServiceDuration:

type: integer

affectedArea:

$ref: '#/components/schemas/EnergyServiceLocation'

isAffectedAreaPriority:

type: boolean

actualOutageEndTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

DsoThresholdMonitor-Single:

allOf:

- $ref: 'TS28623_ThresholdMonitorNrm.yaml#/components/schemas/ThresholdMonitor-Single'

- type: object

properties:

targetThresholdLocation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AreaOfInterest'

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

OutageAndRecoveryInfo-Multiple:

type: array

items:

$ref: '#/components/schemas/OutageAndRecoveryInfo-Single'

DsoThresholdMonitor-Multiple:

type: array

items:

$ref: '#/components/schemas/DsoThresholdMonitor-Single'

#--------------------------------- Definition ------------------------------------

resources-DSORecovery:

oneOf:

- $ref: '#/components/schemas/OutageAndRecoveryInfo-Single'

- $ref: '#/components/schemas/DsoThresholdMonitor-Single'

<CODE ENDS>

End of Changes