3GPP TSG-SA5 Meeting #160	S5-251738

Goteborg, Sweden, 07 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0696 | rev | - | Current version: | 18.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 Corrections on Filter Definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To make the filter definition clear to avoid confusion with the sub-counter |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct filter definition |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect implementation due to the incorrect term |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 4.2	Filters and filter naming

### 4.2.0	General

In case a performance measurement is applicable and can be measured for a sub-group(e.g. SNSSAI, QCI, 5QI, PLMN etc…), it is convenient to use Filter to define the sub-group for which the performance measurement shall be supported.

### 4.2.1	Filters

Performance measurements may be sub-divided by use of applicable filters to form new Performance measurements (or sub counters). Any applicable Filter(s) are identified in each performance measurements definition. Performance measurements may also be defined without any applied Filter.

When no Filter is applied the performance measurement should exclude the Filter extension.

Example of possible Filters and potential values:

-	5QI: value can be any supported 5QI value assigned at run-time

-	QCI: value can be any supported QCI value assigned at run-time

-	SNSSAI, where SNSSAI represents the S-NSSAI: value can be any supported SNSSAI

-	PLMN, where PLMN represents the PLMN ID: value can be any supported PLMN

### 4.2.2	Filter naming

For the Performance measurements that indicate Filters, the resulting Performance measurement name is in the following form:

<Performance measurement>_<Filter>

For appending Filter, the separator '_' is used to append the filter to a given Performance measurement name. Vendor may also define any other separator.

If  appending multiple Filters, the name is in the following form:

<Performance measurement>_<Filter1>_<Filter2>

The separator ‘_’ or vendor specific separator is used between filters. The order is not important.

If no Filter is used, the name is in the form:

<Performance measurement>

The Filter is in the form: NameValue(s) where the name could be any of the possible Filter name defined for the performance measurement. When multiple values are provided for a given Filter name, values are separated by ‘|’.  Value ranges are defined as ‘x-y’. Combination of using ‘|’ and ranges for the values of a given Filter name is possible. The order of the values is not important. Any other vendor specific multiple value form is allowed for the combination.

Examples:

-	single Filter name with single value: DRB.PdcpF1DelayDl_5QI22

-	single Filter name with multiple values: DRB.PdcpF1DelayDl_5QI32|35-40

-	multiple (two) Filter names with single value per Filter name: DRB.PdcpF1DelayDl_PLMN12_5QI20

-	multiple (two) Filter names and multiple values per Filter name: DRB.PdcpF1DelayDl_PLMN2|5_5QI20-25

-	without any Filter: DRB.PdcpF1DelayDl

## 4.3	Measurement type naming

The measurement type is constructed as follows:

-	"<familyName>.<measurementName>.<Subcounter1>.<…>.<SubcounterM>_<Filter1>_…_<FilterN>" for measurement result(s) of a combination of Subcounters and Filters

-	"<familyName>.<measurementName>.<Subcounter1>.<…>.<SubcounterM>" for a measurement result of Subcounters only

-	"<familyName>.<measurementName>_<Filter1>_…_<FilterN>" for measurement result(s) of Filters only

-	"<familyName>.<measurementName>" for a measurement result without any Subcounter or Filter

-	"<familyName>" for a list of measurement results of all applicable measurementNames without any Subcounter or Filter

End of changes