3GPP TSG-SA5 Meeting #160	S5-251829

Goteborg, Sweden, 07 - 11 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.271 | CR | 0029 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Extend the scope and charging architecture to support 5G LCS converged charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CH_5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is no converged charging architecture for 5G LCS |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend the converged charging architecture to support 5G LCS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Converged charging for 5G LCS will not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

1	Scope

The present document is part of a series of Technical Specifications (TSs) that specify charging functionality and charging management in 3GPP networks. The 3GPP core network charging architecture and principles are specified in TS 32.240 [1], which provides an umbrella for other charging management documents that specify

-	the content of the CDRs per domain and subsystem (converged and offline charging);

-	the content of real-time charging events per domain / subsystem (converged charging);

-	the functionality of online and offline charging for those domains and subsystems;

-	the interfaces that are used in the charging framework to transfer the charging information (i.e. CDRs or charging events).

The complete document structure for these TSs is defined in TS 32.240 [1].

The present document specifies the LCS Offline, Online Charging and Converged Charging description for the LCS domain, based on the functional stage 2 description of the LCS in TS 23.271 [201], and the Ranging and Sidelink Positioning based on TS 23.586 [206]. This charging description includes the offline and online charging and converged charging architecture and scenarios specific to the LCS and the Ranging and Sidelink Positioning, as well as the mapping of the common 3GPP architecture specified in TS 32.240 [1] onto the LCS domain including Ranging and Sidelink Positioning. It further specifies the structure and content of the CDRs for offline charging and the charging events for online charging. The present document is related to other 3GPP charging TSs as follows:

-	The common 3GPP charging architecture is specified in TS 32.240 [1];

-	The parameters, abstract syntax and encoding rules for these CDR types are specified in TS 32.298 [51].

-	A transaction based mechanism for the transfer of CDRs within the network is specified in TS 32.295 [54].

-	The file based mechanism used to transfer the CDRs from the network to the operator’s billing domain (e.g. the billing system or a mediation device) is specified in TS 32.297 [52].

-	The 3GPP Diameter application that is used for LCS domain offline and online charging is specified in TS 32.299 [50].

-	The services, operations and procedures of charging, using Service Based Interface are specified in TS 32.290 [55].

-	The charging service of 5G system is specified in TS 32.291 [56].

All terms, definitions and abbreviations, used in the present document, that are common across 3GPP TSs, are defined in TR 21.905 [100]. Those that are common across charging management in GSM/UMTS domains, services, or subsystems are provided in the umbrella document TS 32.240 [1] and are copied into clause 3 of the present document for ease of reading. Finally, those items that are specific to the present document are defined exclusively in the present document.

Furthermore, requirements that govern the charging work are specified in TS 22.115 [101].

Next change

4.4	LCS converged charging architecture

The LCS converged charging architecture is depicted in figure 4.4.1 in service-based representation.

Figure 4.4.1: LCS Converged charging architecture

Figure 4.4.2 depicts the LCS converged charging architecture in reference point representation for non-roaming:

Figure 4.4.2: LCS converged charging architecture non-roaming reference point representation

End of changes