3GPP TSG-SA5 Meeting #160	S5-251953

Goteborg, Sweden, 07 - 11 April 2025

# Source:	Nokia

# Title:	Rel-19 pCR TS 28.572 Improve clause RESTful HTTP-based solution set

# Document for:	Approval, Information, Discussion

Agenda Item:	6.19.9.1 - Management of planned configurations

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]		3GPP TS 28.572: "Management of planned configurations"

# 3	Rationale

None.

# 4	Detailed proposal

The following changes are proposed for TS 28.572[1].

Begin of modifications

## A.1	RESTful HTTP-based solution set

Format of planned configurations

A planned configuration might target MnS producers whose data node tree is described using YANG or OpenAPI.

When described in OpenAPI, the value of the "configuration-content-type" is either "application/vnd.3gpp.json-patch+json" defined in TS 32.158 [2] or "application/json-patch+json".

When described in YANG, the value of the "configuration-content-type" is "application/yang-patch+json", defined in RFC 8072 [3].

A single planned configuration shall only include one configuration-content-type. If some items are modelled in YANG while others in OpenAPI these should be placed in separate planned configurations and made members of a plan configuration group.

Editor's note: Can the content types above be used given that a key is added for identifying an operation in the operation set? What is the content type for a restore configuration?

Editor's note: It is ffs if the key should be added only when we deal with YANG defined current configurations.

Note that JSON Patch is not used together with HTTP Patch. Therefore, the atomicity and synchronicity properties of HTTP Patch do not apply here.

CRUD operations

Editor's note: It is ffs if the CRUD operations defined in TS 32.158 are used or if dedicated CRUD operations are specified.

Format of condition expressions

Conditions expressions shall be expressed using Jex as defined in TS 32.161 [5].

Resource structure

The resource structure on the MnS producer is as follows:

…/{MnSName}/{/MnSVersion}/plan-descriptors/{id}

…/{MnSName}/{/MnSVersion}/plan-group-descriptors/{id}

…/{MnSName}/{/MnSVersion}/plan-validation-jobs/{id}

…/{MnSName}/{/MnSVersion}/plan-activation-jobs/{id}

Creating, updating and deleting planned configurations

The data node tree on the MnS producer is as follows upon system start up.

{ "mopc": { "plan-descriptors": {}, "plan-group-descriptors": {}, "fallback-config-descriptors": {}, "trigger-condition-descriptors": {}, "plan-validation-jobs": {}, "plan-activation-jobs": {} } }

A new item of the collection resource "plan-descriptors" is created by MnS consumers using HTTP POST.

POST 3gpp/ProvMnS/v1/plan-descriptors HTTP/1.1 Host: example.org Content-Type: application/json { "name": "NewBts10Plan", "version": "2.0", "description": "This is the plan for the new BTS 10.", "planConfigContentType": "application/vnd.3gpp.json-patch+json", "currentConfigAddress": "http://example.org/3gpp/ProvMnS/v1", "applyMode": "ATOMIC", "planConfig": [ { "key": "op1", "op": "add", "path": "/SubNetwork=SN1/ManagedElement=ME10", "value": {1 "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ] }

The MnS producer allocates the identifier "p1" for the new resource and returns the response. The location header contains the URI of the new resource. The response body contains the representation of the new resource which is equal to the representation received in the request with the "lastModifiedAt" and "validationState" properties added.

HTTP/1.1 201 Created Date: Wed, 21 Aug 2024 15:39:57 GMT Location: http://example.org/3gpp/ProvMnS/v1/plan-descriptors/p1 Content-Type: application/json { "name": "NewBts10Plan", "version": "2.0", "description": "This is the plan for the new BTS 10.", "planConfigContentType": "application/vnd.3gpp.json-patch+json", "currentConfigAddress": "http://example.org/3gpp/ProvMnS/v1", "applyMode": "ATOMIC", "lastModifiedAt": " 2025-03-06T16:50:26-08:00 ", "validationState": "NOT_VALIDATED", "planConfig": [ { "key": "op1", "op": "add", "path": "/SubNetwork=SN1/ManagedElement=ME10", "value": { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ] }

The resource structure on the MnS producer contains the new resource.

{ "plan-descriptors": { "p1": { "name": "NewBts10Plan", "version": "2.0", "description": "This is the plan for the new BTS 10.", "planConfigContentType": "application/vnd.3gpp.json-patch+json", "currentConfigAddress": "http://example.org/3gpp/ProvMnS/v1", "applyMode": "ATOMIC", "lastModifiedAt": " 2025-03-06T16:50:26-08:00 ", "validationState": "NOT_VALIDATED", "planConfig": [ { "key": "op1", "op": "add", "path": "/SubNetwork=SN1/ManagedElement=ME10", "value": { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ] } }, "plan-group-descriptors": {}, "fallback-config-descriptors": {}, "trigger-condition-descriptors": {}, "plan-validation-jobs": {}, "plan-activation-jobs": {} }

The next example shows how the value of the "location" attribute in the operation can be changed from "Castle Charlottenburg" to "Summer palace Charlottenburg".

PATCH 3gpp/ProvMnS/v1/plans/p1 HTTP/1.1 Host: example.org Content-Type: application/json-patch+json [ { "op": "replace", "path": "/planConfig/0/value/attributes/location", "value": "Summer palace Charlottenburg" } ]

To delete the planned configuration "p1" a MnS consumer might send.

DELETE 3gpp/ProvMnS/v1/plan-descriptors/p1 HTTP/1.1 Host: example.org

In case of success, the MnS producer returns the following message.

HTTP/1.1 204 No Content Date: Wed, 21 Aug 2024 16:12:45 GMT

The next example shows how a planned configuration can be created for the case where the current configuration data node tree is specified with YANG.

POST 3gpp/ProvMnS/v1/plan-descriptors HTTP/1.1 Host: example.org Content-Type: application/json Accept : application/json,application/problem+json { "name" : "planxyz", "activation-mode" : "BEST_EFFORT", # Execute in a best effort manner trying all operations "configuration-content-type" : "application/yang-patch+json" "configuration": { "ietf-yang-patch:yang-patch" : { "patch-id" : "NRCellDU", "edit" : [ { "operation": "create", "edit-id" : "opId-001", "target": "/_3gpp-common-subnetwork:SubNetwork=Irl/3gpp-common-mecontext:MeContext=Dublin-1/3gpp-common-managed-element:ManagedElement=Dublin-1/3gpp-nr-nrm-gnbdufunction:GNBDUFunction=1", "value": { "NRCellDU": [ { "id": "1", "attributes": { "typeISinglePanelRiRestriction": "15", "ailgDlPrbLoadLevel": "0", "drxOnDurationTimer": "ONDURATIONTIMER_8MS", "trsResourceShifting": "DEACTIVATED", "ulStartCrb": "0", ... } } ] } } } }

End of modifications