3GPP TSG-SA5 Meeting #160	S5-252021

Goteborg, Sweden, 07 - 11 April 2025

Source:	Huawei

Title:	Pseudo-CR on TS 28.561 Procedure for consuming NDT capability

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TS 28.561

Version:	0.1.0

Work Item:	NDT

Comments

Add a general procedure for consuming capability provided by NDT.

Proposed Changes

* * * First Change * * * *

6	Management service for NDT (stage 2)

Editor's note: this clause will specifiy the stage2 definition for NDT capabilities and services which may include the procedure, management operations and management information (e.g., NDT modelling).

6.X	Procedure for consuming NDT management service

Figure 6.X-1: Procedure of consuming NDT management service

1. NDT MnS producer (the entity who provides the NDT for network simulation/emulation) receives a request from MnS consumer to create a instance (see createMOI operation defined in TS 28.532 [3]) which represents service requirements for an NDT job. The NDT job can be a signalling storm replay, network configuration verification, etc.

2. NDT MnS producer initiates the NDT job according to the request from MnS consumer.

3. MnS Producer sends a response (see createMOI operation defined in TS 28.532[a]) to the MnS Consumer.

4. MnS producer receives a request from MnS consumer to start the execution of the NDT job in an NDT, which indicates that the NDT is activated to execute the NDT modelling task.

5. MnS producer executes the NDT job and create an NDT report instance which contains the execution result of NDT job provided by the NDT.

6. MnS producer sends the NDT report to MnS consumer to indicates the network simulation/emulation results.

* * * End of Change * * * *