3GPP TSG-SA5 Meeting #160	S5-251253

Goteborg, Sweden, 07 - 11 April 2025

Source:	Nokia, Ericsson

Title:	Pseudo-CR on NDT concept and scope

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TS28.561

Version:	0.1.0

Work Item:	Management aspects of Network Digital Twins

Comments

This pCR is to Add a description of NDT concept and scope

Proposed Changes

* * * First Change * * * *

# 4	Concepts and overview

## 4.X	Introduction and Overview

### 4.X.1	Digital Twins and Network Digital Twins

A digital twin is a representation of an object that models the characteristics and behaviours of a real-life object or system. The digital twin provides support to network management and operations by creating a virtual representation of the corresponding physical network process(es).  A digital twin can be created for any physical object, including any objects in communication networks. The digital twin may also be created for a group of objects, e.g. for the sets of network objects that form the RAN segment.

Accordingly, a digital twin modelling of an object of a communication network is called Network Digital Twin

Network Digital Twin (NDT) is used as a replica of a mobile network, in order to learn how an actual mobile network would behave in certain scenarios, without causing any changes to the actual mobile network. To provide meaningful results, NDT needs to model the behavior of the mobile network, so that the result of the operations on the virtual replica are good approximations to similar operations on the actual network. The implementation of an NDT can rely on simulation, emulation, AI-based modelling, or any other technique that enables the NDT to mimic the behaviour of the network with a certain fidelity level. The decision on the right technique to be used by an NDT depends on cost, accuracy, prediction capabilities needed, etc., and it is out-of-scope of standardization.

NOTE:	Simulation implies that the NDT uses a mathematical model to characterize the behavior of the system. This is used e.g. to model the traffic generated by users in the RAN to model the movement of users, or a simplified behaviour of a NF. Emulation implies that the NDT uses a realizable object such as software that replaces the live object to characterize the behavior of at least one aspect of the system. This is used e.g. to replicate or mimic the functionalities of core network functions which are implemented using the real software of NFs.

### 4.X.2	Scope of NDTs

Existing automation capabilities include analytics services and decision-making capabilities with the assumption that the knowledge about network behavior is available within the automation functions. Accordingly, the unaddressed gap for network automation is the capabilities for modelling the behavior of the network. The Network Resource Model (NRM) is already used to describes the attributes of a mobile network and their relationships. The concept of Network Digital Twin also adds the ability to model the behavior of a mobile network. This behavioral model can be implemented by emulating or simulating a complete mobile network or limited aspects of a mobile network.

## 4.2	NDT Life-cycle management

An NDT is an analytical tool that can provide modelling capabilities to be used by the network automation functions to obtain information useful in operation decision-making.  An NDT may provide multiple predictions by executing multiple modelling sessions  considering different network scenarios. The NDT is an object that can be managed, the LCM includes LCM of the NDT and of the modelling sessions. Accordingly, the following capabilities are included:

-	NDT Creation: The MnS producer who provides the NDT creates an instance to model a  specific scenarios. The NDT instance can be  configured by the MnS consumer at any time

-	Modelling session instantiation: NDT MnS producer receives the request to create a modelling task instance or job. The NDT MnS producer instantiates and executes the simulation/emulation. The modelling session can be configured by the MnS consumer at any time

-	Modelling session suspension: NDT MnS producer receives the request to pause or suspend a modelling session task instance or job.

- 	modelling session deletion: NDT MnS producer receives the request to delete a simulation/emulation task instance/job. The NDT MnS producer stops the execution of the simulation/emulation and deletes the simulation/emulation task instance or job.

-	NDT Deletion: the MnS producer may delete an NDT instance that is not under execution.

NOTE: All of the life cycle operations may not be valid for all NDT instances. For example, a particular NDT instance may not support suspending the execution.

## 4.3 NDT Management Service

The NDT MnS can be requested by the NDT MnS consumer to model the behaviour of the network and generate the simulation/emulation output. The simulation/emulation capabilities provided by NDT MnS Producer can be invoked by the network intelligence and automation management functions (e.g. MDAF, AIML Inference Function, Intent Handling Function, etc.) playing the role of NDT MnS Consumer to support their intelligence and automation functionality.

Figure 4.1-1

* * * End of Changes * * * *