3GPP TSG SA5 Meeting #160	S5-251326

Goteborg, Sweden, 07 - 11 April 2025

Source:	RAN3

Title:	LS on Continuous MDT

Document For:	Information

Agenda Item:	6.1

3GPP TSG-RAN WG3#127	R3-250891

Athens, GR, 17-21 February 2025

Title:	LS on Continuous MDT

Release:	Rel-19

Work Item:	NR_AIML_NGRAN_enh-Core

Source:	RAN3

To:	SA5

Cc:

Contact Person:

#### Name:                Angelo Centonza

E-mail Address:   angelo.centonza@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

# 1. Overall Description

RAN3 is working on the following objective as part of the AI/ML for NG-RAN WID (RP-242385):

  - Continuous MDT collection targeting the same UE across RRC states

Continuous MDT is a functionality under discussion in RAN3 that consists of using Management Based MDT for collection of MDT measurements from the same UE while it moves between different NG-RAN nodes and while it changes RRC states.

To fulfil the objective above RAN3 has identified the following problem statements that need to be resolved to enable the Continuous MDT solution:

-	Measurement Continuity Problem: how to ensure that the same UE is selected for collection of MDT measurements across RRC states and during RRC_Connected mobility.

-	Trace Correlation Problem: how to ensure that the TCE receiving the Trace Record files containing MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection process from the same UE.

During the course of Rel19, RAN3 has also taken the following agreements on Continuous MDT:

- The Continuous MDT functionality should be enabled by a solution applicable to legacy UEs and not only to Rel-19 UEs.

- RAN3 assumes that the OAM configures Management Based MDT towards the NG-RAN to establish Continuous MDT.

To solve the Measurement Continuity Problem, RAN3 agreed that:

- The OAM provides to the RAN means to identify that the Management Based MDT is for Continuous MDT, e.g. reusing existing Management Based MDT configuration information.

- RAN3 will derive solutions for the Measurement Continuity Problem across RRC_Connected mobility. SA5 is requested to focus on the Measurement Continuity Problem across different RRC states.

- A solution should minimise impacts on the current Management Based MDT framework.

To solve the Trace Correlation Problem, namely to enable correlation of MDT measurements generated by the same UE, it is required to use a temporary identifier for the Continuous MDT, not revealing the UE identity. Such identifier should be made available at the TCE to enable correlation of the management-based MDT reports collected during the Continuous MDT, in a way that minimises the impacts on the Management Based MDT solution.

# 2. Actions

To SA5

ACTION: RAN3 would like to ask SA5 whether the above is feasible to fulfil the established requirements and to solve the problem statements outlined. RAN3 would also like to ask SA5 to provide guidance on possible solutions for Continuous MDT.

# 3. Date of Next TSG-RAN3 Meetings

TSG-RAN WG3 Meeting RAN3#127bis			7 - 11 April 2025		Wuhan, CN

TSG-RAN WG3 Meeting RAN3#128			19 - 23 May 2025		Malta, MT