3GPP TSG-SA5 Meeting #160	S5-251345

Goteborg, Sweden, 07 - 11 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.271 | CR | 0033 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Extend data description for 5G LCS converged charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CH_5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is no data description for 5G LCS converged charging |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend data description to support 5G LCS converged charging |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G LCS converged charging will not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

6.2a	Data description for 5G LCS converged charging

6.2a.1	Message contents

6.2a.1.1	General

The Charging Data Request and Charging Data Response are specified in TS 32.290 [55] and include charging information. The Charging Data Request can be of type [Event].

Table 6.2a.1.1.1 describes the use of these messages for converged charging.

Table 6.2a.1.1.1: Converged charging messages reference table

| Message | Source | Destination |
| --- | --- | --- |
| Charging Data Request | GMLC | CHF |
| Charging Data Response | CHF | GMLC |

The following clauses describe the different fields used in the Charging Data messages and the category in the tables is used according to the charging data configuration defined in clause 5.4 of TS 32.240 [1].

6.2a.1.2	Structure for the converged charging message formats

6.2a.1.2.1	Charging Data Request message

Table 6.2a.1.2.1.1 illustrates the basic structure of a Charging Data Request message as used for 5G LCS converged charging.

Table 6.2a.1.2.1.1: Charging Data Request message contents

| Information Element | Category | Description |
| --- | --- | --- |
| Subscriber Identifier | OM | Described in TS 32.290 [55]. |
| Tenant Identifier | OC | Described in TS 32.290 [55], and holds the identifier of the AF. |
| NF Consumer Identification | M | Described in TS 32.290 [55] |
| NF Functionality | M | Described in TS 32.290 [55]. |
| NF Name | OC | Described in TS 32.290 [55]. |
| NF Address | OC | Described in TS 32.290 [55]. |
| NF PLMN ID | OC | Described in TS 32.290 [55]. |
| Charging Identifier | OM | Described in TS 32.290 [55]. |
| Invocation Timestamp | M | Described in TS 32.290 [55] |
| Invocation Sequence Number | M | Described in TS 32.290 [55] |
| One-time Event | OC | Described in TS 32.290 [55]. |
| One-time Event Type | OC | Described in TS 32.290 [55]. |
| Retransmission Indicator | OC | Described in TS 32.290 [55]. |
| Supported Features | OC | Described in TS 32.290 [55]. |
| Service Specification information | OC | Described in TS 32.290 [55]. |
| LCS Charging Information | OC | This field holds the 5G LCS and Ranging and Sidelink Positioning specific information described in clause 6.3.1.3. |

6.2a.1.2.2	Charging Data Response message

Table 6.2a.1.2.2.1 illustrates the basic structure of a Charging Data Response message as used for 5G LCS converged charging.

Table 6.2a.1.2.2.1: Charging Data Response message content

| Information Element | Category | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [55] |
| Invocation Timestamp | M | Described in TS 32.290 [55] |
| Invocation Result | OC | Described in TS 32.290 [55] |
| Invocation Result | OC | Described in TS 32.290 [55] |
| Failed parameter | OC | Described in TS 32.290 [55] |
| Failure Handling | OC | Described in TS 32.290 [55] |
| Invocation Sequence Number | M | Described in TS 32.290 [55] |
| Session Failover | OC | Described in TS 32.290 [55] |
| Supported Features | OC | Described in TS 32.290 [55] |

6.2a.2	Ga message contents

Refer to clause 5.4.4.

6.2a.3	CDR description on the Bl interface

6.2a.3.1	General

This clause describes the CDR content and format generated for 5G LCS converged charging.

The following tables provide a brief description of each CDR parameter. The category in the tables is used according to the charging data configuration defined in clause 5.4 of TS 32.240 [1]. Full definitions of the CDR parameters, sorted by the name in alphabetical order, are provided in TS 32.298 [7].

6.2a.3.2	5G LCS charging CHF CDR data

If enabled, CHF CDRs for 5G LCS converged charging shall be produced for 5G LCS chargeable events.

The fields of 5G LCS converged charging CHF CDR are specified in table 6.2a.3.2-1.

Table 6.2a.3.2.1: 5G LCS converged charging CHF record data

| Field | Category | Description |
| --- | --- | --- |
| Record Type | M | Described in TS 32.298 [51]. |
| Recording Network Function ID | OM | Described in TS 32.298 [51]. |
| Tenant Identifier | OC | Described in TS 32.298 [51]. |
| NF Consumer Information | M | This field holds the information of the entity that used the charging service (i.e. GMLC). |
| NF Functionality | M | Described in TS 32.298 [51]. |
| NF Name | OC | Described in TS 32.298 [51]. |
| NF Address | OC | Described in TS 32.298 [51]. |
| NF PLMN ID | OC | Described in TS 32.298 [51]. |
| Record Opening Time | M | Described in TS 32.298 [51]. |
| Duration | M | Described in TS 32.298 [51]. |
| Record Sequence Number | C | Described in TS 32.298 [51]. |
| Cause for Record Closing | M | Described in TS 32.298 [51]. |
| Local Record Sequence Number | OM | Described in TS 32.298 [51]. |
| Record Extensions | OC | Described in TS 32.298 [51]. |
| LCS Charging Information | OC | This field holds the 5G LCS and Ranging and Sidelink Positioning charging information defined in clause 6.3.1.3. |

End of changes