3GPP TSG-SA5 Meeting #160	S5-251370

Goteborg, Sweden, 07 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.260 | CR | 0438 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.260 Support duration-based charging for standalone IMS data channel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2-CH |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Charging for standalone IMS data channel should be supported, but duration-based charging for standalone IMS data channel has not been considered in this specification. The CR is related to solution #1.1 in clause 5.1.4.1 of TR 28.851. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add charging support of standalone IMS data channel sessions. Enhance general charging principle of duration-based charging for IMS data channel. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Charging for standalone IMS data channel is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.15, 5.1.15.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.1.15	Charging support of duration-based charging for IMS data channel

5.1.15.x	General

The basic procedures related to IMS data channel are specified in TS 23.228 [201], with the signalling procedures for IMS data channel specified in clause 4.20 of TS 24.229 [204] and clause 9 of TS 24.186 [224]. UE can establish data channel(s) simultaneously while establishing an IMS audio/video session or upgrade an ongoing IMS audio/video session through a re-INVITE to add IMS data channel(s). The IMS data channel is kept as long as there is an independent IMS audio/video SDP. The data channel is either terminated along with the release of the call or is or be terminated by closing the data channel. The charging principle of this senario is same as duration-based charging for IMS audio/video session.

The information of each DC stream is available in SDP which can be used for IMS data channel charging. The media handling and interaction for IMS data channel are specified in TS 26.114 [223]. The SDP can contain media description for data channel:

-	"m=application" line with webrtc-datachannel.

-	"a=dcmap" lines with stream ID for different types of data channels, and the stream ID values are specified in TS 26.114 [223].

-	"a=3gpp-req-app" attribute with a "req-app-id" parameter to identify the application for data channels, as specified in clause 6.2.13 of TS 26.114 [223].

Second change

5.1.15.y	Support of standalone IMS Data Channel sessions

As specified in clause AC.10 of TS 23.228 [200], a standalone IMS data channel session establishes data channels without MMTel media (e.g. audio, video, messaging). Bootstrap data channels enable application list/downloads. Application data channels between UEs may be initiated simultaneously or sequentially with bootstrap channels during session establishment.

Similar to regular IMS data channel, the SDP can contain different DC media components for standalone data channel in different procedures specified in clause 9.3.2.1.6 of TS 24.186 [224]. The charging information of each DC media components which specified in clause 5.1.15.x is applicable for duration-based charging for standalone IMS data channel.

There are two standalone IMS data channel session types:

a)	In scenarios where only the bootstrap data channel (targeting Public Service Identifier) is utilized, the Public Service Identifier can be contained within the Called Party Address IE.

b)	In scenarios where only bootstrap and application data channels (targeting peer UE) are utilized, the Public User ID of the terminating UE can be contained within the Called Party Address IE.

End of changes