3GPP TSG-SA5 Meeting #160	S5-251386

Gotenburg, SE, 7 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.310 | CR | 0072 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.310 Add information model definitions for energy related information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Energy_OAM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 22.261 requires that energy related information, including ratio of renewable energy and carbon emission information for the 5G system, be measured. The Energy Information Function (EIF) newly introduced into TS 23.501 may consume not only gNB and UPF Energy Consumption and Data Volume measurements from OAM, but also other energy related information such as carbon emission and renewable energy. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce new energy related information model into TS 28.310. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EIF cannot get the expected energy related information about gNBs and UPFs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ETSI ES 203 228: "Environmental Engineering (EE); Assessment of mobile network energy efficiency".

[3]	ETSI ES 202 336-1 V1.2.1: "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 1: Generic Interface".

[4]	ETSI ES 202 336-12 V1.1.1: "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[5]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[6]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[7]	Void.

[8]	3GPP TS 32.432: "Telecommunication management; Performance measurement: File format definition".

[9]	3GPP TS 32.435: "Telecommunication management; Performance measurement; eXtensible Markup Language (XML) file format definition".

[10]	3GPP TS 32.436: "Telecommunication management; Performance measurement: Abstract Syntax Notation 1 (ASN.1) file format definition".

[11]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[12]	3GPP TS 38.401: "NG-RAN; Architecture description".

[13]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[14]	Void.

[15]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[16]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[17]	3GPP TS 32.551: "Energy Saving Management (ESM); Concepts and requirements".

[18]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[19]	ETSI GR NFV-IFA 015 V3.4.1 (2020-06): "Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Report on NFV Information Model".

[20]	ETSI GR NFV 003 V1.6.1 (2021-03): "Network Functions Virtualisation (NFV); Terminology for Main Concepts in NFV".

[21]	3GPP TS 28.530: " Management and orchestration; Concepts, use cases and requirements".

[22]	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks".

[23]	ETSI ES 202 706-1 V1.7.1 (2022-08): "Environmental Engineering (EE); Metrics and measurement method for energy efficiency of wireless access network equipment; Part 1: Power consumption - static measurement method".

[24]	Void.

[25]	ETSI GS NFV-IFA 027 (V4.3.1) (2022-06): "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Performance Measurements Specification".

[26]	ETSI ES 202 336-12 (V1.2.1) (2019-02): "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[27]	ETSI GS NFV-IFA 008 V3.5.1 (2021-11): "Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Ve-Vnfm reference point - Interface and Information Model Specification".

[28]	ETSI GS NFV-TST 008 V3.3.1 (2020-12): "Network Functions Virtualisation (NFV) Release 3; Testing; NFVI Compute and Network Metrics Specification".

[29]	ETSI GS NFV-IFA 006 (V3.7.1) (2022-11): " Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Vi-Vnfm reference point – Interface and Information Model Specification".

[30]	3GPP TS 28.111: "Fault management".

[y]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[z]	ITU-T L.1333: "Carbon data intensity for network energy performance monitoring".

Next change

# X	Information model definitions for Energy NRM

## X.1	Imported and associated information

### X.1.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [y], IOC, SubNetwork | SubNetwork |
| TS 28.622 [y], IOC, ManagedElement | ManagedElement |
| TS 28.622 [y], IOC, ManagedFunction | ManagedFunction |
| TS 28.622 [y], IOC, Top | Top |

### X.1.2	Associated information entities and local labels

| Label reference | Local label |
| --- | --- |

## X.2	Class diagram

### X.2.1	Relationships

Figure X.2.1-1: Energy related information NRM fragment

### X.2.2	Inheritance

This clause depicts the inheritance relationships.

Figure X.2.2-1: Energy related information NRM fragment

## X.3	Class definitions

### X.3.1	EnergyInfo

#### X.3.1.1	Definition

This IOC represents energy related information for a network element or a network function or a group thereof. It may represent energy related information for a group of network elements and/or network functions when they all share the same energy supply mode. This information is configured by the operator with the information obtained from external sources. How this information is obtained by the operator is out of scope of the present document.

The attribute energySupplyMode indicates the means by which the energy is supplied to a network element or a network function or a group thereof.

The attribute energySupplyRatio indicates the proportion of the energy provided via the energy supply mode relatively to the total energy supplied to a network element or a network function or a group thereof.

The attribute energySourceInfoList represents the information on the energy source(s) used by a network element or a network function or a group thereof, via the energy supply mode.

The attribute associatedMEList contains the DN of a ManagedElement MOI or of a ManagedFunction MOI or a list of ManagedElement MOIs and/or of a ManagedFunction MOIs representing managed entities sharing the same energy supply mode.

#### X.3.1.2	Attributes

The EnergyInfo IOC includes the attributes inherited from Top IOC and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| energySupplyMode | M | T | T | F | T |
| energySupplyRatio | M | T | T | F | T |
| energySourceInfoList | M | T | T | F | T |
| Attributes related to role |  |  |  |  |  |
| associatedMEList | M | T | T | F | T |

#### X.3.1.3	Attribute constraints

None

#### X.3.1.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions.

### X.3.2	EnergySourceInfo <<datatype>>

#### X.3.2.1	Definition

This data type defines the information related to a type of energy source.

The attribute energySource is used to indicate the source of energy used to power the managed entity via the energy supply mode.

The attribute energySourceRatio is used to indicate the percentage of energy source used to power the managed entity via the energy supply mode.

The attribute energySourceCef is used to indicate the Carbon Emission Factor of the energy source.

The attribute renewableEnergy is used to indicate if the energy source is renewable or non-renewable.

.

#### X.3.2.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| energySource | M | T | T | F | T |
| energySourceRatio | M | T | T | F | T |
| energySourceCef | M | T | T | F | T |
| renewableEnergy | M | T | T | F | T |

#### X.3.2.3	Attribute constraints

None.

#### X.3.2.4	Notifications

The clause X.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

## X.4	Attribute definitions

### X.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| associatedMEList | This attribute contains the DN of a managed entity or of a group of managed entities sharing the same energy supply mode. allowedValues: DN of ManagedElement or ManagedFunction (or sub-classes) MOIs. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| energySupplyMode | The means by which energy is supplied to a managed entity or a group thereof. It can be one of the following: - GRID_ELECTRICITY: the energy comes from local grid electricity (See Grid electricity in clause 6 of ITU-T Recommendation L.1333 [z]) - BACKUP_ENERGY: the energy comes from backup diesel or other types, or from generators such as solar panels. (See Backup energy in clause 6 of ITU-T Recommendation L.1333 [z]) - LOCALLY_GENERATED_ENERGY: the energy is provided by local generators not used for backup functionality, e.g., solar, wind generator. (See Locally generated energy in clause 6 of ITU-T Recommendation L.1333 [z]) allowedValues: - “GRID_ELECTRICITY” - “BACKUP_ENERGY” - “LOCALLY_GENERATED_ENERGY” | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| energySupplyRatio | The proportion of the energy provided by the energy supply mode relatively to the total energy supplied to the managed entity or a group thereof. allowedValues: integer between 0 and 100. | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 100 isNullable: False |
| energySourceInfoList | This attribute contains a list of energy source informations. allowedValues: Not applicable. | type: EnergySourceInfo multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| energySource | The type of energy source. allowedValues: N/A This attribute shall follow the following pattern: "<source document>:<energy source>". For example, the values can be "ITU-T Rec xxx:solar", "ETSI yyy:wind", etc. Which source document is to be used by an Operator and how the Operator gathers this information are out of scope of the present document. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| energySourceRatio | The proportion of the energy provided by the energy source type relatively to the total energy supplied to the ManagedEntity (or a group thereof) through an energy supply mode. allowedValues: integer between 0 and 100 | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 100 isNullable: False |
| energySourceCef | The Carbon Emission Factor of a type of energy source. Its unit is kg CO2eq / kWh. allowedValues: non-negative integers | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| renewableEnergy | This indicates whether the type of energy source is deemed renewable or not. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |

## X.5	Common notifications

### X.5.1	Configuration notifications

This clause presents a list of notifications, defined in TS 28.532 [16], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

Table X.5.1-1

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyMOICreation | O | -- |
| notifyMOIDeletion | O | -- |
| notifyMOIAttributeValueChanges | O | -- |
| notifyEvent | O | -- |

Next change

Forge MR link: XXX at commit YYY

*** START OF CHANGE ***

*** OpenAPI/TS28310_EnergyNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: 3GPP Energy NRM

version: 19.1.0

description: >-

OAS 3.0.1 specification of the 5G Energy Info NRM

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.310; 5G NRM, 5G Energy NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.310/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

EnergySupplyMode:

type: string

description: any of enumerated value

enum:

- GRID_ELECTRICITY

- BACKUP_ENERGY

- LOCALLY_GENERATED_ENERGY

EnergySourceInfo:

type: object

properties:

energySource:

type: string

energySourceRatio:

type: integer

energySourceCef:

type: integer

renewableEnergy:

type: boolean

EnergySourceInfoList:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/EnergySourceInfo'

#-------- Definition of types for name-containments ----------------------------

SubNetwork-ncO-EnergyInfoNrm:

type: object

properties:

EnergyInfo:

$ref: '#/components/schemas/EnergyInfo-Multiple'

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

EnergyInfo-Multiple:

type: array

items:

$ref: '#/components/schemas/EnergyInfo-Single'

#-------- Definition of concrete IOCs --------------------------------------------

EnergyInfo-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedElement-Attr'

- type: object

properties:

energySupplyMode:

$ref: '#/components/schemas/EnergySupplyMode'

energySupplyRatio:

type: integer

energySourceInfoList:

$ref: '#/components/schemas/EnergySourceInfoList'

End of change