3GPP TSG-SA5 Meeting #160	S5-251441

# Goteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0198 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel19 CR TS28.104 add use case and requirement on correlation analytics for NF Scaling and dimensioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The use case of “Measurement data correlation analytics for NF Scaling and dimensioning” has been discussed and agreed upon in the study (clause 5.3.6 in TR 28.866). This CR is to normatively propose the solution. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding solution for the use case description and requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No solution for the use case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.x.2, 8.5.x (new clauses added) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS28.104 CR0197 |  |  |
| Other comments: |  | The new clause 8.4.x.2 is to be after 8.4.x.1 (new clause 8.4.x is from CR0197) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

#### 8.4.x.2	Analytics for NF Scaling and dimensioning

##### 8.4.x.2.1	MDA type

The MDA type for NF Scaling and dimensioning analysis is: CorrelationAnalytics.NFScalingDataAnalysis and CorrelationAnalytics.DimensiningDataAnalysis.

##### 8.4.x.2.2	Enabling data

The enabling data for CorrelationAnalytics.NFScalingDataAnalysis and CorrelationAnalytics.DimensiningDataAnalysis MDA types are provided in table 8.4.x.2.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.x.2.2-1: Enabling data for correlation analysis for NF Scaling and dimensining

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | The performance metric or KPI data for management data correlation analysis for NF Scaling and dimensining. | The performance metric data defined in TS 28.552 [4] And KPI data defined in TS 28.554 [5] |
| Geographical data | The geographical information (longitude, latitude, altitude) of the deployed 5GC Core NF. | The geographical information (longitude, latitude, altitude) information (it may be provided in the peeParametersList attribute of the ManagedFunction IOC in TS 28.622 [19]). |
| Configuration data | The NRMs of the 5GC NF, Network Slice, SliceSub Network | The 5GC Functions, slice related NRM defined in TS 28.541 [15]. |

##### 8.4.x.2.3	Analytics output

The specific information elements of the analytics output for NF Scaling and dimensioning analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.x.2.3-1.

Table 8.4.x.2.3-1: Analytics output for NF Scaling and dimensioning analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| applicationScenario | It indicates this recommend report is for which application scenario analysis request. allowedValue: NF_SCALLING, DIMENSIONING | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| correlationResult | It indicates the correlation results for the relationship between requested NF, including a pair of NF name, and the correlation value. With pattern: '^([^:]+):([^:]+):(0(\.\d{1,2})?¦1(\.00)?)$' , for example “AMF:UDM:0.42”. allowedValue: N/A | O | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| associationResult | It indicates the list of association analysis for the relationship between requested NF, including NF name, association value. allowedValue: N/A | O | type: AssociationInfo multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| proportionalInfo | It indicate the proportional information among the NF for scaling or dimentioning optimization. This may be generated by correlation and/or association analysis. allowedValue: N/A | M | type: AttributeValuePair multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| timePeriod | It indicate the time window for the recommended scaling operation. allowedValue: N/A | O | type: TimeWindow multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| recommendedActions | The recommended actions to orchestrate the resource allocation for NFs as per the correction & association analysis. The recommended action may be (but not limited to): - scale in a list of NFs; - scale out a list of NFs. | M | type: RecommendedAction multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| orderInfo | It indicates the ordering information related to NF Scaling. The value can be NFType, or DN of the instance. This attribute is only supported for scenario of NF_SCALLING. allowedValue: N/A | CM | type: DN or NFType (defined inTS28.541[15]) multiplicity: 2..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| orderInterval | It indicates the interval of the order of scaling operations. The unit is second. allowedValue: Integer | O | type: Integer multiplicity: 2..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| reliability | It indicate the recommendation report credibility level. Higher value represents higher reliability. The value range could be 0.0 .. 1.0. allowedValue: 0.0..1.0 | M | type: Float multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

Next change

### 8.5.x	AssociationInfo <<dataType>>

#### 8.5.x.1	Definition

This data type specifies the association information.

#### 8.5.x.2	Information elements

Table 8.5.x.2-1

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| associationPair | This attribute indicates the two NF for a association relationship. Pattern: ^([^:]+):([^:]+)$ allowedValues: N/A | M | type: String multiplicity:1 isOrdered: N/A isUnique: N/A defaultValue: none isNullable: False |
| associationSupport | It indicates the support value for a given association rule/relationship. allowedValues: 0..1.0 | M | type: Float multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| associationConfidence | It indicates the confidence value for a given association rule/relationship.. allowedValues: 0..1.0 | M | type: Float multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of First change