3GPP TSG-SA5 Meeting #160	S5-251501

Gothenberg, Sweden, 7 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.421 | CR | 0146 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 32.421 Enhancement to Trace Reference |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Data_SREP |  |  |  |  |  | Date: |  |  | 2024-03-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Trace Reference shall be globally unique as stated in clause 5.6 of TS 32.422. Due to the increasing number of consumers it becomes a challenge to provide globally uniqueness as stated in TR 28.873. Trace Reference uniqueness needs to be achieved with limited impact in RAN and especially the air interface. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhance the Trace Reference parameter by an additional optional identifier and introduce a mapping in case of NG-RAN trace activation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Restrictions on the number of consumers which can initiate a trace session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 5.3.1	Requirements for Trace Session activation

The high level requirements for Trace Session activation, common to both Management activation and Signalling based activation), are as follows:

-	In the case of a subscriber Trace, the Trace Session will be activated for a certain subscriber whose identification (IMSI in UTRAN/CS/PS) shall be known in the NEs where subscriber Trace is needed. 
In the case of E-UTRAN the IMSI shall not be included in the Trace Parameter Propagation data to the e-NodeB. In the case of NG-RAN the IMSI/SUPI shall not be included in the Trace Parameter Propagation data to the NG-RAN node.

-	In the case of a UE Trace, the Trace Session will be activated for a certain UE whose identification (IMEI or IMEISV) shall be known in the NEs where UE Trace is needed. In the case of E-UTRAN, neither the IMEI nor IMEISV shall be included in the Trace Parameter Propagation data to the e-NodeB -	Trace Session activation shall be possible for both home subscribers and visiting subscribers. In the case of NG-RAN, neither the IMEI/SUPI nor IMEISV shall be included in the Trace Parameter Propagation data to the NG-RAN node.

-	There are two methods for Trace Session activation: Management activation and Signalling activation.

-	For an established call/session within a Network Element, it is optional for the Network Element to start a Trace Recording Session for the associated Subscriber or UE upon receipt of the Trace activation request from the management system.

-	A globally unique ID shall be generated for each Trace Session to identify the Trace Session. 
This is called the Trace Reference. 
The method for achieving this is to divide the Trace reference into Country (MCC), Operator (MNC), trace Id and optional Trace Differentiator.

-	Trace Session may be activated from the management system simultaneously to multiple NEs with the same Trace Reference (i.e. same Trace Session).

-	The Trace Scope and Depth shall be specified within the control and configuration parameters during Trace Session activation.

-	There can be cases in a NE when it receives multiple Trace Session activations for the same connection (e.g. simultaneous CS/PS connections). In these cases the starting time of the Trace Session Activation and the starting time of the first Trace Recording Session is the same using signalling based activation. For these cases there are two different cases for the Trace Session activation in a Network Element when it receives another Trace Session activation to the same subscriber or MS:

-	If the Trace Reference is equal to an existing one, a new Trace Session shall not be started;

-	If the Trace Reference is not equal to an existing one, a new Trace Session may be started.

-	The management system shall always provide the trace control and configuration parameters to the appropriate NEs at the time of Trace Session activation.

-	The Trace collection entity shall be notified, in case of theTrace Session activation has failed, by the response message with the specific cause (e.g. overload) from the NE on which the Trace Session activation failure happened.

-	It shall be possible to specify the trace reporting method (file-based vs. streaming) during Trace Session Activation.

-	In case of streaming trace reporting method being selected, the data producer shall establish the connection to the data consumer upon Trace Session Activation and provide data consumer with information about Trace Session.

The high-level requirements for Trace Session activation, specific to Signalling Based activation, are as follows:

-	Signalling based activation shall be able to capture signalling messages as early in a session as possible, e.g. by means of a piggybacked  trace invocation message in the case of a new connection or new bearer setup

For active users, it shall be possible to start trace recording when the trace order is received,  by means of a distinct trace invocation message.

The high-level requirements for Trace Session activation, specific to Management activation, are as follows:

-	In the case of a subscriber Trace, the Trace Session will be activated for a certain subscriber whose identification (IMSI in UTRAN/CS/PS or Public User Identity in IMS) shall be known in the NEs where subscriber Trace is needed.

In the case of a Cell Traffic Trace, Trace Session activation should be possible for all calls active in a cell or multiple cells without knowledge of the UEs’ identification (IMEI or IMEISV).

-	In the case of a Cell Traffic Trace, Trace Sessions should be activated for all the NEs where Cell Traffic Trace is specified.

-	In the case of Cell Traffic Trace (in a shared network only), a Trace Session shall be started for UEs which are served by the Participating Operator that has made the request to the Primary Operator.

The high-level requirements for Trace Session activation specific for Service Level Tracing for IMS are as follows:

The following high-level OMA Service Level Tracing requirements apply [9]:

- [SLT-COM-2] with the following clarification:

-	The OMA term component in the context of Service Level Tracing for IMS shall be understood as IMS NE and UE.

- [SLT-HL-2] with the following clarification:

-	The OMA terms device and component shall be understood as UE and IMS NE, respectively;

-	The IMS NEs HSS, P/I/S-CSCF, AS and UE, apply only.

- [SLT-AC-1] with the following clarification:

-	The OMA term Authorised Actor shall be understood as NE, EM or NM;

-	The OMA term trace indication shall be understood as Start Trigger Event.

- [SLT-AC-2] with the following clarification:

-	The OMA term Service Provider shall be understood as Service Provider;

-	The OMA term marking request shall be understood as the ability to send the Trace Parameter Configuration to either the UE or IMS NE.

- [SLT-AC-6]

- [SLT-AC-7] with the following clarification:

-	The OMA term criteria shall be understood as Trace Configuration Parameters.

End of changes