3GPP TSG-SA5 Meeting #160	S5-251566

Goteborg, Sweden, 07 - 11 April 2025

Source:	Huawei

Title:	Pseudo-CR on TS 28.561 Use case of RAN energy saving policy verification

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TS 28.561

Version:	0.1.0

Work Item:	NDT

Comments

It is proposed to add RAN ES policy verification use case.

Proposed Changes

* * * First Change * * * *

5.3	NDT support for verification

### 5.3.1	Description

NDTs can be used for verification on different use cases.

5.3.2	Use cases

5.3.2.1	RAN ES policy verification

When configuring the energy saving for RAN, normally the policy is applied in execution with monitoring and optimization loop to minimize the influence on network service quality. That's to say, there could be multiple ES policies executed iteratively in actual mobile network until the network performance, e.g. energy efficiency of NG-RAN, UE throughput in gNB, etc., meets certain requirements from operators. The digital twin technology may be used to evaluate the impact of RAN ES policy while satisfying simulation performance requirements.

In this scenario, MnS consumer (the entity who consumes the network simulation service) sends the request to verify the RAN ES policy to MnS producer. The request includes RAN ES Policy and performance requirements, which is MnS consumer’s requirements for network simulation performance, e.g. precision, maximum run time, etc. Then, the NDT performs network simulation, i.e., simulating the execution of RAN ES policy of the mobile network according to the performance requirements and collects its impact on the simulated network. The RAN ES policy can be seen as a specific set of network management operations achieved by provisioning MnS. After that, MnS producer sends the simulation report to MnS consumer to indicates the network simulation results (e.g. Energy efficiency, RAN UE throughput, etc).

5.3.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-NDTVER_ES_01 | The 3GPP management system should support a capability to estimate the impact of RAN ES policies. | RAN ES policy verification (see clause 5.3.2.1) |
| REQ-NDTVER_ES_02 | The 3GPP management system should support a capability enabling the consumer to configure the RAN ES policies. | RAN ES policy verification (see clause 5.3.2.1) |
| REQ-NDTVER_ES_03 | The 3GPP management system should support a capability to report the simulated/emulated impact of RAN ES policies. | RAN ES policy verification (see clause 5.3.2.1) |