3GPP TSG-SA5 Meeting #160	S5-251608

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | draftCR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Input to draftCR Rel-19 TS 28.105 Use case on ML explainability for inference |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To enable a consumer to indicate the need to support local explainability during ML model training. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Introduction of an use case on ML explainability in inference 2. Introduction of two requirements to have a capability allowing an authorized MnS consumer to request and receive local explanations during ML model inference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for ML explainability during ML inference. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.X.1, 6.5.X.2.1, 6.5.X.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

# 6	AI/ML management use cases and requirements

### 6.5	AI/ML inference

### 6.5.X	AI/ML inference explainability

### 6.5.X.1	Description

Explainable ML refers to a process that enables the consumers to understand and trust the outputs provided by ML models. In essence, explainable ML is about making the decision-making of ML comprehensible to its consumers. On a broad level, explainable ML can be categorized into two types:

- Local explanation: The aim is to explain individual outputs provided by an ML model, i.e., it focuses on explaining why a specific output was generated by the ML model for a particular input data sample.

- Global explanation: The aim is to explain the whole ML model behaviour, i.e., it focuses on explaining how the ML model works in general across several or all possible input samples.

#### 6.5.X.2	Use cases

#### 6.5.X.2.1	AI/ML inference local explanation

Once the ML model is trained to generate both outputs and corresponding explanations, it is deployed for inference. The generated local explanations by the AI/ML inference MnS producer may need to be reported to the AI/ML inference MnS consumer. These local explanations provide additional information that the MnS consumer can use to analyze the ML model’s potential impact on network performance. This analysis helps in making informed decisions regarding the deactivation of AI/ML inference or fallback to a previous version of the ML model. By considering network performance PM/KPIs, model performance, and local explanations, the MnS consumer can effectively identify the ML model contributing to network performance degradation when several ML models are active in the network.

6.5.X.3		Requirements for AI/ML inference local explanation

Table 6.5.X.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ- AI/ML_INF_XAI-01 | The AI/ML inference MnS Producer should have a capability allowing an authorized consumer to request the reporting of local explanations in AI/ML inference. | AI/ML inference explainability (clause 6.5.X.2.1) |
| REQ- AI/ML_INF_XAI-02 | The AI/ML inference MnS Producer should have the capability to report local explanations for AI/ML inference. | AI/ML inference explainability (clause 6.5.X.2.1) |

End of changes