3GPP TSG-SA5 Meeting #160	S5-251635

Goteborg, Sweden, 07 - 11 April 2025

# Source:	Nokia

# Title:	Rel-19 pCR TS 28.572 Improve clauses on Planned configuration concepts

# Document for:	Approval, Information, Discussion

Agenda Item:	6.19.9.1 - Management of planned configurations

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]		3GPP TS 28.572: "Management of planned configurations"

# 3	Rationale

None.

# 4	Detailed proposal

The following changes are proposed for TS 28.572[1].

Begin of modifications

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS)".

[3]	IETF RFC 8072: " YANG Patch Media Type".

[4]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[5]	3GPP TS 32.161: "Management and orchestration; JSON expressions (Jex)".

Next modification

# 6	Solution description

# Editor's note: Clarify that annotations are only for the user and are not processed by the MnS producer.

## 6.1	Planned configurations

### 6.1.1	Definition

The current configuration is the configuration currently used by the managed system. It is represented by (a tree of) configuration data nodes. State data nodes are not included. The format of the configuration data node tree is defined by (stage 2) NRM definitions and (stage 3) schema definitions.

Note that the data node tree representing a manged system typically comprises besides configuration data nodes also (read-only) state data nodes, that represent the current state of the managed system. Configuration data nodes and state data nodes may be separated in dedicated subtrees, but they may be also contained in one tree.

A planned configuration is a configuration that can be manipulated without impacting the current configuration of the system. It relates to configuration data nodes and not to state data nodes. It is maintained side-by-side with the current configuration.

### 6.1.2	Format of planned configurations

A planned configuration is represented by a set of operations to be applied to the current configuration. The operations are described by the input parameters of the "changeMOIs" operation specified in TS 28.532 [4], with the addition that each operation in the operation set has a key for identifying the operation.

The following guidelines may ease the processing of operation sets during validation and activation:

- Parent objects should be created before child objects.

- Objects that are referenced should be created before the reference.

### 6.1.3	Meta data for planned configurations

Each planned configuration is described by meta data. This includes the name, the version, and a human readable textual description. It is also possible to specify additional MnS consumer defined properties (key value pairs) to further describe and qualify the planned configuration, for example to specify who created the planned configuration. The aforementioned annotations are for usage by a (human) MnS consumer only. The are not processed by the MnS producer.

Editor's note: Clarify that "version" may help the consumer to realize a version management system (use multiple plans with different version numbers).

Furthermore, it includes the content type of the planned configuration and a pointer to the current configuration. The pointer allows for deployments where the planned configuration is on another MnS producer than the current configuration.

Furthermore, it is specified if the operations of the operations set are an atomic set, where all operations need to be validated and activated successfully, or if some operations are allowed to fail (non-atomic or best effort set). In other words, for atomic sets, all operations of the set must be processed successfully or no operation at all. Operations, that are already applied when an error occurs, must be rolled back. For best effort sets, the operations that can be processed successfully are processed, and those that for whatever reason cannot be processed successfully are not processed. In this mode the validation or activation process continues on the occurrence of an error. In a third mode the validation or activation process stops on the occurrence of an error.

For example, subscriptions to alarm notifications on multiple Network Functions may have value also in case the subscription on some Network Functions fails. When analytics are computed based on measurements collected at some well selected Network Functions, then all measurement collection jobs must be created. If one job cannot be created it does not make sense to collect the remaining measurements.

The date and time at which information in the planned configuration was modified the last time is provided as well.

### 6.1.4	Planned configuration descriptor

The operation set and related meta data are specified in a planned configuration descriptor. Each descriptor has a unique identifier.

## 6.2	Planned configuration groups

### 6.2.1	Definition

A planned configuration group is a set of planned configurations that shall be processed (validated and activated) together.

For example, one plan may create a new cell, and another plan may start the collection of measurements for the new cell, or one plan may configure one base station and another plan may configure another base station, when both base stations need to be consistently configured to enable handovers between them.

### 6.2.2	Planned configuration group members

A planned configuration belonging to a planned configuration group is also called a member of that planned configuration group. A planned configuration can be a member of more than one planned configuration group.

The members of a planned configuration group shall be considered as atomic or non-atomic (best effort), in the same way as the operations of an operation set may form an atomic or a non-atomic (best effort) set.

Furthermore, the members of a planned configuration group can be ordered or not ordered. If ordered, the planned configuration group members are processed in the order as specified in the planned configuration group. If not ordered, the members can be processed in any order or in parallel.

For example, when the configuration for a new Network Function is in one plan and the creation of a performance metric collection job in another plan, then the plans must be ordered (assuming the job object is contained under a configuration object).

Not ordered are for example plans adding performance metric collection jobs to already configured Network Functions.

Editor's note: A planned configuration group should be allowed to be a member of a planned configuration group. This is ffs.

### 6.2.3	Conflicts between planned configuration group members

Conflicting operations may be contained in planned configuration group members. For example, a planned configuration may set an attribute to some value and another planned configuration sets the same attribute to some other value.

If the configuration group members are ordered, the result is deterministic. The conflicting operations are processed in the order as they appear in the ordered plans. The last processed operation from the ordered set of operations manipulating the same node determines the result. If the configuration group members are not ordered, the result is not deterministic. Depending on which operation from the unordered set of operations manipulating the same node is processed last, the result may be different.

It is configurable if a detected conflict should invalidate the planned configuration group.

### 6.2.4	Meta data for planned configuration groups

Each planned configuration group is described by meta data. This includes annotations such as the name, the version, and a human readable textual description.

Furthermore, it includes the information if the group members are atomic or non-atomic, if they are ordered or unordered, and if a conflict between planned configuration group members shall invalidate the planned configuration group and prevent its activation.

The date and time at which information in the planned configuration group was modified the last time is provided as well.

### 6.2.5	Planned configuration group descriptor

The planned configuration group members and its meta data are specified in a planned configuration group descriptor. Each descriptor has a unique identifier.

## 6.3	Fallback configurations

### 6.3.1	Definition

Fallback is the process of returning to the configuration that was used by the managed system before the activation of a planned configuration or planned configuration group. A fallback configuration is the configuration describing this configuration.

Fallback configurations are generated by the system. Fallback may be applied when the activation of a planned configuration or a planned configuration group does not yield the desired results. Use cases include:

- A wrong planned configuration was activated. The operator wants to return to the old configuration.

- The new configuration does not perform as expected. The operator or the automation system wants to return to the old configuration.

- The new configuration was deployed on existing Network Functions in anticipation of some new Network Functions being rolled out. The roll out does not happen. The existing Network Functions are rolled back to the configuration state at the time the activation was triggered.

Fallback may be implemented by one of the following methods:

- "undo operations": This method only reverses the changes resulting from the activation of a planned configuration or planned configuration group. It has no effect on configuration data nodes that are not affected by the activation of the planned configuration or planned configuration group. Therefore, changes to other data nodes, that occur after activation and before fallback, are not undone.

- "restore complete configuration": This method reverts all configuration changes done after the activation of a planned configuration or planned configuration group, i.e. those resulting from the activation of a planned configuration or planned configuration group and those done via some other means. This method restores the complete old configuration. Therefore, changes to data nodes, that occur after activation and before fallback, are undone as well. Note that the complete data node tree is restored in this mode. Therefore, this mode may not be supported in deployment scenarios with large data node trees.

### 6.3.2	Fallback configuration preparation

A fallback configuration is created by the system upon the start of an activation process when enable fallback is requested. The format of the fallback configuration is dependent on the fallback method.

In case of "undo operations" the fallback configuration shall be visible to the consumer as a read-only planned configuration. A fallback configuration is always a single configuration (not a plannned configuration group) and is atomic. For example, for a planned configuration that adds a new object to the current data node tree the fallback configuration simply removes this object. Note that the fallback operations are not always simply the opposite of the operations contained in the activated planned configuration:

- Only successful operations shall be included in the fallback configuration.

- For removed objects, the fallback configuration needs to include the representation of the object in the current data node tree at the time of removal.

- The removal of objects that do not exist cannot result in the addition of an object, leave alone that there would be no representation for that object.

- The addition of an object that already exists shall not result in the removal of that object.

Editor's note: For "changeMOIs" it is not specified if the addition (operation "add") of an object or attribute, that already exists, is an error. Furthermore, it is not specified if the removal (operation "remove") of an object, that does not exist, is an error. This may have to be clarified. Maybe it is also dependent on the protocol.

In case of "restore configuration" the fallback configuration is implementation specific.

### 6.3.3	Meta data for fallback configurations

Each fallback configuration is described by meta data. This includes annotations such as the name, the version, and a human readable textual description.

Furthermore, an identifier of the activation process whose result is to be rolled back must be provided.

A fallback is always atomic.

### 6.3.4	Fallback configuration descriptor

Editor's note: This descriptor is ffs. A fallback configuration has the following differences compared to a normal planned configuration:

- apply mode is always atomic (plan descriptor)

- must include the id of the activation job (plan descriptor)

- planConfigContentType is set by the MnS producer and is read-only (plan descriptor)

- currentConfigAddress is set by the MnS producer and is read-only (plan descriptor)

- cannot be a member of a planned configuration group (group descriptor)

- cannot be activated by trigger conditions (trigger conditions)

- restore flavour cannot be validated (validation)

- no fallback is possible (activation)

- no cancelling is possible (activation)

## 6.4	Trigger conditions

### 6.4.1	Definition

A trigger condition is a set of definitions controlling the points in time when associated planned configurations and planned configuration groups are activated. The set of definitions typically includes expressions on data node trees that evaluate to true or false. Trigger conditions are evaluated permanently.

### 6.4.2	Usage

Trigger conditions are provided for conditional activation of planned configurations and planned configuration groups. The planned configuration is activated automatically when the provided conditions are met.

For example, additional performance metric collection jobs or assurance functions shall be created when metrics that are always collected cross certain thresholds.

### 6.4.3	Specification of trigger conditions

The trigger for activation is specified as follows:

- a condition expression evaluating to true or false.

- an evaluation period.

- a hysteresis that may require the evaluation to yield true for a specified time or a specified number of times before the trigger is activated.

The condition expression works on the data node tree of the current configuration. The notation used to express the condition depends on the MnS protocol used.The transition of the evaluation result of the trigger condition from false to true is considered as a trigger. The transition from true to false is not considered as a trigger.

The evaluation period determines how often the trigger condition is evaluated.

The hysteresis may be used to ensure that the trigger is activated only when the trigger condition evaluates to true for a certain time or a certain number of times. This prevents the trigger from being activated also when the evaluation result toggles between false and true.

Editor's note: Time based triggers are ffs.

6.4.4	Meta data for trigger conditions

Each trigger condition is described by meta data. This includes annotations such as the name, the version, and a human readable textual description.

6.4.5	Control parameters for trigger conditions

The trigger may be activated each time the result of the trigger condition evaluation changes from false to true or only once. A control parameter allows to configure this behavior.

6.4.6	Monitor parameters for trigger conditions

Monitor parameters are provided that display the current result of the evaluation, the data and time at which the trigger was last activated, and a Boolean indicating if the trigger is inactive for the case that only one activation is requested.

6.4.6	Trigger condition descriptor

The trigger condition descriptor includes the specification of the trigger condition itself, and associated meta-data, control parameters and monitor parameters.

## 6.5	Validation of planned configurations

### 6.5.1	Definition

Planned configurations may have problems and it may be impossible to activate them. Validation is the process to unveil potential problems with planned configurations before activation of the planned configuration.

The validation function has two input arguments: the operation set representing the planned configuration and a snapshot of the current configuration. The planned configuration is applied to the snapshot of the current configuration. The resulting configuration data node tree must comply to the (stage 2) NRM definitions and (stage 3) NRM schema definitions.

### 6.5.2	Potential problems

Validation discovers problems of planned configurations. These problems may be classified as follows:

- Information model problems

  - Problems of the planned configuration itself

  - Problems of the planned configuration in relationship to the current configuration

- Application layer problems

Examples for problems of the planned configuration itself:

- Update of an attribute with a value that does not match the data type defined for the value of the attribute

- Creation of an attribute name value pair that is not defined

- Update of an existing attribute that is defined as read-only

- Creation of an object with an object class that is not defined

- Creation of an object with a representation does not match the object class defined for the object

- Creation of an object under an object whose class does not support this containment

Examples for problems of the planned configuration in relationship to the current configuration:

- Creation of objects under parents that do not exist

- Deletion of leaf objects that do not exist

- Deletion of objects that are no leaves

- Violation of the cardinality property of an object

- Violation of the multiplicity property of an attribute

Examples for application layer problems

- An attribute value in the planned configuration is in the value range allowed by the attribute properties but the value is not allowed in the current state of the application.

### 6.5.3	Validation process

The purpose of validation is to detect problems of the planned configuration. As described in the previous clause, problems can be categorized into three groups.

Problems of the operations itself are detected by analyzing the operations together with the NRM schema. The current configuration is not involved in this process. Problematic operations are always invalid. If an atomic operation set contains at least one invalid operation, the complete planned configuration is invalid. If a non-atomic operation set contains a bad operation, only the operation is invalid. The operation set with the other operations may still be valid, dep ending on the validation result for the other operations.

Problems of the planned configuration in relationship to the current configuration are detected by applying the operations of the operation set one by one to a snapshot of the current configuration. This may result in intermediate states that do not comply to the constraints defined by the NRM schema. For example, an object of class A shall contain always exactly five objects of class B. To replace an object of class B, the object to be replaced may be deleted in the first step. In the second step the new object is added. The other possibility is to add the new object first and delete the object to be removed afterwards. In both cases the cardinality has a temporary invalid state. The validation process needs to take this into account: Problems of the planned configuration in relationship to the current configuration may be temporary only and be healed later when more operations are processed. A final assessment on the validity of operations can be made only after processing all operations.

In addition to problems of the planned configuration in relationship to the current configuration, problems of the planned configuration at application layer may occur. These problems are detected by evaluating the updates specified by the operations in the context of the configuration that is produced by applying the planned configuration against the current configuration. As for problems of the planned configuration in relationship to the current configuration, a final assessment on the validity of operations can be made only after processing all operations.

In the atomic mode for validation all operations must be valid for the planned configuration to be valid. If only one operation is invalid the complete planned configuration is invalid. The former case is also referred to as a successful validation and the latter as a failed validation.

In the best effort mode for validation all operations must be valid for the planned configuration to be valid. If at least one operation is invalid (but not all), the planned configuration is partially valid (or partially invalid). If all operations are invalid the planned configuration is invalid. These three cases are also referred to as successful validation, partially successful validation (or partially failed validation), and failed validation.

All constraint violations shall be reported.

A successful validation does not guarantee a successful activation.

### 6.5.4	Validation of planned configuration groups

The planned configuration group members are validated in the order specified. If no order is specified, they can be validated in any order. Each member is validated as specified in the previous clause. The validation of atomic members concludes with either validated or failed. The validation of best effort members concludes with either validated, partially validated or failed.

An atomic group is considered as valid, if atomic members are validat, and best effort members are validat or partially valid. If the validation of one (atomic or best effort) member fails, the validation of the complete group shall fail.

A best effort group is considered as valid, if atomic members are valid, and best effort members are valid or partially valid. If the validation of at least one (atomic or best effort) member fails, the group is considered as partially valid. If the validation of all members fails, the validation of the complete group shall fail.

### 6.5.5	Validation of fallback configurations

When the fallback configuration is the complete old configuration, the complete current configuration is replaced when activating the fallback configuration. In this case the concept of validation has no meaning. Therefore, the fallback configuration cannot be validated when the fallback configuration is represented by the complete old configuration.

When the fallback configuration is a set of undo operations, the fallback configuration should be validated like a planned configuration prior to activation.

### 6.5.6	Validation control parameters

Control parameters for validation are not specified.

Editor's note: Add "cancelRequest". A cancellation request stops an ongoing validation process. The state is set to "NOT_VALIDATED".

## 6.6	Activation of planned configurations

### 6.6.1	Definition

Activation is the process of applying a planned configuration to the managed system.

### 6.6.2	Potential problems

A planned configuration is validated before activation. Therefore a planned configuration that is activated should not have any of the problems that are discovered by validation. Only valid or partially valid planned configurations or planned configuration groups should be activated. Therefore, other types of problems are encountered during activation. These include

- A Network Function cannot be updated, because it is not reachable.

- A Network Function cannot be updated, because it is locked.

- A Network Function cannot be updated, because it does not exist.

- A Network Function cannot be updated, because it lacks necessary resources.

- A Network Function cannot be updated, because necessary access rights are not in place.

### 6.6.3	Activation process

Activation is the process of applying the updates specified by the operations contained in a planned configuration to the managed system. After the managed system is updated, the updates are reflected in the current configuration. Note that the system may not apply the operations to the managed system exactly as specified. For example, to change the configuration of a function, the function may have to be locked. Locking operations may not be included in the planned configuration and therefore the configuration update operation is augmented with locking and unlocking operations.

In atomic mode, the system may have to roll back to the current configuration that was effective when the activation process started. Therefore, the system needs to take and store a (conceptual) snapshot of the impacted part of the current configuration upon starting the activation process.

If not in atomic mode, the activation process may stop or continue when the first error occurs.

The activation process may run for quite some time, for example when large configurations are downloaded to the Network Functions as part of the activation process.

The planned configuration to be activated should be validated first.

Problems related to the activation of a planned configuration shall be reported.

### 6.6.4	Activation of planned configuration groups

The planned configuration group members are activated in the order specified. If no order is specified, they can be activated in any order. Each member is activated as specified in the previous clause. The activation of atomic members concludes with either activated or failed. The activation of best effort members concludes with either activated, partially activated or failed.

An atomic group is considered as activated, if atomic members can be activated, and best effort members can be activated or partially activated. If the activation of one (atomic or best effort) member fails, the activation of the complete group shall fail (and all updates must be undone).

A best effort group is considered as activated, if atomic members can be activated, and best effort members can be activated or partially activated. If the activation of at least one (atomic or best effort) member fails, the group is considered as partly activated. If the activation of all members fails, the activation of the complete group shall fail (and all updates must be undone).

### 6.6.5	Activation control parameters

Editor's note: Explain "enable-fallback", "service-impact and "immediate-activation". Also "cancel-request".

## 6.7	Conditional activation of planned configurations

## 6.8	Asynchronous configuration updates

Updating the current configuration may take some time, more time than an update operation based on a synchronous request response pattern, where the request provides the desired updates and the response information about the outcome of the requested updates, may be able to cope with. In a synchronous request-response pattern, the response is sent shortly after reception of the request.

This problem is addressed by so-called asynchronous update operations, where the request response pattern just provides the desired updates to the MnS producer. The response does not contain information about the outcome but a confirmation of reception of the desired updates. The information about the outcome of the update operations is provided asynchronously, i.e. later when the information is available.

Management of planned configuration includes as a special case also an asynchronous update operation. It is not mandatory to create a planned configuration before validation or activation. A new configuration can be provided also together with the validation or activation request. This is conceptually equivalent to an asynchronous update operation.

End of modifications