3GPP TSG-SA5 Meeting #160	S5-251706

Goteborg, Sweden, 07 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.156 | CR | 0110 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel18 CR TS 32.156 Enhancement of the description of the “isNotifyable” attribute property and correction of the default value for the “isReadable” attribute property |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2025-03-25 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The current description of the “isNotifyable” attribute property states that “It identifies if a notification shall be sent incase of a value change” which narrows the scope of this attribute property. - The default value for the “isReadable” attribute property is set to “False” |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Enhancement of the description of the “isNotifyable” attribute property to include all the scenarios in which this attribute property would be applicable. - Changing the default value for the “isReadable” attribute property to “True” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Leads to incoherrent specifications regarding the description of the “isNotifyable” attribute property vs it’s applicability - Inconsistencies in the default value for the “isReadable” attribute property |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 5.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Begin Change

### 5.2.1	Attribute

#### 5.2.1.1	Description

An attribute is a typed element representing a property of a class defined in (Unified Modelling Language (OMG UML), Infrastructure [1], clause 10.2.5). An element that is typed implies that the element can only refer to a constrained set of values. See OMG "Unified Modelling Language (OMG UML), Infrastructure" [1] clause 10.1.4 for more information on type.

See clauses 5.3.4 and 5.4.3 for predefined data types and user-defined data types that can apply type information to an attribute.

The properties of an attribute are described by a set of attribute properties categorized as follows:

-	Attribute properties defining valid attribute values: type, allowedValues, multiplicity, isOrdered, isUnique, isNullable, passedById.

-	Attribute properties defining valid interactions of managers and agents with attributes values: isInvariant, isWritable, isReadable, isNotifyable, defaultValue.

-	Other attribute properties: documentation, supportQualifier.

The following tables provide definitions for the attributes of the three categories.

Table 5.2.1.1-1: Attribute properties defining valid attribute values

| Property name | Description | Legal values |
| --- | --- | --- |
| type | Refers to a predefined (subclause 5.4.3) or user defined data type (section 5.3.4). See also subclause 7.3.44 of OMG "Unified Modelling Language (OMG UML), Superstructure" [2], inherited from StructuralFeature. | N/A |
| allowedValues | Specifies restrictions to the data type defined by type. This property is useful when no dedicated data type, that includes the restriction, shall be defined. The property may be absent when no restrictions are defined. | Dependent on type |
| defaultValue | Identifies a value at specification time that is used at object creation time under conditions defined in Annex B. If there is no defined default value, the property shall be omitted from the attribute description or specified as ‘defaultValue: None.’. | None (default) or a value that is dependent on allowedValues |
| multiplicity | Defines the number of values the attribute can simultaneously have. See subclause 7.3.44 of OMG "Unified Modelling Language (OMG UML), Superstructure" [2]; inherited from StructuralFeature. | See 5.2.8 Default is 1 |
| isOrdered | For a multi-valued multiplicity, this specifies if the values of this attribute instance are sequentially ordered. See subclause 7.3.44 and its Table 7.1 of OMG "Unified Modelling Language (OMG UML), Superstructure" [2]. If the property is present for attributes with a multiplicity of greater than “1”, it shall be set to either “True” or “False”. It shall not be set to “N/A”. | True, False (default) |
| isUnique | For a multi-valued multiplicity, this specifies if the values of this attribute instance are unique (i.e., no duplicate attribute values). See subclause 7.3.44 and its Table 7.1 of OMG "Unified Modelling Language (OMG UML), Superstructure" [2]. If the property is present for attributes with a multiplicity of greater than “1”, it shall be set to either “True” or “False”. It shall not be set to “N/A”. | True (default), False |
| isNullable | Identifies if an attribute can carry no information. The implied meaning of carrying “no information” is context sensitive and is not defined in this Model Repertoire. Note, the property "isNullable: True" is semantically identical to adding the value "0" to the "multiplicity" specified. Usage of the "multiplicity" property is preferred to express an attribute can have no value or carry no information. | True, False (default) |
| passedById | Usage of the value False is deprecated. The property is only applicable to attributes related to roles, for other attributes it has no significance, See Table 5.2.9.1-1: passedById property | True(default), False |
| lifecycleStatus | See Table 5.2.11.1-1 | Current (default), Deprecated |

Table 5.2.1.1-2: Attribute properties defining valid interactions with attributes

| Property name | Description | Legal values |
| --- | --- | --- |
| isInvariant | If an attribute has an "isInvariant: True" property, its value can be set only upon object creation. After object creation, the initial value cannot be modified by any entity. If an attribute has an "isInvariant: False" property, its value can be set at object creation time. After object creation, the initial value can be modified. Details on how initial values are provided upon object creation are specified in Annex B. | True, False (default) |
| isWritable | If an attribute has an "isWritable: True" property, a manager can set its value upon object creation. After object creation, a manager can modify the initial value if "isInvariant: False". If "isInvariant: True", a manager cannot modify the initial value. The "isInvariant" property supersedes hence the "isWritable" property. If an attribute has an "isWritable: False" property, a manager cannot set the value upon object creation nor modify it later. A "isWritable: True" property might be restricted by access control. | True, False (default) |
| isReadable | Specifies if the attribute can be read by a manager. A "isReadable: True" property might be restricted by access control. | True (default), False |
| isNotifyable | Identifies if the creation of the attribute value pair (which may or may not occur as part of an object creation), deletion of the attribute value pair (which may or may not occur as part of an object deletion) or attribute value change shall be notified. | True (default), False |

Table 5.2.1.1-3: Attribute properties related to the specification of attributes

| Property name | Description | Legal values |
| --- | --- | --- |
| documentation | Contains a textual description of the attribute. Should refer (to enable traceability) to the specific requirement. | Any |
| supportQualifier | Identifies the required support of the attribute. See also subclause 6. | M, O (default), CM, CO, C |

Upon completion of any manipulation of an attribute the attribute properties related to valid attribute values shall be respected. If an interaction results in violating at least one of these properties, the manipulation request shall be rejected.

The value N/A (Not applicable) shall not be used for attribute properties except for properties "isOrdered", "isUnique" and "allowedValues".

#### 5.2.1.2	Example

This example shows three attributes, i.e., a, b and c, listed in the attribute (the second) compartment of the class Xyz.

Figure 5.2.1.2-1: Attribute notation

#### 5.2.1.3	Name style

An attribute name shall use the LCC style.

Well Known Abbreviation (WKA) is treated as a word if used in a name. However, WKA shall be used as defined in the specification document that originally defined the WKA (its letter case cannot be changed) except when it is the first word of a name; and if so, its first letter must be in lower case.

End Change