3GPP TSG-SA5 Meeting #160	S5-251733

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0381 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.532 Add new notifications for reliability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson India Private Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBMA_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As decribed in TR 28.871 V19.0.0 (2024-12) clause 5.13 in some cases the producer is not able to provide reliable information to the consumer and may not be able to reliably prepare and send all the notifications that the consumer requested in NTFSubscriptionControl. Reasons for this may include, overload or internal errors of the producer or limitations in generating data change notifications in case of big configuration updates. If the producer detects such problems it should notify the consumer about the onset of the problems, but even more importantly when normal, reliable operations are resumed. The latter is important as this is the point where the consumer may take corrective actions to retrieve the data lost in the notifications. 2 notificatiosn are defined to serve this purpose. Based on TR 28.871 V19.0.0 (2024-12) clauses 5.13. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new notifications notifyDataNodeTreeUnreliable and notifyDataNodeTreeSynchRecommended |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No way to indicate unreliable notification transport caused by producer issues |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.1.x, 11.1.1.x.1, 11.1.1.x.2, 11.1.1.y, 11.1.1.y.1, 11.1.1.y.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 11.1.1.x	Notification notifyDataNodeTreeUnreliable

##### 11.1.1.x.1	Definition

This notification may be used by the producer to notify the MnS consumer, who has a subscription receiving this type of notification, that certain parts of the data node tree contain unreliable data. This can be caused by e.g. internal errors of the producer or loss of connection between the producer and underlying data sources/managed entities.

If the notification notifyDataNodeTreeUnreliable is supported by a producer, it shall also support the notifyDataNodeTreeSynchRecommended notification.

If the notification notifyDataNodeTreeUnreliable is sent by a producer, it shall also send the notifyDataNodeTreeSynchRecommended notification when the (potentially) faulty situation ends.

##### 11.1.1.x.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | See clause 11.0.2 |  |
| objectInstance | M | DN | Identifies the root object instance of the subtree for data which has become unreliable. |
| notificationId | M | See clause 11.0.2 |  |
| notificationType | M | "notifyDataNodeTreeUnreliable" |  |
| eventTime | M | See clause 11.0.2 |  |
| systemDN | M | See clause 11.0.2 |  |
| sequenceNo | CM | See clause 11.0.2 |  |
| subscriptionId | CM | See clause 11.0.2 |  |
| scopeType | O | enum ScopeType { BASE_ONLY, BASE_NTH_LEVEL, BASE_SUBTREE, BASE_ALL }; | The scope specifies the number of levels in the tree below the objectInstance which are affected by this notification. See also clause 11.1.1.2.2 If the parameter is not present its default value is BASE_ALL. |
| scopeLevel | O | Non-negative integer | Only relevant if scopeType is BASE_NTH_LEVEL or BASE_SUBTREE |

Next change

#### 11.1.1.y	Notification notifyDataNodeTreeSynchRecommended

##### 11.1.1.y.1	Definition

This notification should be used by the producer to notify the MnS consumer, who has a subscription receiving this type of notification, that it should synchronize (read) a certain section of the data tree. It may be sent if e.g.

- Notifications about the section of data were not prepared and thus not sent by the producer to the consumer, so the consumer is not up to date with the relevant data

- The data in the data node tree was unreliable due to internal problems of the producer or its underlying data sources.

The notification should be sent when the previous error condition no longer exists, and the consumer can read reliable data again.

If the error condition for a section of data exist only momentarily this notification should be sent to notify the consumer even if notifyDataNodeTreeUnreliable was not sent previously.

If the notification notifyDataNodeTreeUnreliable is supported by a producer, it shall also support the notifyDataNodeTreeSynchRecommended notification.

##### 11.1.1.y.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | See clause 11.0.2 |  |
| objectInstance | M | DN | Identifies the root object instance of the subtree for which data should be read by the consumer. |
| notificationId | M | See clause 11.0.2 |  |
| notificationType | M | "notifyDataNodeTreeSynchRecommended" |  |
| eventTime | M | See clause 11.0.2 |  |
| systemDN | M | See clause 11.0.2 |  |
| sequenceNo | CM | See clause 11.0.2 |  |
| subscriptionId | CM | See clause 11.0.2 |  |
| scopeType | O | enum ScopeType { BASE_ONLY, BASE_NTH_LEVEL, BASE_SUBTREE, BASE_ALL }; | The scope specifies the number of levels in the tree below the objectInstance which are affected by this notification. See also clause 11.1.1.2.2 If the parameter is not present its default value is BASE_ALL. |
| scopeLevel | O | Non-negative integer | Only relevant if scopeType is BASE_NTH_LEVEL or BASE_SUBTREE |

End of  changes