3GPP TSG-SA5 Meeting #160	S5-251741

Goteborg, Sweden, 07 - 11 April 2025

# Source:	Nokia

# Title:	Continuous MDT Discussion

# Document for:	Endorsement

Agenda Item:	6.1

# 1	Decision/action requested

The group is asked to discuss and endorse detailed proposals.

# 2	References

[1]	S5-251326 (R3-250891), “LS on Continuous MDT”, RAN3

[2]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace: Trace control and configuration management ".

# 3	Rationale

SA5 received an LS from RAN3 [1] regarding Continuous MDT which relates to using management based MDT for collection of MDT measurements targeting the same UE while it moves between different NG-RAN nodes and while it changes RRC states. According to the incoming LS, RAN3 has identified two problems statements to solve Continuous MDT:

- Measurement Continuity Problem: how to ensure that the same UE is selected for collection of MDT measurements across RRC states and during RRC Connected mobility.

- Trace Correlation Problem: how to ensure that the TCE receiving the Trace Record files containing MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection process from the same UE.

Related to the Measurement Continuity Problem, RAN3 agreed that:

- The OAM provides to the RAN means to identify that the Management Based MDT is for Continuous MDT, e.g. reusing existing Management Based MDT configuration information.

- RAN3 will derive solutions for the Measurement Continuity Problem across RRC Connected mobility. SA5 is requested to focus on the Measurement Continuity Problem across different RRC states.

- A solution should minimise impacts on the current Management Based MDT framework.

Related to the Trace Correlation problem, RAN3 agreed that it is required to use a temporary identifier for the Continuous MDT, not revealing the UE identity. Such identifier should be made available at the TCE to enable correlation of the management-based MDT reports collected during the Continuous MDT, in a way that minimises the impacts on the Management Based MDT solution.

RAN3 asked SA5 in the LS [1] whether the above is feasible to fulfil the established requirements and to solve the problem statements. Furthermore, RAN3 asked SA5 to provide guidance on possible solutions for Continuous MDT.

In the following, the different problems from the RAN3 LS and the different solutions are discussed

# 3	Discussion

## 3.1 Measurement Continuity Problem

SA5 is responsible for the trace control and configuration management, specified in TS 32.422[2]. This will include the configuration of management based MDT for Continuous MDT and in turn, providing themeans to RAN to identify that a Management based MDT session is for Continuous MDT. SA5 can confirm RAN3 understanding that OAM provides the means to identify that a Management based MDT Configuration is for Continuous MDT.

Proposal 1:	OAM provides the means to identify that a Management based MDT session is for Continuous MDT.

SA5 will need to determine how OAM configures Management based Continuous MDT to the RAN. From SA5 perspective we can identify various approaches, e.g.:

- A list of Trace References is reserved for Management-based Continuous Immediate MDT and a different list of Trace References is reserved for Management-based Continuous Logged MDT.

- A new attribute is introduced in Management-based MDT Configuration. The attribute indicates the configuration is for “Continuous MDT”.

- One or several new attribute value(s) are introduced for attribute Job Type such as “Continuous Immediate MDT”, “Continuous Logged MDT” or “Continuous Immediate and Logged MDT”. However, this has an impact on backwards compatibility and is therefore not recommended from our point of view. Furthermore, introducing “Continuous Immediate and Logged MDT” would additionally change the assumption that one instance of Trace Job applies for a single MDT mode.

For solutions 2 and 3, SA5 will need to also discuss how a gNB can correlate two MDT configurations, one for Immediate and one for Logged MDT and determine that they need to be linked in Continuous MDT operation. Namely, a gNB that receives multiple Immediate and Logged MDT Configurations from OAM, it needs to be able to determine that Continuous MDT is for a certain pair of Immediate and Logged MDT Configurations.

Proposal 2: SA5 needs to discuss how a gNB can determine that a given Immediate and a given Logged MDT Configuration at a gNB need to be linked for Continuous MDT operation.

However, before SA5 can conclude how to configure Continuous MDT, SA5 needs clarification of the below queries:

- Which scenario shall be considered for “Continuous MDT”?
We identify following possible scenario:
Management based Immediate and Logged MDT is configured in a gNB. While the UE is in RRC Connected state, the gNB selects a UE for Continuous MDT and configures the UE with an Immediate and a Logged MDT Configuration. Immediate MDT is activated while the UE is in RRC Connected state. The UE changes from RRC Connected state to RRC Idle /RRC Inactive state, the UE stops Immediate MDT measurements and starts logging Logged MDT measurements according to the Logged MDT configuration. When the UE comes back to RRC Connected state, the gNB will need to reconfigure the UE with the “same” Immediate  MDT Configuration since upon transition to RRC Idle state the Immediate MDT Configuration will be erased from the UE. The UE will continue measurements in RRC Connected State according to the configured Immediate MDT Configuration.

- Which measurements shall be supported by “Continuous MDT”?

The description about measurement continuity is with respect to a given UE and across different RRC states. However, Logged MDT complies with the principles for RRC Idle and RRC Inactive state measurements in the UE and Immediate MDT complies with the principles for RRC Connected state measurements in the UE. SA5 needs clarification whether data collection corresponding to Continuous MDT assumes that there exist at least one UE measurement that is common across RRC Idle/RRC Inactive and RRC Connected states (e.g., RSRP)  or  whether it is possible to have Continuous MDT when the UE is configured to provide different measurements in different RRC states (e.g., provide Average Delay measurement in RRC Connected state and RSRP measurement in RRC Idle state).

- Which reporting/logging intervals shall be supported by “Continuous MDT”?
A given Immediate MDT measurement that is reported periodically can be reported with different periods (ReportInterval) e.g., 120ms, 240ms, 640ms, etc. Similarly, a Logged MDT configuration for the same measurement may have different logging durations or logging intervals supported. It is unclear how continuity is defined as per RAN3 understanding and whether there can be equivalent configurations (Immediate or Logged MDT) with respect to continuity e.g., when the same measurement is reported or logged under different reporting or logging intervals. This question is relevant since it would define how OAM can indicate Continuous MDT to the RAN, according to our related discussion to Proposal 2. For example, if Continuous MDT is indicated through a list of (equivalent) Trace References, a gNB would need to be able to determine which TRs are equivalent for the purpose of continuity.

- Which reporting triggers shall be supported by “Continuous MDT”?
In NR it can be configured in Immediate MDT and in Logged MDT whether measurements shall be performed periodically or event based. However, the trigger events are vary for the different measurements. SA5 needs clarification whether and which reporting triggers shall apply for Continuous MDT

Proposal 3: 	Ask RAN3 the following questions:
- 	Is the SA5 understanding of the “Continuous MDT” scenario correct?
- 	Is Continuous MDT only when there is at least one measurement that is common for Logged and Immediate MDT, e.g., at least RSRP measurements are reported by a UE in Immediate MDT?
- 	Can there be equivalent configurations where a given measurement is configured with possibly different reporting/logging intervals or reporting triggers?

Another aspect that needs to be evaluated is the area scope of Continuous MDT. We believe that SA5 would need to determine whether the area scope of the continuous MDT job can be the same as the area in which the NG-RAN selects UEs for continuous MDT. E.g., if the area scope is very large, e.g., PLMN-wide, selection of UEs within the same area scope might lead to a too high number of UEs being selected. Such scenarios should be avoided since they would require complex OAM coordination with significant configuration burden if the continuous MDT solution requires all gNBs within the area scope to be configured with the continuous MDT job.  It is also important to avoid such scenarios in order to avoid unnecessary UE impacts by requesting UE measurements from an unnecessary high number of UEs.

In order to solve the problem, we propose to define a configuration area:

Configuration area: Area within which NG-RAN nodes will select UEs for a given continuous MDT job.

It can also be useful to clarify the term “Area Scope” in the context of continuous MDT, which would be the area within which a UE that is selected for continuous MDT will collect the continuous MDT data.

As one example, NG-RAN nodes within a city (“configuration area”) may be configured to select UEs for a given continuous MDT job, while the area scope of the continuous MDT job could be PLMN-wide, meaning that the selected UEs may move within the full PLMN coverage area while collecting the continuous MDT data.

Proposal 4:	 For Continuous MDT, it shall be differentiated between “area scope” presenting the area in which the UE and network will collect Continuous MDT data and “configuration area” presenting the area within which NG-RAN nodes will select UEs for a given Continuous MDT job.

Another question that needs to be addressed is how to maintain the Continuous MDT configuration during UE transitions from RRC Idle state to RRC Connected state. We can think of the following solutions:

- Solution 1: When a UE configured for Continuous MDT reconnects from RRC Idle state, the NG-RAN node uses OAM provisioned information to configure the UE with corresponding Immediate MDT.

- Solution 2: Immediate MDT configuration is stored in the UE (since it will be otherwise erased upon transition to RRC Idle state).

- Solution 3: Immediate MDT configuration is stored in the core network.

Solution 1 will require that the continuous MDT configuration area (as defined above) corresponds to the full area scope of the continuous MDT session, and this solution should therefore be excluded based on the discussion above. Concerning solution 2, according to the LS from RAN3, Continuous MDT should be enabled by a solution applicable to legacy UEs and not only to Rel-19 UEs. Therefore, solution 2 should also be excluded.

Therefore, we think that storing of the Immediate MDT configuration to the context of the UE in the Core Network seems to be the simplest approach.

Proposal 5:	In order to maintain the Continuous MDT configuration during UE transitions from RRC Idle state to RRC Connected state Immediate MDT configuration is stored in the core network.

Due to the Core Network impacts of this solution, we think that SA2 needs to be copied in the reply LS to RAN3.

Proposal 6:	SA5 adds SA2 in CC in the reply LS to RAN3 so that they are informed about the Continuous MDT discussions.

## 3.2 Trace Correlation Problem

Trace Records contain the Trace Reference (TR) and the Trace Recording Session Reference (TRSR) of the MDT Configuration. We identify two options to solve the Trace Correlation problem:

- A gNB sends to the TCE the TR/TRSR of the Logged MDT session and the TR/TRSR of the Immediate MDT session that need to be correlated at the TCE.

- Reuse of Cell Traffic Trace mechanism. This means that a gNB sends CELL TRAFFIC TRACE message to the AMF with anonymization level set to a new identifier for trace correlation every time it allocates a TR/TRSR for Immediate or Logged MDT. This identifier is allocated by the AMF and is not passed to the RAN in order to protect user identity. Every time a gNB allocates a TR/TRSR for Logged or Immediate MDT the AMF sends to the TCE the correlation identifier together with the TR/TRSR of the Logged or Immediate MDT configuration. In order to support repeated selection of the UE for continuous MDT, SA5 assumes that the correlation identifier has a lifetime corresponding to the duration of the UE context within the CN. The correlation identifier should not enable the TCE to identify a specific UE identity.

The latter solution is the most aligned with the current MDT framework and therefore, it is our preferred solution.

Proposal 7: SA5 shall discuss and agree on the following  solution for trace correlation:

- Reuse of Cell Traffic Trace mechanism. This means that a gNB sends CELL TRAFFIC TRACE message to the AMF with anonymization level set to a new identifier for trace correlation every time it allocates a TR/TRSR for Immediate or Logged MDT. This identifier is allocated by the AMF. Every time a gNB allocates a TR/TRSR for Logged or Immediate MDT the AMF sends to the TCE new correlation identifier together with the TR/TRSR of the Logged or Immediate MDT configuration.

Proposal 8: 	SA5 shall send a reply LS to RAN3 asking for clarification of above discussed questions and informing about discussed and agreed solutions.

# 4	Detailed proposal

Proposal 1:	OAM provides the means to identify that a Management based MDT session is for Continuous MDT.

Proposal 2:	SA5 needs to discuss how a gNB can determine that a given Immediate and a given Logged MDT Configuration at a gNB need to be linked for Continuous MDT operation.

Proposal 3: 	Ask RAN3 the following questions:
- Is the SA5 understanding of the “Continuous MDT” scenario correct?
- Is Continuous MDT only when there is at least one measurement that is common for Logged and Immediate MDT, e.g., at least RSRP measurements are reported by a UE in Immediate MDT?
- Can there be equivalent configurations where a given measurement is configured with possibly different reporting/logging intervals or reporting triggers?

Proposal 4:	 For Continuous MDT, it shall be differentiated between “area scope” presenting the area in which the UE and network will collect Continuous MDT data and “configuration area” presenting the area within which NG-RAN nodes will select UEs for a given Continuous MDT job.

Proposal 5:	In order to maintain the Continuous MDT configuration during UE transitions from RRC Idle state to RRC Connected state Immediate MDT configuration is stored in the core network.

Proposal 6:	SA5 adds SA2 in CC in the reply LS to RAN3 so that they are informed about the Continuous MDT discussions.

Proposal 7: SA5 shall discuss and agree on the following  solution for trace correlation:

- Reuse of Cell Traffic Trace mechanism. This means that a gNB sends CELL TRAFFIC TRACE message to the AMF with anonymization level set to a new identifier for trace correlation every time it allocates a TR/TRSR for Immediate or Logged MDT. This identifier is allocated by the AMF. Every time a gNB allocates a TR/TRSR for Logged or Immediate MDT the AMF sends to the TCE new correlation identifier together with the TR/TRSR of the Logged or Immediate MDT configuration.

Proposal 8: 	SA5 shall send a reply LS to RAN3 asking for clarification of above discussed questions and informing about discussed and agreed solutions.