3GPP TSG-SA5 Meeting #160	S5-251896

Goteborg, Sweden, 07 - 11 April 2025 	Revision of S5-251253

Source:	Nokia, Ericsson

Title:	Pseudo-CR on NDT concept and scope

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TS28.561

Version:	0.1.0

Work Item:	Management aspects of Network Digital Twins

Comments

This pCR is to Add a description of NDT concept and scope

Proposed Changes

* * * First Change * * * *

# 4	Concepts and overview

## 4.X	Introduction and Overview

## 4.X.Y	NDT Life-cycle management

An NDT is an analytical function that can provide simulation/emulation capabilities to be used by the network automation functions to obtain information useful in operation decision-making.  An NDT may have multiple simulation/emulation jobs (here called NDT jobs) each considering a different network scenario and use case. The NDT is an object that can be managed, the LCM includes LCM of the NDT function instance and of the NDT job instances. Accordingly, the following capabilities are included:

-	NDT Creation: The MnS producer who provides the NDT simulation/emulation capabilities creates an NDT function instance that is capable to model a specific scenario.

-	NDT job instantiation: NDT MnS producer receives the request to create a task instance or job. The NDT MnS producer instantiates and executes the simulation/emulation. The NDT job instance can be configured by the MnS consumer at any time.

-	NDT job suspension: NDT MnS producer receives the request to pause or suspend a session task instance or job.

- 	NDT job deletion: NDT MnS producer receives the request to delete a simulation/emulation task instance/job. The NDT MnS producer stops the execution of the simulation/emulation and deletes the NDT job instance.

-	NDT Deletion: the MnS producer may delete an NDT Function instance that is not active.

NOTE: All of the life cycle operations may not be valid for all NDT Function instances. For example, a particular NDT instance may not support suspending the execution.

## 4.3 NDT Management Service

The NDT MnS can be requested by the NDT MnS consumer to model the behaviour of the network and generate the simulation/emulation output. The simulation/emulation capabilities provided by NDT MnS Producer can be invoked by the network intelligence and automation management functions (e.g. MDAF, AIML Inference Function, Intent Handling Function, etc.) playing the role of NDT MnS Consumer to support their intelligence and automation functionality.

Figure 4.1-1

* * * End of Changes * * * *