3GPP TSG-SA5 Meeting #160	S5-252002

Goteborg, Sweden, 07 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0683 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.552 Add use case descriptions on monitoring of NWDAF data collection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NWDAF_OAM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-3-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The NWDAF Data Collection feature permits NWDAF to retrieve data from various sources (e.g. NF such as AMF, SMF, etc.), as a basis of the computation of network analysis. The procedures of NWDAF performing the NWDAF data collection is defined in TS 23.288 clause 6.2. In a period of time, if the data collected from one specific data source comprises hundreds of time of collection, but for each time of the aforesaid collection, only a small amount of data is collected, this will dramatically increase the accumulated overhead of the transmission for NWDAF data collection and these overhead may become an extra burden which needs to be considered for deploying NWDAF. It is meaningful for the operators to consider both the amount of the collected data and the corresponding accumulated transmission overhead related to the procedures of the NWDAF data collection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Annex A.114 is modified to add use case descriptions related to efficiency of data collection of NWDAF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operator will not be able to monitor the data efficiency of NWDAF and to take NWDAF management actions for configuration, resource allocation, load balancing and placement purposes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | S5-252002 was S5-251340 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

5.18	Performance measurements for NWDAF

5.18.5	Measurements related to the NWDAF Data Collection

Annex A (informative):
Use cases for performance measurements

A.114		Monitoring of NWDAF data collection

The Data Collection feature permits NWDAF to retrieve data from various data sources, which can be NF such as AMF, SMF, PCF, UDM, AF, UPF or OAM, as a basis of network analytics (See TS 23.288 [59]). Data Collection related measurements, such as the amount and the frequency of the data collection from different data sources, can be used to optimize the deployment of NWDAF and improve the other services provided by NWDAF. For example, based on the Data Collection related measurements, the NWDAF instance may be recommended to be geographically deployed closer to its major data source to reduce the latency and save network resources.

Therefore, the Data Collection related measurements are important factors that reflect the NWDAF Data Collection performance. Since the Data Source and the services used are different, the measurements related to different data sources need to be monitored respectively.

By monitoring the number of subscriptions/notifications for Data Collection services in a period, the operators can know how often the NWDAF collects data from the same data source and the differences from different data sources . These measurements can be used to infer the performance of NWDAF on data collection.

By monitoring the amount of data collected by the NWDAF from a specific data source over a period of time, the operators can know the total amount of data collected by the NWDAF from all data sources over a period of time, which reflects the NWDAF workload. The operators can also know whether the data collection from this specific data source is the primary factor influencing the workload of the NWDAF by comparing it with the total amount of data, so as to evaluate and optimize the entire NWDAF data collection.

In the other hand, the efficiency considering the overhead related to the transmission of the data collected by the  NWDAF needs to be monitored. For example, in a period of time, there are hundreds of time of data collection performed by NWDAF from one data source, but for each time, only a small amount of data is collected. In this case,  the accumulated overhead of NWDAF data collection is significant comparing to the amount of data collected. It will become an unexpected burden and needs to be reduced. Eventhrough the transmission overhead are not considered as a part the data collected by NWDAF from the data source, it is still meaningful for the operators to evaluate the NWDAF data collection efficiency reflecting the impact of transmission overhead.

Network resource is occupied by the NWDAF to perform the data collection from its data source. The NWDAF and its data sources may not be deployed close to each other from the transmission point of view, the transmission rate can be measured as the ratio between a certain amount of data collected by NWDAF and the time used for data transmission from the data source to the NWDAF. A low or a decreasing transmission rate may indicate that there is a congestion or a bottleneck in the network resource occupied by the NWDAF to perform the data collection from this data source during the observation. In such cases, providing alternative network resources, limiting the usage of NWADF at some time periods or deploying an (additional) NWDAF closer to this data soruce can be considered for the operators.

In the other hand, the efficiency considering the overhead related to the transmission of the data collected by the  NWDAF needs to be monitored. For example, in a period of time, there are hundreds of time of data collection performed by NWDAF from one data source, but for each time, only a small amount of data is collected. In this case,  the accumulated overhead of NWDAF data collection is significant comparing to the amount of data collected. It will become an unexpected burden and needs to be reduced. Eventhrough the transmission overhead are not considered as a part the data collected by NWDAF from the data source, it is still meaningful for the operators to evaluate the NWDAF data collection efficiency reflecting the impact of transmission overhead.

Network resource is occupied by the NWDAF to perform the data collection from its data source. The NWDAF and its data sources may not be deployed close to each other from the transmission point of view, the transmission rate can be measured as the ratio between a certain amount of data collected by NWDAF and the time used for data transmission from the data source to the NWDAF. A low or a decreasing transmission rate may indicate that there is a congestion or a bottleneck in the network resource occupied by the NWDAF to perform the data collection from this data source during the observation. In such cases, providing alternative network resources, limiting the usage of NWADF at some time periods or deploying an (additional) NWDAF closer to this data soruce can be considered for the operators.

Moreover, according to TS 23.288 [2] clause 6.2.6, NWDAF is able to directly collect data from NFs or collect data via DCCF or an NWDAF hosting DCCF. Therefore, the case where NWDAF collects data via DCCF or an NWDAF hosting DCCF can also be considered.

End of Changes