3GPP TSG-SA5 Meeting #160	S5-252081

Goteborg, Sweden, 07 - 11 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.557 | CR | 0016 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.557 Update procedure of management of tenant to align with access control solution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OAM_NPN_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The solution description in clause 6.1.3 doesn’t align with TS 28.319. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the solution procudure of management of tenant to align with access control solution specified in TS 28.319 - Remove the editor’s note and add reference to TS 28.319 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with TS 28.319 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.1.3, B.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[3]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[4]	3GPP TS 22.261: "Service requirements for the 5G system".

[5]	5G-ACIA White paper: "5G Non-Public Networks for Industrial Scenarios", July 31, 2019.

[6]	3GPP TS 23.003: "Numbering, addressing and identification".

[7]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[8]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[9]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[10]	3GPP TS 38.473: "NG-RAN; F1 Application Protocol (F1AP)".

[11]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[12]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[13]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[14]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[15]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM); Integration Reference Point (IRP); Information Service (IS)".

[16]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[17]	3GPP TS 28.537: "Management and orchestration; Management capabilities".

[18]	5G-ACIA: Exposure of 5G Capabilities for Connected Industries and Automation Applications, https://5g-acia.org/whitepapers/exposure-of-5g-capabilities-for-connected-industries-and-automation-applications-2/.

[19]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

[20]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace; Trace data definition and management".

[21]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

[22]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains; Stage 1"

[23]	3GPP TS 28.536: "Management and orchestration; Management services for communication service assurance; Stage 2 and stage 3"

[m]	3GPP TS 28.319: " Management and orchestration; Access Control for Management services "

Next Change

6.1.3	Solution for management of NPN service customer

The NPN service customer (i.e. NPN-SC) is a role defined for NPN management to represent the CSC (e.g. verticals) which consumes communication services for non-public use (see clause 4.2). The management of NPN-SC focuses on the granular access control of NPN-SC (e.g., tenant that defined in clause 4.8 of TS 28.533 [21]), which covers the following:

-	Enable the authentication of NPN-SC.

-	Enable the authorization of NPN-SC. This authorization is used to define the specific MnS information (e.g., MnS type and MnS components) the NPN-SC is allowed/disallowed to consume.

Figure 6.1.3-1 shows the procedure of management of NPN-SC.

Figure 6.1.3-1: Procedure of management of NPN-SC

The pre-condition of the procedure is that the authentication/authorization information of the NPN-SC is configured to MASC producer.

1)	The MnS consumer, which represents NPN-SC, gets authenticated by MSAC producer. MSAC producer validates the identity and credential of MnS consumer.

2)  The MnS consumer request authorization from MSAC producer. MSAC producer provides role-based access control on management services in the granularity of MnS component type (see TS 28.319 [m]).

3)	MSAC producer performs authorization to MnS consumer.

4)  MSAC producer returns authorization results to MnS consumer.

Next Change

B.5	Procedure for management of tenant

The following PlantUML source code is used to describe the procedure of management of the tenant, as depicted by Figure 6.1.3-1:

@startuml

skinparam NoteBackgroundColor White

participant "MnS Consumer" as cmer

participant "MSAC Producer" as prov

cmer <-> prov: 1. authentication

cmer -> prov: 2. request for authorization

prov -> prov: 3. perform the authorization

prov -> cmer: 4. return authorization results

@enduml

End of changes