3GPP TSG-SA5 Meeting #161	S5-252301

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | 0543 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.623 Update Attribute definition for TimeWindow |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Timewindow is defined as follows: - When startTime and endTime is present or configured by MnS consumer, the time window starts when startTime is reached and ends when endTime is reached. - When only the startTime attribute is present or configured by MnS consumer, the time window starts when startTime is reached and runs until deletion of the managed object instance including this timeWindow. - When only the endTime attribute is present or configured by MnS consumer, the time window starts when the managed object instance including this TimeWindow is created and ends when endTime is reached. It implies both startTime and endTime are mandatory attributes to be supported by MnS producer. However, the Timewindow is defined as choice for different run time conditions instead of design time conditions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update Timewindow as an <<dataType>> includes two mandatory attributes startTime and endTime. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect definition for Timewindow |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Following YAML files normatively defined in the forge are updated: - OpenAPI/TS28623_ComDefs.yaml |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1723 at commit ebf194e4f44bfc3dd2f5f1c18b1a0b3809f4f252 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1723 at commit ebf194e4f44bfc3dd2f5f1c18b1a0b3809f4f252

*** START OF CHANGE 1 ***

*** OpenAPI/TS28623_ComDefs.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: Common Type Definitions

version: 19.3.0

description: >-

OAS 3.0.1 specification of common type definitions in the Generic NRM

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.623; Generic NRM; Common type definitions

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.623/

paths: {}

components:

schemas:

Float:

description: This datatype is used for writable attribute

type: number

format: float

FloatRo:

description: This datatype is used for readOnly attribute

type: number

format: float

readOnly: true

DateTime:

description: This datatype is used for writable attribute

type: string

format: date-time

DateTimeRo:

description: This datatype is used for readOnly attribute

type: string

format: date-time

readOnly: true

FullTime:

type: string

description: format (e.g., full-time) is from RFC 3339

format: full-time

DateMonth:

type: string

format: date-month

DateMonthDay:

type: string

format: date-mday

DayInYear:

type: object

properties:

month:

$ref: '#/components/schemas/DateMonth'

monthDay:

$ref: '#/components/schemas/DateMonthDay'

Latitude:

type: number

format: float

minimum: -90

maximum: 90

Longitude:

type: number

format: float

minimum: -180

maximum: 180

Dn:

description: This datatype is used for writable attribute

type: string

DnRo:

description: This datatype is used for readOnly attribute

type: string

readOnly: true

DnList:

description: This datatype is used for writable attribute

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/Dn'

DnListRo:

description: This datatype is used for readOnly attribute

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/DnRo'

Mcc:

description: This datatype is used for writable attribut

type: string

pattern: '^[0-9]{3}$'

MccRo:

description: This datatype is used for readOnly attribute

type: string

pattern: '^[0-9]{3}$'

readOnly: true

Mnc:

description: This datatype is used for writable attribut

type: string

pattern: '^[0-9]{2,3}$'

MncRo:

description: This datatype is used for readOnly attribute

type: string

pattern: '^[0-9]{2,3}$'

readOnly: true

Nid:

type: string

pattern: '^[A-Fa-f0-9]{11}$'

PlmnId:

description: This datatype is used for writable attribute

type: object

properties:

mcc:

$ref: '#/components/schemas/Mcc'

mnc:

$ref: '#/components/schemas/Mnc'

PlmnIdRo:

description: This datatype is used for readOnly attribute

type: object

properties:

mcc:

$ref: '#/components/schemas/MccRo'

mnc:

$ref: '#/components/schemas/MncRo'

Tac:

type: string

pattern: '(^[A-Fa-f0-9]{4}$)|(^[A-Fa-f0-9]{6}$)'

UtraCellId:

type: integer

EutraCellId:

type: string

pattern: '^[A-Fa-f0-9]{7}$'

NrCellId:

type: string

pattern: '^[A-Fa-f0-9]{9}$'

TimeWindow:

description: This datatype is used for writable attribute

type: object

properties:

startTime:

$ref: '#/components/schemas/DateTime'

endTime:

$ref: '#/components/schemas/DateTime'

TimeWindowRo:

description: This datatype is used for readOnly attribute

type: object

properties:

startTime:

$ref: '#/components/schemas/DateTimeRo'

endTime:

$ref: '#/components/schemas/DateTimeRo'

GeoCoordinate:

type: object

properties:

latitude:

$ref: '#/components/schemas/Latitude'

longitude:

$ref: '#/components/schemas/Longitude'

altitude:

$ref: '#/components/schemas/Float'

GeoArea:

oneOf:

- type: object

properties:

geoPolygon:

type: array

items:

$ref: '#/components/schemas/GeoCoordinate'

minItems: 1

GeoAreaToCellMapping:

type: object

properties:

geoArea:

$ref: '#/components/schemas/GeoArea'

associationThreshold:

type: integer

AreaOfInterest:

oneOf:

- type: array

items:

$ref: '#/components/schemas/GeoAreaToCellMapping'

- type: array

items:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/Tai'

minItems: 1

maxItems: 8

- type: array

items:

$ref: '#/components/schemas/NrCellId'

minItems: 1

maxItems: 32

- type: array

items:

$ref: '#/components/schemas/EutraCellId'

minItems: 1

maxItems: 32

- type: array

items:

$ref: '#/components/schemas/UtraCellId'

minItems: 1

maxItems: 32

Fqdn:

type: string

description: This datatype is used for writable attribute

pattern: '^([0-9A-Za-z]([-0-9A-Za-z]{0,61}[0-9A-Za-z])?\.)+[A-Za-z]{2,63}\.?$'

minLength: 4

maxLength: 253

FqdnRo:

type: string

description: This datatype is used for readOnly attribute

pattern: '^([0-9A-Za-z]([-0-9A-Za-z]{0,61}[0-9A-Za-z])?\.)+[A-Za-z]{2,63}\.?$'

minLength: 4

maxLength: 253

readOnly: true

Ipv4Addr:

description: This datatype is used for writable attribute

type: string

pattern: '^(([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])\.){3}([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])$'

example: '198.51.100.1'

Ipv4AddrRo:

description: This datatype is used for readOnly attribute

type: string

pattern: '^(([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])\.){3}([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])$'

example: '198.51.100.1'

readOnly: true

Ipv6Addr:

description: This datatype is used for writable attribute

type: string

allOf:

- pattern: '^((:|(0?|([1-9a-f][0-9a-f]{0,3}))):)((0?|([1-9a-f][0-9a-f]{0,3})):){0,6}(:|(0?|([1-9a-f][0-9a-f]{0,3})))$'

- pattern: '^((([^:]+:){7}([^:]+))|((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))$'

example: '2001:db8:85a3::8a2e:370:7334'

Ipv6AddrRo:

description: This datatype is used for readOnly attribute

type: string

allOf:

- pattern: '^((:|(0?|([1-9a-f][0-9a-f]{0,3}))):)((0?|([1-9a-f][0-9a-f]{0,3})):){0,6}(:|(0?|([1-9a-f][0-9a-f]{0,3})))$'

- pattern: '^((([^:]+:){7}([^:]+))|((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))$'

example: '2001:db8:85a3::8a2e:370:7334'

readOnly: true

Ipv6Prefix:

description: This datatype is used for writable attribute

type: string

allOf:

- pattern: '^((:|(0?|([1-9a-f][0-9a-f]{0,3}))):)((0?|([1-9a-f][0-9a-f]{0,3})):){0,6}(:|(0?|([1-9a-f][0-9a-f]{0,3})))(\/(([0-9])|([0-9]{2})|(1[0-1][0-9])|(12[0-8])))$'

- pattern: '^((([^:]+:){7}([^:]+))|((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))(\/.+)$'

example: '2001:db8:abcd:12::0/64'

Ipv6PrefixRo:

description: This datatype is used for readOnly attribute

type: string

allOf:

- pattern: '^((:|(0?|([1-9a-f][0-9a-f]{0,3}))):)((0?|([1-9a-f][0-9a-f]{0,3})):){0,6}(:|(0?|([1-9a-f][0-9a-f]{0,3})))(\/(([0-9])|([0-9]{2})|(1[0-1][0-9])|(12[0-8])))$'

- pattern: '^((([^:]+:){7}([^:]+))|((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))(\/.+)$'

example: '2001:db8:abcd:12::0/64'

readOnly: true

IpAddr:

description: This datatype is used for writable attribute

oneOf:

- $ref: '#/components/schemas/Ipv4Addr'

- $ref: '#/components/schemas/Ipv6Addr'

- $ref: '#/components/schemas/Ipv6Prefix'

IpAddrRo:

description: This datatype is used for readOnly attribute

oneOf:

- $ref: '#/components/schemas/Ipv4AddrRo'

- $ref: '#/components/schemas/Ipv6AddrRo'

- $ref: '#/components/schemas/Ipv6PrefixRo'

HostAddr:

#  This definition will be deprecated, when all occurances of HostAddr

#  are replaced by Host.

oneOf:

- $ref: '#/components/schemas/Ipv4Addr'

- $ref: '#/components/schemas/Ipv6Addr'

- $ref: '#/components/schemas/Fqdn'

Host:

oneOf:

- $ref: '#/components/schemas/IpAddr'

- $ref: '#/components/schemas/Fqdn'

HostRo:

oneOf:

- $ref: '#/components/schemas/IpAddrRo'

- $ref: '#/components/schemas/FqdnRo'

Uri:

description: This datatype is used for writable attribute

type: string

UriRo:

type: string

description: This datatype is used for readOnly attribute

readOnly: true

TimeInterval:

type: object

properties:

intervalStart:

$ref: '#/components/schemas/FullTime'

intervalEnd:

$ref: '#/components/schemas/FullTime'

AdministrativeState:

type: string

enum:

- LOCKED

- UNLOCKED

default: LOCKED

BasicAdministrativeState:

type: string

enum:

- LOCKED

- UNLOCKED

OperationalState:

type: string

enum:

- ENABLED

- DISABLED

default: DISABLED

readOnly: true

UsageState:

type: string

readOnly: true

enum:

- IDLE

- ACTIVE

- BUSY

AvailabilityStatus:

type: string

enum:

- IN_TEST

- FAILED

- POWER_OFF

- OFF_LINE

- OFF_DUTY

- DEPENDENCY

- DEGRADED

- NOT_INSTALLED

- LOG_FULL

readOnly: true

AttributeNameValuePairSet:

description: >-

The key of this map is the attribute name, and the value the attribute value.

type: object

minProperties: 1

additionalProperties:

nullable: true

AttributeValueChangeSet:

description: >-

The first array item contains the attribute name value pairs with the new values,

and the second array item the attribute name value pairs with the optional old values.

type: array

items:

$ref: '#/components/schemas/AttributeNameValuePairSet'

minItems: 1

maxItems: 2

Filter:

description: >-

The filter format shall be compliant to Jex (TS 32.161).

type: string

JexConditions:

type: string

description: >-

The value of the string shall be a Jex expression that is compliant to the

Jex conditions profile specified in clause 7.6 of TS 32.161.

JexNodeSelectionBasic:

type: string

description: >-

The value of the string shall be a Jex expression that is compliant to the

Jex basic profile specified in clause 7.4 of TS 32.161.

JexNodeSelectionAdvanced:

type: string

description: >-

The value of the string shall be a Jex expression that is compliant to the

Jex advanced profile specified in clause 7.5 of TS 32.161.

SystemDN:

type: string

NotificationId:

type: integer

readOnly: true

NotificationType:

oneOf:

- $ref: 'TS28111_FaultNrm.yaml#/components/schemas/AlarmNotificationTypes'

- $ref: 'TS28532_ProvMnS.yaml#/components/schemas/CmNotificationTypes'

- $ref: 'TS28532_PerfMnS.yaml#/components/schemas/PerfNotificationTypes'

- $ref: 'TS28532_HeartbeatNtf.yaml#/components/schemas/HeartbeatNotificationTypes'

- $ref: 'TS28532_FileDataReportingMnS.yaml#/components/schemas/FileNotificationTypes'

NotificationHeader:

type: object

properties:

href:

$ref: '#/components/schemas/Uri'

notificationId:

$ref: '#/components/schemas/NotificationId'

notificationType:

$ref: '#/components/schemas/NotificationType'

eventTime:

$ref: '#/components/schemas/DateTime'

systemDN:

$ref: '#/components/schemas/SystemDN'

sequenceNo:

type: integer

subscriptionId:

type: string

required:

- href

- notificationId

- notificationType

- eventTime

- systemDN

ErrorResponse:

description: >-

Default schema for the response message body in case the request

is not successful.

type: object

properties:

error:

type: object

properties:

errorInfo:

type: string

Subscription:

type: object

properties:

consumerReference:

$ref: '#/components/schemas/Uri'

timeTick:

type: integer

filter:

$ref: '#/components/schemas/JexConditions'

ErrorResponseGet:

description: >-

Default schema for the response message body in case the Get

is not successful.

type: object

properties:

status:

type: string

type:

type: string

reason:

type: string

title:

type: string

badQueryParams:

type: array

items:

type: string

otherProblems:

type: array

items:

type: object

properties:

status:

type: string

type:

type: string

reason:

type: string

title:

type: string

badQueryParams:

type: array

items:

type: string

required:

- type

required:

- type

ErrorResponseDefault:

type: object

properties:

status:

type: string

type:

type: string

reason:

type: string

title:

type: string

badAttributes:

type: array

items:

type: string

badObjects:

type: array

items:

type: string

otherProblems:

type: array

items:

type: object

properties:

status:

type: string

type:

type: string

reason:

type: string

title:

type: string

badAttributes:

type: array

items:

type: string

badObjects:

type: array

items:

type: string

required:

- type

required:

- type

ErrorResponsePatch:

description: >-

Default schema for the response message body in case the patch

is not successful.

type: object

properties:

status:

type: string

type:

type: string

reason:

type: string

title:

type: string

badOP:

type: string

otherProblems:

type: array

items:

type: object

properties:

status:

type: string

type:

type: string

reason:

type: string

title:

type: string

badOp:

type: string

required:

- type

- badOp

required:

- type

- badOp

<CODE ENDS>

*** END OF CHANGE 1 ***

End of Changes