3GPP TSG-SA5 Meeting #161	S5-252360

Fukuoka, Japan, 19 - 23 May 2025

Source:	Nokia

Title:	Pseudo-CR on CCL Concurrent actions conflicts

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS28.567

Version:	0.2.0

Work Item:	Closed Control Loop Management

Comments

This pCR is to add use case and requirements for CCL scope conflicts handling as was agreed in the CCL study in TR28. 867

Proposed Changes

* * * First Change * * * *

# 5	Management Capabilities

## 5.7	CCL Conflict management Capability - CONF

### 5.7.1	Description

A CCL may experience direct conflicts on its goals, targets, scopes, trigger time and execution time. The management system needs to support capabilities to avoid, detect and resolve the conflicts.

### 5.7.2	Use Cases

#### 5.7.2.1	CCL Trigger-time conflicts handling - CONF_01

Typically, a CCL whose start is triggered based on conditions, needs to be triggered to run at a specific time and terminate when certain conditions are met, to run when a certain performance threshold is crossed. If triggered independently, there may be conflicts among the CCLs. The triggers for different CCLs to be executed need to be coordinated to avoid conflicts among the CCLs.

The management system should support the capability for avoidance of action-execution-time conflicts. Since each CCL focuses on a smaller scope of the network problem space, several CCLs may need to be executed.  For actions in a given network scope, the CCLs can be explicitly scheduled by the management system. Where the scopes overlap, the CCLs need to align the action plans, for example, which action plan to execute and when. There is a need to assess each plan and choose the most appropriate combination of action plan(s) based on the selection policy and then notify the selected action plan(s) to the related CCLs. The MnS consumer may also be notified when it is safe to ignore the conflict. The MnS consumer may configure the criteria for evaluating the severity of conflicts.

#### 5.7.2.2	CCL action conflicts handling –CONF_02

When two (or more) CCLs attempt to adjust the same network parameter but with different and contradicting values, the desired actions of the 2 CCL will be in conflict. For example, a CCL assuring throughput of a slice may be scaling-out the virtual resources of the slice. Whereas a CCL minimizing the energy consumption may be scaling-in the virtual resource of the same slice. It can be when the CCLs execute actions at the same time. However, it also happens when the CCLs execute at different times, and the scenario for actions to be separated in time is the more likely than actions occurring simultaneously. casein these conflict scenarios, the network parameter continuously ping-pongs between the two values. Such a conflict may be called an action conflict.

NOTE:	A potential conflict can for example be detected if a CCL observed that PMs on a certain object keep flipping between two values. The constant flipping can be an indication that 2 CCL instances are attempting to change the same scope.

The CCL may detect or observe events that identify the conflicts. The conflict can be avoided using some information or the policies (e.g. priority) provided by the consumer. If the conflict actually occurs, the CCL MnS producer should support services to inform MnS consumers the confirmed detected conflicts. It is needed to maximize the avoidance of conflict, including “requesting” information from MnS consumer and to inform MnS consumer about the potential conflict. CCL MnS Producer may also provide recommendations, for updating/deleting the conflicting CCLs, that would result in the resolution of detected conflict. The recommendation for update may include suggestions for modified targets.

Editor’s Note:	The exact information that can be exchanged is FFS

#### 5.7.2.3	CCL Indirect-target conflicts handling – CONF_03

Two (or more) CCLs configuring different control parameter may all influence the same metric. In other cases, the two CCLs influence two metrics Y1 and Y2 that are couple, i.e., which have a logical relationship between them. E.g. handover (HO) failure and SINR are coupled since a bad SINR can lead to more HO failures. If the two CL desire different values for the metric, or different values for two target metrics Y1 and Y2 but the targets are coupled, the CCLs are in conflict for the metric resulting into a target-metric-value conflict or indirect target conflicts.

Two target metrics Y1 and Y2 may be coupled such that actions to optimize any of them lead to correlated oscillations/degradations in Y1 or Y2, e.g. Y1 ensuring "HO failure is < 2 %" and Y2 wanting "SINR > 10dB". The correlated oscillations indicate a potential conflict, but the CCLs may not see the oscillations in the metric that is not of their interest. The management system should support the capability for detecting potential indirect targets conflicts. An MnS consumer may analyse the correlations to detect the potential conflict between CCL1 and CCL2. The MnS consumer should be able to inform CCL1 and CCL2 about the detected potential conflict represented by the correlated oscillations.

This severity of degradation in the performance metrics of the related CCLs could be the confirmation that a detected potential conflict is an actual harmful conflict. The management system should support the capability for detecting or confirming actual indirect targets conflicts. The threshold to determine the severity may be defined by the MnS consumer (e.g. the operator) so that if the degree of degradation is higher than the threshold then it is a confirmed conflict that requires resolution.

The management system should support the capability for avoiding potential indirect target conflicts. CCLs need to avoid large and frequent changes to network parameters which may affect network stability since they increase the probability of occurrence of conflicts. CCLs should take small smooth changes in the cases where the impact is not so clear and only make the large changes when the CCL is sure that the impact is positive. It is desirable for the CCL to notify to the MnS consumer the planned change, its claimed/predicted performance improvement and reliability/confidence in that action/decision. The MnS consumer may evaluate the claimed performance improvement and reliability/confidence to determine if the action should be allowed or not. The MnS consumer should be enabled notify the decision and possibly the failed criteria to the CCL - to either be executed or to be used to compute better decisions. Based on the inputs, the CCL may update its decision-making and repeat the decision evaluation process. If the CCL has consistently made good large-action-decisions, the MnS consumer should be enabled to inform the CCL that the CCL has consistently made good decisions and achieved its ultimate trust and that no more coordination of its decisions is needed.

The management system should support the capability for resolving detected indirect targets conflict. The MnS consumer should be enabled to trigger one or more CCLs to respond to the detected potential conflict. And if the triggered CCLs is unable to resolve that conflict, the CCL should inform the MnS consumer about the failure to resolve the problem. The MnS consumer can set the thresholds for performance degradation that triggers conflict detection and resolution.

Editor’s Note 1: The criteria for accurately setting the thresholds for performance degradation is FFS.

Editor’s Note 2: The name and description of this type of conflict will be revisited.

#### 5.A.2.Y	CCL Concurrent actions conflicts handling - CONF_0y

Several CCLs may want to execute actions onto the network. It may not be desirable that their actions are executed within the same time frame. For example, if executed so close to one another, their effects will be super-imposed and neither CCL can identify the effect of its actions on the network.

The management system should support the capability for detection of potential concurrent actions conflicts, also called action-execution-time conflicts. An MnS consumer acting as a supervisory action-critic oversees the actions of the different CCLs may need to receive information enabling the detection of such conflicts. The action-critic functionality takes the for the end-to-end performance of the Network enabling evaluation of cases when the actions of multiple CCLs collide.

For a given CCL, the MnS consumer may need to receive the recommended changes from the CCLs, to evaluates them and see if they overlap with other proposed changes from other CCLs. Where there are likely conflicts and expected undesired impacts, the MnS consumer may propose to the CCLs, the changes that should be undertaken to minimize concurrent changes on the same network resources. The MnS consumer may need to provide feedback to the CCL instance (s) regarding their recommended actions.

In some instances, the conditions in the network may be such that it is not clear which CCL should be triggered, requiring to trigger multiple CCL in sequence. The CCLs may operate in a hierarchy with each CCL having an operational profile indicating the specific level of hierarchy. The MnS consumer that coordinates the execution times of the CCLs needs to configure the appropriate hierarchy for the CCLs. The triggering by a coordination capability based on information from the CCL allows resolution of CCL Trigger-time conflicts.

### 5.7.3	Requirements

Table 5.7.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-CONF_01-01 | The 3GPP Management System should support a capability to detect and inform an authorized MnS consumer about a potential or actual CCL trigger-time conflicts. | CONF_01 |
| REQ-CONF_01-02 | The 3GPP Management System should support a capability to confirm and inform an authorized MnS consumer about a detected CCL trigger-time conflict after it is confirmed. | CONF_01 |
| REQ-CONF_01-03 | The 3GPP Management System should enable authorized MnS Consumer to provide information that can be used to support a capability to avoid or resolve a CCL trigger-time conflict. | CONF_01 |
| REQ-CONF-CONF_02-01 | The 3GPP Management System should support a capability to detect and inform an authorized MnS consumer about a potential action conflict. | CONF-CONF_02 |
| REQ-CONF-CONF_02-02 | The 3GPP Management System should support a capability to confirm and inform an authorized MnS consumer about an actual action conflict. | CONF-CONF_02 |
| REQ-CONF-CONF_02-03 | The 3GPP Management System should enable authorized MnS consumers to provide information that can be used to resolve a CCL action conflict. | CONF-CONF_02 |
| REQ-CONF-CONF_02-04 | The 3GPP Management System should enable authorized MnS consumers to provide information that can be used to avoid the action conflict. | CONF-CONF_02 |
| REQ-CONF-CONF_02-05 | The 3GPP Management System should support a capability to coordinate the resolution of CCL action conflicts among multiple CCLs | CONF-CONF_02 |
| REQ-CONF_03-01 | The 3GPP Management System should support a capability to detect and inform an authorized MnS consumer about a potential or actual CCL Indirect targets conflicts. | CONF-CONF_03 |
| REQ-CONF_03-02 | The 3GPP Management System should support a capability to confirm and inform an authorized MnS consumer about a detected CCL Indirect targets conflict after it is confirmed. | CONF-CONF_03 |
| REQ-CONF_03-03 | The 3GPP Management System should support a capability to avoid or resolve a CCL Indirect targets conflict that has been detected | CONF-CONF_03 |
| REQ- CONF_0y-01 | The 3GPP Management System should support a capability to detect and inform an authorized MnS consumer about a potential or actual CCL action-execution-time targets conflicts. | CONF-CONF_0y |
| REQ-CONF_0y-02 | The 3GPP Management System should support a capability to confirm and inform an authorized MnS consumer about a detected CCL action-execution-time targets conflict after it is confirmed. | CONF-CONF_0y |
| REQ-CONF_0y-03 | The 3GPP Management System should support a capability to avoid or resolve a CCL action-execution-time targets conflict that has been detected | CONF-CONF_0y |
| REQ-CONF_0y-04 | The 3GPP Management System should support a capability enabling the MnS consumer to configure a hierarchy of a CCL | CONF-CONF_0y |

* * * End of Changes * * * *