3GPP TSG-SA5 Meeting #161	S5-252371

Fukuoka, Japan, 19 - 23 May 2025

Source:	Nokia, CMCC

Title:	Pseudo-CR on NRM for automation & Verification NDTs

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TS28.561

Version:	0.2.0

Work Item:	Management aspects of Network Digital Twins

Comments

This pCR is to add NRM for the NDT supporting automation and verification  following agreements in the NDT study.

Proposed Changes

* * * First Change * * * *

## 6.2	Information model for NDT (MnS component typeB)

#### 6.2.1.3	Class definition

##### 6.2.1.3.2	NDTJob <<InformationObjectClass>>

###### 6.2.1.3.2.1	Definition

This IOC represents the properties of an NDT job demand created by an MnS consumer.

###### 6.2.1.3.2.2	Attributes

The NDTJob IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.2.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| networkResponseTask | M | T | T | F | T |
| networkConfigTask | M | T | T | F | T |
| autoFunctionConfigTask | M | T | T | F | T |
| Attribute related roles |  |  |  |  |  |
| ndtFunctionRef | M | T | T | F | T |
| ndtReportRefList | M | T | T | F | T |

##### 6.2.1.3.3	NDTReport <<InformationObjectClass>>

###### 6.2.1.3.3.1	Definition

This IOC represents the properties of an NDT report corresponding to an NDT job.

The attribute "PerformanceData" is specified performance metrics and/or alarm types that are collected and reported by NDT after the behaviour is modelled in NDT.

###### 6.2.1.3.3.2	Attributes

The NDTReport IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.3.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attribute related roles |  |  |  |  |  |
| ndtJobRef | M | T | T | F | T |
| ndtFunctionRef | M | T | T | F | T |

##### 6.2.1.3.y		NetworkEvent 	<<dataType>>

###### 6.2.1.3.y.1	Definition

This defines criteria and conditions characterizing a network event.

###### 6.2.1.3.y.2	Attributes

The NetworkEvent <<datatype>> includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.Z.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| performanceEventInfo | M | T | T | F | T |
| provisioningEventInfo | M | T | T | F | T |
| faultEventInfo | M | T | T | F | T |

###### 6.2.1.3.Z.3	Attribute constraints

None.

###### 6.2.1.3.Z.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC.

6.3.z	FaultEventInfo <<datatype>>

6.3.z.1	Definition

This will define various fault related information that need to be modelled by the NDT instance .

6.3.z.2	Attributes

Table 6.2.1.3.Z.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| targetNode | M | T | T | F | T |
| alarmSeverityThreshold | M | T | T | F | T |
| alarmTypeThreshold | M | T | T | F | T |

###### 6.2.1.3.Z.3	Attribute constraints

None.

###### 6.2.1.3.Z.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC.

## 6.3	Attribute definitions

### 6.3.1	Attribute properties

Table 6.3.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| ndtJobRef | It indicates an DN of a NDTJob Instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtFunctionRef | It indicates an DN of a NDTFunction Instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtReportRefList | It indicates a list of DN for NDTReport Instances. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| networkResponseTask | It indicates a network issue or an event for which the network response needs to be evaluated. A network issue contains one or more network events. The networkResponseTask can be used for - Verification of network response to one or more events - evaluation of the impact of one or more failure events, e.g. a signalling storm - Evaluating one or more network issue, e.g. a coverage issue. The network issues involves one or more network events. | type: NetworkEvent multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| networkConfigTask | It indicates a configuration of the network that needs to be evaluated in the NDTjob. It is a DN of a planned configuration as defined in TS 28.572 [X]. The networkResponseTask can be used for - evaluation of high-risk network operations which are listed within the planned configuration - Verification of network configurations which are listed within the planned configuration | type: PlannedConfigurationDescriptor multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| autoFunctionConfigTask | It indicates a configuration of an automation function that needs to be evaluated in the NDTjob. It is a DN of a planned configuration as defined in TS 28.572 [X] | type: PlannedConfigurationDescriptor multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| performanceEventInfo | This defines the information related with performance measurements and KPIs that characterize a network event. This is defined as a Jex string supporting the following attributes: - Target Node: The identification of the Managed Object for which the performance is captured. - Measurement/KPI Name: Name/identification of the performance measurement and KPI - Trigger Value: Trigger value of the measurement. | Type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| provisioningEventInfo | This defines provisioning events that can be introduced into the NDT. This is defined as a Jex string supporting the following attributes: - Target Node: This can be a particular IOC or a DN. E.g Intent IOC - Provisioning Location: The CCL will be created only when the object created is targeting a specific location. - Provisioning Event (Create, Modify, Delete): The CCL will be created when the given event occur on the given IOC/DN. - Provisioning Time: The CCL will be created only when the given event occur at a specified time. | Type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| faultEventInfo | This defines the information related with network faults that characterize a network event. | Type: FaultEventInfo multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| targetNode | This defines the node which need to be monitored for the emitted alarms (i.e objectInstance in AlarmInformation). | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| alarmSeverityThreshold | This defines the “perceivedSeverity” threshold (i.e threshold for each Severity). If total number of alarms, belonging to particular perceivedSeverity (e.g critical, major etc.), goes beyond the threshold,. | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| alarmTypeThreshold | This defines the “AlarmType” threshold (i.e threshold for each AlarmType). If total number of alarms, belonging to a particular alarmType, goes beyond the threshold. | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

* * * Next Change * * * *

Annex A (informative):
PlantUML source code

## A.1	Information model definition for NDT

### A.1.1	Relationship UML diagram for NDT management

@startuml

hide empty members

skinparam ClassStereotypeFontStyle normal

hide circle

skinparam class {

BackgroundColor White

ArrowColor Black

BorderColor Black

}

'skinparam linetype ortho

'skinparam BoxPadding 40

skinparam nodesep 2

class ManagedEntity <<ProxyClass>>

class NDTFunction <<InformationObjectClass>>

class NDTJob <<InformationObjectClass>>

class NDTReport <<InformationObjectClass>>

note right of ManagedEntity

Represents SubNetwork

end note

ManagedEntity "1" *-- "*" NDTFunction: <<names>>

NDTFunction "1" --> "*" NDTFunction

NDTFunction "1" *-- "*" NDTJob: <<names>>

NDTFunction "1" *-- "*" NDTReport: <<names>>

NDTJob "1" -r-> "*" NDTReport

@enduml

### A.1.2	Inheritance UML diagram for NDT management

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class Top << InformationObjectClass >>

class ManagedFunction << InformationObjectClass >>

class NDTFunction <<InformationObjectClass>>

class NDTJob <<InformationObjectClass>>

class NDTReport <<InformationObjectClass>>

ManagedFunction <|-- NDTFunction

Top <|-- NDTJob

Top <|-- NDTReport

@enduml

* * * End of Changes * * * *