3GPP TSG-SA5 Meeting #161	S5-252415

Fukuoka, Japan, 19 - 23 May 2025

Source:	China Mobile, Nokia

Title:	Pseudo-CR on TS 28.561 NDT relation to automation

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TS 28.561

Version:	0.2.0

Work Item:	NDT

Comments

This pCR is to add the description of NDT relation to automation and NDT LCM.

Proposed Changes

* * * First Change * * * *

4	Concepts and overview

4.x	Relations of NDTs and network automation functions

Existing automation capabilities include analytics services and decision-making capabilities with the assumption that the knowledge about network behaviour is available within the automation functions. Accordingly, the unaddressed gap for network automation is the capabilities for modelling the behaviour of the network. The Network Resource Model (NRM) is used to describes the attributes of a mobile network and their relationships. The concept of Network Digital Twin adds the ability to model the behaviour of a mobile network. Therefore, the digital twins can provide modelling capabilities that are used by the network automation functions (e.g. MDA, SON, etc.) to accomplish their automation functionality. When the network digital twin provides modelling capabilities for the network, it can be integrated inside or outside of existing network automation functions. - see figure 4.x-1.

Option 1	NDT internal/integrated into existing Network Automation Functions.

Option 2	NDT separate from existing Network Automation Functions

Figure 4.1.3-1: Relation of NDTs with network automation functions

NOTE:	The double headed arrows indicate candidate flow of data or control while one headed arrows indicate only flow of data.

4.x	NDT Life-cycle management

An NDT is an analytical function that can provide simulation/emulation capabilities to be used by the network automation functions to obtain information useful in operation decision-making.  An NDT may have multiple simulation/emulation jobs (here called NDT jobs) each considering a different network scenario and use case. The NDT is an object that can be managed, the LCM includes two aspects: the LCM of the NDT function instance and the LCM of the NDT job instances. Accordingly, the following capabilities are included:

-	NDT Creation: The MnS producer who provides the NDT simulation/emulation capabilities creates an NDT function instance that is capable to model a specific scenario.

-	NDT job instantiation: NDT MnS producer receives the request to create a task instance or job. The NDT MnS producer instantiates and executes the simulation/emulation. The NDT job instance can be configured by the MnS consumer at any time.

-	NDT job suspension: NDT MnS producer receives the request to pause or suspend a session task instance or job.

- 	NDT job deletion: NDT MnS producer receives the request to delete a simulation/emulation task instance/job. The NDT MnS producer stops the execution of the simulation/emulation and deletes the NDT job instance.

-	NDT Deletion: the MnS producer may delete an NDT Function instance that is not active.

NOTE: All of the life cycle operations may not be valid for all NDT Function instances. For example, a particular NDT instance may not support suspending the execution.

* * * End of Changes * * * *