3GPP TSG-SA5 Meeting #161	S5-252493

Fukuoka, Japan, 19 - 23 May 2025

# Source:	Huawei

# Title:	Moderated R20 topic discussion on use cases for charging reliability enhancement

# Document for:	Discussion

Agenda Item:	7.2

1	Decision/action requested

To collect uses cases for charging reliability enhancement.

2	References

[1]	S5-250168, SA5 Rel-20 5GA Topics Consolidated List

[2]	3GPP TS 32.290 Telecommunication management; Charging management; Charging Data Record (CDR) parameter description

[3]	3GPP TS 32.255 Telecommunication management; Charging management; Charging Data Record (CDR) parameter description

[4]	GSMA NG.113 5GS Roaming Guidelines v12.0

3	Rationale

The charging reliability enhancement is a candidate SA5 Rel-20 5GA Topic in S5-250168 [1].

This DP is a starting point for the moderated discussion on this topic, and can focus on collecting use cases that requires charging reliability enhancement.

4	Detailed proposal

Use Case #1 The CTF detected failure scenarios

In Rel-18, the CTF detected failure handling mechanism has been extended to allow the possibility of storing charging information or Charging Data Request(s), as following:

“In case the CHF is determined not reachable, the CTF uses application level failure handling and may store Charging Data Request(s) or charging information. Once the connection to a CHF is established by the NF consumer (CTF), the CHF may receive the Charging Data Request(s) or charging information that were previously stored by NF consumer (CTF).”  --- TS 32.290 [2] clause 5.5.1.1 (CR 0225)

However, in case the CHF is determined not reachable, the resource at CHF may be freed (i.e. charging session is released) or not (i.e. charging session is kept).

-	Scenario #1: When the resource is freed, how to establish another charging session and send the charging data request(s) or charging information that were previously stored by CTF?

-	Scenario #2: When the resource is not freed, i.e. the charging session at the CHF is still kept, how to send the charging information once the connection is re-established, e.g. forward the Charging Data Request(s) in sequence or accumulate the charging information into one Charging Data Request?

Use Case #2 The inter-CHF failure scenarios

In the LBO roaming, there are two charging architectures specified, i.e. N40+N47 in Figure 4.2.6 and N40+N107 in Figure 4.2.6a in TS 32.255 [3]. According to the 5GS Roaming Guidelines defined in GSMA NG.113 [4], the support of N47 is not recommended.

However, the mechanism for the inter-CHF communication may not be completely ready for live production environment. There are a few perspectives to be further studied:

-	Scenario #1: In case of CTF failure, e.g. not reachable to deliver the Charging Data Response from V-CHF, how to handle the reserved resources for the charging session between V-CHF and H-CHF?

-	Scenario #2: In case of V-CHF failure, e.g. not reachable to deliver the Charging Data Request/Response from CTF/H-CHF, how to handle the reserved resources for the charging session between V-CHF and H-CHF or between CTF and V-CHF?

-	Scenario #3: In case of H-CHF failure, e.g. not reachable to the Charging Data Request from V-CHF, how to handle the reserved resources for the charging session between CTF and V-CHF?

-	Scenario #4: The unaligned information elements between CHFs, e.g. unused unit timeout, may lead to failure, e.g. one resource is released while the other is still kept.