3GPP TSG-SA5 Meeting #161	S5-252570

Fukuoka, Japan, 19 - 23 May 2025

Title:	reply LS on OAM-centric solution for NW-side data collection

Response to:	R2-2503170

Release:	Rel-19

Work Item:	AIML_MGT_Ph2

Source:	SA5

To:	RAN2

Cc:	RAN3

Contact person:	shixiaoli

shixiaoli@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

SA5 would like to thank RAN2 for their LS in R2-2503170/S5-25338 on OAM-centric solution for NW-side data collection.

For the RAN2 agreements:

“RAN2 assumes to only adopt management based immediate MDT for NW-side data collection.

For NR-DC, the configuration and reporting of NW sided data collection can only be via MN.”

SA5 answer: SA5 has defined the Management based immediate MDT in TS 32.422. The parameter can have the following values in NR as defined in clause 5.10.3 in TS 32.422:

-	M1: DL signal quantities measurement results for the serving cell and for intra-frequency/Inter-frequency/inter-RAT neighbour cells, including cell/beam level measurement by UE.

-	M2: Power headroom (PH) measurement by UE

-	M3 is not supported by this release

-	M4: PDCP SDU Data volume measurement separately for DL and UL, per DRB per UE by gNB

-	M5: Average UE throughput measurement separately for DL and UL, per DRB per UE and per UE for the DL, per DRB per UE and per UE for the UL, by gNB

-	M6: Packet delay measurement, separately for DL and UL, per DRB per UE by gNB

-	M7: Packet loss rate measurement, separately for DL and UL, per DRB per UE by gNB

-	M8: RSSI measurement by UE for WLAN and Bluetooth®

-	M9: RTT measurement by UE for WLAN

-	And any combination of above

SA5 confirm the current Management based immediate MDT procedure defined in TS 32.422 can be reused to support RAN2 NW-side data collection. SA5 can extend new values accordingly if there are new RAN2 requirements regarding NW-side data collection.

2	Actions

To RAN2 and RAN3

ACTION: 	SA5 respectfully asks RAN2 and RAN3 to take the above information into account.

3	Dates of next TSG SA WG5 meetings

SA5#162		25 August - 29 August 2025		Goteborg, Sweden

SA5#163		13 October - 17 October 2025		China