3GPP TSG-SA5 Meeting #161	S5-252593

Fukuoka, Japan, 19 - 23 May 2025

# Source:	NTT DOCOMO, Orange, SK Telecom, FiberCop, CMCC

# Title:	Issue on supporting vendor specific value in NDT

# Document for:	Endorsement

Agenda Item:	6.19.5.1

1	Decision/action requested

The group is asked to endorse the proposal.

2	References

[1]	3GPP TS 28.561 "Management and orchestration;Management aspects of Network Digital Twins"

3	Rationale

During the SA5#160 meeting, a proposal regarding IOCs related to NDT was discussed. In the contribution, an attribute named "supportedNdtCapabilities" is defined within the NDT Function IOC, which represents the simulations and emulations supported by the NDT Function. Additionally, the NDT Job IOC includes an attribute called "ndtJobType," specifying the types of simulations and emulations that the NDT consumer requests from the NDT Function.

While candidate values such as "FAILURE_PREDICTION" were provided for these attributes, it was also noted that "The list can be extended with vendor-specific values." During the SA5#160 meeting, it was agreed not to add these attributes.

Below is an excerpt from the proposal that pertains to this issue.

6.2.1.3.1	NDTFunction <<InformationObjectClass>>

6.2.1.3.1.1	Definition

This IOC represents the properties of an NDT Function.

The attribute "supportedNdtCapabilities" indicates the capabilities that are support by the NDTFunction instance. The ENUM value is FAILURE_PREDICTION ,FAULT_INJECTION, ML-TRAINING_DATA_GENERATION, RISKY-ACTIONS_PREDICTION, EVENTS-IMPACTS_VERIFICATION, NETWORK_CONFIGURATIONS_VERIFICATION, AUTOMATION_CONFIGURATION_VERIFICATION.

6.2.1.3.1.2	Attributes

The NDTFunction IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [b]) and the following attributes.

Table 6.2.1.2.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedNdtCapabilities | M | T | F | F | T |

6.2.1.3.2	NDTJob <<InformationObjectClass>>

6.2.1.3.2.1	Definition

This IOC represents the properties of an NDT job demand created by an MnS consumer.

The attribute "ndtJobType" indicates the service type requested by MnS consumer. The ENUM value is FAILURE_PREDICTION ,FAULT_INJECTION, ML-TRAINING_DATA_GENERATION, RISKY-ACTIONS_PREDICTION, EVENTS-IMPACTS_VERIFICATION, NETWORK_CONFIGURATIONS_VERIFICATION, AUTOMATION_CONFIGURATION_VERIFICATION.

X.2.1.3.2.2	Attributes

The NDTJob IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [b]) and the following attributes.

Table 6.2.1.2.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ndtJobType | M | T | T | F | T |
| Attribute related roles |  |  |  |  |  |
| ndtFunctionRef | M | T | T | F | T |
| ndtReportRefList | M | T | T | F | T |

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| supportedNdtCapabilities | It indicates the different types of simulations/emulations which the NDTFunction can support. Candidate values include: “FAILURE_PREDICTION“, “FAULT_INJECTION“, “ML-TRAINING_DATA_GENERATION“, “RISKY-ACTIONS_PREDICTION“, “EVENTS-IMPACTS_VERIFICATION“, “NETWORK_CONFIGURATIONS_VERIFICATION“, “AUTOMATION_CONFIGURATION_VERIFICATION“ The meaning of these values is as described in clause 5. The list can be extended with vendor specific values. | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ndtJobType | It indicates the type of simulation/emulation that is desired to be executed. allowedValues: “FAILURE_PREDICTION“, “FAULT_INJECTION“, “ML-TRAINING_DATA_GENERATION“, “RISKY-ACTIONS_PREDICTION“, “EVENTS-IMPACTS_VERIFICATION“, “NETWORK_CONFIGURATIONS_VERIFICATION“, “AUTOMATION_CONFIGURATION_VERIFICATION“ The meaning of these values is as described in clause 5. The list can be extended with vendor specific values. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

-----

"The list can be extended with vendor specific values" has following issues.

If vendor specific values are used, other vendor's consumer cannot use the functionality, and may cause malfunction. Therefore, it costs to the operators to modify their automation function such as CCL to prevent malfunction in multi-vendor environment. Also, it may lead to reduce company’s motivation to standardize new NDT capability, because it is already SA5 compatible with vendor specific capabilities.

For example, if an operator use vendor-a's CCL and vendor-b's NDT and vendor-b's NDT provide "NEW-CAPABILITY-A" in supportedNdtCapabilities, vendor-a's CCL may not recognize this value. Moreover, if vendor-a's CCL interprets this value differently, CCL unintentionally triggers NDT job, leading to malfunctions. This scenario suggests that we cannot use this specification in multi-vendor environment if vendor specific value are used.

Also, if vendor specific values are not stated in SA5 spec, vendors can support the functionality outside of the SA5 interface. (vendor's UI to run functionalities, vendor's own interfaces). Therefore, Operators can use new functionalities which are not standardized using vendor's own interface.

4	Detailed proposal

It is proposed to introduce "supportedNdtCapabilities" and "ndtJobType" without stating "The list can be extended with vendor specific values.".