3GPP TSG-SA5 Meeting #161	S5-252615

Fukuoka, Japan, 19 - 23 May 2025

Source:	Ericsson France S.A.S

Title:	Rel-19 pCR TS 28.561 Clarify NDT management service

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	TS 28.561

Version:	0.2.0

Work Item:	NDT

Comments

Description in 4.2 is not correct. MnS consumer cannot request an NDT MnS but request is directed to MnS producer. Further some statements in 6.4 needs correction, for ex. receiving a request does not mean the activation of NDT. Updating abbreviations.

Proposed Changes

First change

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

NDT	Network Digital Twin

S-NSSAI	Single Network Slice Selection Assistance Information

Next change

## 4.2	NDT Management Service

NDT MnS enables an authorized consumer to request modelling a network scenario for analysis and for receiving the output report. The simulation/emulation capabilities provided by NDT MnS Producer can be invoked by  management functions, e.g. MDAF, AIML Inference Function, Intent Handling Function, playing the role of NDT MnS Consumer to support their intelligence and automation functionality.

Figure 4.2-1

Next change

## 6.4	Procedure for consuming NDT management service

Figure 6.4-1: Procedure of consuming NDT management service

1. NDT MnS producer  receives a request from MnS consumer to create an instance (see createMOI operation defined in TS 28.532 [8]) which represents service requirements for an NDT job. Examples of NDT jobs can be a signalling storm replay, network configuration verification, etc.

2. NDT MnS producer initiates the NDT job according to the request from MnS consumer.

3. MnS Producer sends a response (see createMOI operation defined in TS 28.532[8]) to the MnS Consumer.

4. MnS producer receives a request from MnS consumer to start the execution of the NDT job.

5. MnS producer executes the NDT job and creates an NDT report instance which contains the execution result of NDT job.

6. MnS producer sends the NDT report to MnS consumer.