3GPP TSG-SA5 Meeting #161	S5-252637

Fukuoka, Japan, 19 - 23 May 2025

Source:	Ericsson France S.A.S

Title:	Rel-19 pCR TS 28.561 Clarify support for data generation

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	TS 28.561

Version:	0.2.0

Work Item:	NDT

Comments

Description in 5.4 is not clear and needs to be improved. This contribution improves the text to clearly shows the benefit of using NDT for data generation. Use case 5.4.2.1 points out the need for large amount of data, but it is not the amount of data but the reliable, useful data from a wider range is what is needed.

In use case 5.4.2.2 describes the capability of NDT to generate user experience data while the focus should be how the data generated by NDT can provide a better opportunity for the consumer to improve the user experience.

Proposed Changes

First change

## 5.4	NDT support for data generation

### 5.4.1	Description

NDTs can be used to support many generation capabilities for different application use cases.

### 5.4.2	Use cases

#### 5.4.2.1	General use case on NDT support for data generation

NDT can be utilized to provide report/output with generated data and information related to network scenarios, configurations, policies, and performance outcomes.

The example scenarios that may have benefit from NDT data generation are described in following sub-clauses.

#### 5.4.2.2	Using NDT to generate ML training data

ML training usually requires reliable data with wide variety and range to guarantee the good performance of ML models. Sufficient ML training data plays a key role for training an ML model. In general, the ML training data for network related use cases is obtained through historical network management data. For instance, for training  an ML model for MDA SLS analysis use case, described in TS 28.104 clause 7.2.2, training data such as UL/DL throughput, uplink/downlink delay, etc., specified in clause 8.4.2 of TS 28.104 can be used.

However, obtaining data from the network has the following limitations:

-	The quantity of issues happened in actual mobile network is limited.

-	The variety of issues happened in actual mobile network is limited. There could be corner network cases that hardly happen in live network but can reduce the performance of the ML model.

To overcome these challenges, the MnS consumer can request the NDT to generate data with an indication of data requirements, e.g. data type, required data period, data sampling periods, etc. When the request is sent by the MnS consumer, the NDT can execute the simulation and as an output generate data corresponding to the request. By using a wider range of training data, the MnS consumer, has a better opportunity to enhance the model accuracy.

#### 5.4.2.3	Using NDT to generate user experience data

For operators, it is important to accurately measure user satisfaction with the network services from network usage perspective. When the performance metrics related to customer satisfaction are low, operators are eager to identify the underlying causes and figure out ways to boost performance. Multiple factors influence user satisfaction, including user service quality, network usage experience and etc. Digital twin technology can be applied here to model and simulate end - user behaviors within the network, providing valuable insights for improving network services.

The satisfaction of network service is affected by combining factors. NDT can be utilized to model and analyse network services by integrating multi-domain data sources which include network performance, user experience data, and fault predictions and report the user experience data. This proactive approach allows CSPs, in this case NDT MnS consumer, to identify potential detractors who are not satisfied with services on the entire network, monitor the end user journey, gain deeper insights into the end user's needs, and perform refined experience management based on user groups.

### 5.4.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-NDTDG-01: | The 3GPP management system should support a capability to provide an output on the simulation/emulation enabling generation of data. | General use case on NDT support for data generation (See clause 5.4.2.1) |
| REQ-NDTDG-02: | The 3GPP management system should have the capability to allow an authorized MnS consumer to request generation of simulated network data to be used for ML training | Using NDT to generate ML training data (See clause 5.4.2.2) Using NDT to generate user experience (See clause 5.4.3) |
| REQ-NDTDG-03: | The 3GPP management system should support a capability to report the generated data for ML training. | Using NDT to generate ML training data (See clause 5.4.2) |
| REQ-NDTDG-05: | The 3GPP management system should support a capability to report user experience data. | Using NDT to generate user experience (See clause 5.4.3) |

Figure 4.2-1

End of changes