3GPP TSG-SA5 Meeting #161	S5-252639

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.533 | CR | 160 | Rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.533 addition of management and orchestration reference architecture in support of SBMA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBMA_Ph3 |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current specification does not explicitly define logical Management Functions (MnFs) as producers or consumers of Management Services (MnSs). This lack of clarity leads to architectural ambiguity, inconsistent interpretation of SA5 specifications across 3GPP Working Groups, and increasing functional overlap, particularly with the introduction of AI/ML and automation in 5G and emerging 6G systems. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This change introduces a complementary management framework to enhance the Service-Based Management Architecture (SBMA). It explicitly defines logical MnFs, clarifies their roles in relation to MnSs, and establishes traceability between MnFs and the MnSs they provide or consume. Normative clauses are added to promote architectural consistency across management domains. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The architectural role of MnFs will remain undefined, leading to continued ambiguity and fragmentation across Working Groups. This may hinder coordination efforts, result in inconsistent implementations, and weaken SA5’s position in driving end-to-end management standardisation for 5G and 6G. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is a draft CR for discussion and potential endorsement. Additional CRs will be created for the relevant specifications to incorporate references to the defined MnFs in those TSs. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

# 5	Architecture reference model

## 5.0	Reference management and orchestration architecture

The reference management and orchestration architecture (see Figure 5.0-1) includes a set of logical Management Functions (MnFs), each of which produces one or more Management Services (MnSs) and/or consumes one or more MnSs or NF services.

MnFs are logical entities and implementation-agnostic, defined independently of any specific deployment architecture.

This reference architecture enables an integrated system view in which MnFs interact with 5G Core (5GC) Network Functions (NFs), User Equipment (UE), and Radio Access Network (RAN) components through both Service-Based Interfaces (SBIs) and legacy management interfaces (e.g., Type 1 interfaces). MnFs are characterised by their management responsibilities and functional scope. Each MnF is defined with a set of MnSs it produces and/or consumes as specified in the corresponding reference specification.

Figure 5.0-x: Management and orchestration reference architecture

Table 5.0-x defines each MnF in the management and orchestration reference architecture along with the corresponding reference specifications.

Table 5.0-x: MnFs definitions and corresponding reference specifications

| MnF | Description | Functionality | Reference specification |
| --- | --- | --- | --- |
| MDAF | Management Data Analytics Function | Management data analytics | TS 28.104 [57] |
| MLTNF | ML Training Function | ML model training | TS 28.105 [66] |
| MLTTF | ML Testing Function | ML model testing | TS 28.105 [66] |
| MLEMF | ML Emulation Function | AI/ML emulation | TS 28.105 [66] |
| IDMF | Intent Driven Management Function | Intent driven management | TS 28.312 [46] |
| UDCF | UE Data Collection Function | Collection of Trace data, MDT measurements, QoE data, and UE level measurements | TS 32.422 [44], 28.405 [49] |
| PMF | Performance Management Function | Collection of performance measurements | TS 28.550 [42] |
| FSF | Fault Supervision Function | Fault supervision | TS 28.530 [3] |
| DMF | Data Management Function | Data management | TS 28.537 [39] |
| PRF | Provisioning Function | Provisioning | TS 28.531 [8] |

End of changes