3GPP TSG-SA5 Meeting #161	S5-252705

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0246 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Use case IAB-node configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MOBILE_IAB_OAM |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This is nomative work for Rel-19 study on management of IAB nodes |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add use case IAB-node configuration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.2, 5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.525: "Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Requirements".

[3]	ETSI GS NFV-IFA 013 (V4.5.1) (2023-09): "Network Function Virtualisation (NFV); Release 4; Management and Orchestration; Os-Ma-nfvo reference point - Interface and Information Model Specification".

[4]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[5]	3GPP TS 22.261 "Service requirements for next generation new services and markets".

[6]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[7]	Void.

[8]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[9]	GSMA NG.116 Generic Network Slice Template v9.0 (2023-04-27) - NG.116-v9.0.pdf (gsma.com).

[10]	3GPP TS 23.501: "Technical Specification Group Services and System Aspects;System Architecture for the 5G System;Stage 2".

[11]	3GPP TS 38.300: "Technical Specification Group Radio Access Network;NR; NR and NG-RAN Overall Description;Stage 2".

[12]	ETSI GS NFV-IFA 014 (V4.2.1) (2021-05): "Network Function Virtualisation (NFV); Release 4; Management and Orchestration; Network Service Templates Specification".

[13]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[14]	ETSI GS NFV-IFA 011 (V4.3.1) (2022-06): "Network Function Virtualisation (NFV); Release 4; Management and Orchestration; VNF Descriptor and Packaging Specification".

[15]	ETSI GS NFV-IFA 008 (V4.3.1) (2022-05): "Network Function Virtualisation (NFV); Release 4; Management and Orchestration; Ve-Vnfm reference point - Interface and Information Model Specification".

[16]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[17]	3GPP TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS)".

[x]	3GPP TS 38.401: "Technical Specification Group Radio Access Network; NG-RAN; Architecture description".

2nd Change

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], TS 28.530 [4] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 28.530 [4].

IAB-node: as defined in TS 38.401 [x].

3rd Change

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 28.530 [4] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GAA	5G Automative Association

5G-ACIA	5G Association for Connected Industries and Automation

5GC	5G Core Network

AMF	Access and Mobility Management Function

CSC	Communication Service Customer

CSP	Communication Service Provider

CP	Control Plane

GST	Generic Slice Template

IOC	Information Object Class

MANO	Management and Orchestration

MnS	Management Service

NEST	Network Slice Type

NF	Network Function

NFV	Network Functions Virtualisation

NRM	Network Resource Model

NSaaS	Network Slice as a Service

NSC	Network Slice Customer

NSI	Network Slice Instance

NSP	Network Slice Provider

NSSI	Network Slice Subnet Instance

P-NEST	Private NEST

S-NEST	Standardized NEST

TN	Transport Network

VNF	Virtualized Network Function

UP	User Plane

IAB	Integrated Access and Backhaul

4th Change

## 5.1.x Use case IAB-node configuration

### 5.1.x.1 Overview

IAB architecture is specified in 3GPP TS 38.401[x]. The IAB-node connects to an upstream IAB-node or an IAB-donor-DU via a subset of the UE functionalities of the NR Uu interface (named IAB-MT function of IAB-node). IAB-node differs from static NE in term of its wireless connection with IAB-donor-node and its mobility behaviours. Mobile IAB can move to different geographic areas or traverse to different PLMNs which as a result may connect to different management system based on its location. Mobile IAB-node should use configuration based on its location at power up and in mobile scenarios as specified in 3GPP TS 38.401 [x].

### 5.1.x.2 Obtain configuration via notifications

Mobile IAB-node may need new configuration while it moves. 3GPP management system should support IAB-node to use notification mechanism to obtain configuration based on its location. When notification mechanism is used, 3GPP management system shall support the mapping of configuration with IAB-node location, see 5.1.x.3.

When IAB-node powers up and after connecting to 3GPP management system for the 1st time, the MnS producer for IAB-node shall provide location information with IAB-node indication via notifications to 3GPP management system as specified in 3GPP TS 28.532 [y] (e.g. notifyMOIChanges) to indicate its current location. Upon the reception of notifications, 3GPP management system shall provision the initial configuration (e.g. via createMOI operation) as specified 3GPP TS 28.532 [y] to the MnS producer for IAB-node based on its current location.

When IAB-node moves to a new location, the MnS producer for IAB-node shall provide location information to 3GPP management system via "notifyMOIChanges" or "notifyMOIAttributeValueChanges" or other notifications as specified in 3GPP TS 28.532 [y]. Upon the reception of notifications of the location, if configuration is needed, 3GPP management system shall provision the configuration using the operations as specified in 3GPP TS 28.532 [y] (e.g., modifyMOIAttributes) to the MnS producer for IAB-node based on its location.

### 5.1.x.3 Location information

The location information should include one or more of the following depending on IAB-node physical location or the mobility scenarios as specified in 3GPP TS 38.401 [x] clause 8.12 and clause 8.23:

-	TAC or TAI pertaining to the cells where IAB-MT is connected.

-	Serving cell ID that IAB-MT is connected to.

-	The gNBId of the IAB-donor-CU and PLMN info that target IAB-DU connects to.

-	The gNBId of the IAB-donor-CU and PLMN info that serves the IAB-MT.

End of change