3GPP TSG-SA5 Meeting #161	S5-252710

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.558 | CR | 0036 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS28.558 Fix UE level measurements unit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Fix UE level measurements unit |  |  |  |  |  |  |  |  |
| Summary of change: |  | - MBits -> Mbits - cleanup |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong unit causes unexpected result |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.5.1.1, 6.3.1.5.1.2, 6.3.1.5.2.1, 6.3.1.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGE ***

##### 6.3.1.3.1	UL PDCP SDU Loss Rate

a)	This measurement provides the fraction of PDCP SDU packets which are not successfully received at gNB-CU-UP. It is a measure of the UL packet loss including any packet losses in the air interface, in the gNB-CU and on the F1-U interface. Only user-plane traffic (DTCH) and only PDCP SDUs that have entered PDCP (and given a PDCP sequence number) are considered. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC architecture [16]), and subcounters per supported S-NSSAI. This measurement is also referred to as UL M7 in TS 37.320 [9].

b)	SI

c)	This measurement is obtained as: 1,000,000* Number of missing UL PDCP sequence numbers, representing packets that are not delivered to higher layers, of a data radio bearer, divided by Total number of UL PDCP sequence numbers (also including missing sequence numbers) of a bearer, starting from the sequence number of the first packet delivered by UE PDCP to gNB-CU-UP until the sequence number of the last packet. If transmission of a packet might continue in another cell, it shall not be included in this count. Separate counters are optionally maintained for mapped 5QI (or QCI for EN-DC architecture [16]) and per supported S-NSSAI.

d)	Each measurement is an integer value representing the loss rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS and S-NSSAI level measurements are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

e)	The measurement name has the form DRB.PacketLossRateUlUe, and optionally
DRB.PacketLossRateUlUe.QoS where QoS identifies the target quality of service class, and DRB.PacketLossRateUlUe.SNSSAI where SNSSAI identifies the S-NSSAI.

f)	GNBCUUPFunction; NRCellCU (applicable to all split scenarios);

NRCellCU (only applicable to 2-split scenario).

g)	S-TMSI

h)	One usage of this measurement is to support ML training and performance evaluation.

*** START OF  2nd CHANGE ***

#### 6.3.1.5	UE Data Volume

##### 6.3.1.5.1	Measurements valid for non-split gNB deployment scenario

###### 6.3.1.5.1.1	DL PDCP SDU Data Volume

a)	This measurement provides the Data Volume (amount of PDCP SDU bits) in the downlink delivered to PDCP layer. The measurement can be filtered per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI. This measurement is also referred to as DL M4 in TS 37.320 [9] clause 5.4.1.1. The unit is Mbit.

b)	CC

c)	This measurement is obtained by counting the number of bits entering the NG-RAN PDCP layers. The measurement is performed at the PDCP SDU level. The measurement is performed per QoS level (mapped 5QI or QCI in NR option 3) and per S-NSSAI.

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1Mbits=1000*1000 bits). The number of measurements is equal to the number of QoS levels multiplied by the number of SNSSAIs.

[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.PdcpSduVolumeDlUe_Filter
, where Filter is a combination of QoS level and SNSSAI, whereQoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

f)	NRCellCU

g)	N/A

h)	One usage of this measurement is to support ML training and performance evaluation.

###### 6.3.1.5.1.2	UL PDCP SDU Data Volume

a)	This measurement provides the Data Volume (amount of PDCP SDU bits) in the uplink delivered from PDCP layer to higher layers. The measurement can be filtered per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI. This measurement is also referred to as UL M4 in TS 37.320 [9] clause 5.4.1.1. The unit is Mbit.

b)	CC

c)	This measurement is obtained by counting the number of bits delivered from PDCP layer to higher layers. The measurement is performed per QoS level (mapped 5QI or QCI in NR option 3) and per S-NSSAI.

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1Mbits=1000*1000 bits). The number of measurements is equal to the number of QoS levels multiplied by the number of S-NSSAIs.

[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.PdcpSduVolumeUlUe_Filter
, where Filter is a combination of QoS level and SNSSAI, where QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

f)	NRCellCU

g)	N/A

h)	One usage of this measurement is to support ML training and performance evaluation.

##### 6.3.1.5.2	Measurements valid for split gNB deployment scenario

###### 6.3.1.5.2.1	DL PDCP SDU Data Volume

a)	This measurement provides the Data Volume (amount of PDCP SDU bits) in the downlink delivered to PDCP layer. The measurement can be filtered per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI. This measurement is also referred to as DL M4 in TS 37.320 [9] clause 5.4.1.1. The unit is Mbit.

b)	CC

c)	This measurement is obtained by counting the number of bits entering the NG-RAN PDCP layers. The measurement is performed at the PDCP SDU level. The measurement is calculated per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI.

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1Mbits=1000*1000 bits). The number of measurements is equal to the number of QoS levels multiplied by the number of S-NSSAIs.

[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form QosFlow.PdcpSduVolumeDlUe_Filter
, where Filter is a combination of QoS level and SNSSAI, where QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

f)	NRCellCU, GNBCUUPFunction

g)	N/A

h)	One usage of this measurement is to support ML training and performance evaluation.

###### 6.3.1.5.2.2	UL PDCP SDU Data Volume

a)	This measurement provides the Data Volume (amount of PDCP SDU bits) in the uplink delivered from PDCP layer to higher layers. The measurement can be filtered per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI. This measurement is also referred to as UL M4 in TS 37.320 [9] clause 5.4.1.1. The unit is Mbit.

b)	CC

c)	This measurement is obtained by counting the number of bits delivered from PDCP layer to higher layers. The measurement is calculated per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI.

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1Mbits=1000*1000 bits). The number of measurements is equal to the number of QoS levels multiplied by the number of S-NSSAIs.

[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form QosFlow.PdcpSduVolumeUlUe_Filter
, where Filter is a combination of QoS level and SNSSAI, where QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

f)	NRCellCU, GNBCUUPFunction

g)	N/A

h)	One usage of this measurement is to support ML training and performance evaluation.

*** END OF CHANGE ***