3GPP TSG-SA5 Meeting #161	S5-252732

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.254 | CR | 0074 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.254 CAPIF NEF API Charging Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CH_CAPIF_EXT |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Missing information on the Tenant that invokes an API through CCF |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include API Invoker reference in NEF API Charging Information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No CDR Available with CAPIF related information |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

##### 6.3.1.4.1	Definition of the NEF API Charging Information

Network Exposure Function API specific charging information is provided within the NEF API Charging Information. The detailed structure of the NEF API Charging Information can be found in table 6.3.1.4.1.

Table 6.3.1.4.1: Structure of the NEF API Charging Information

| Information Element |  | Category |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| External Individual Identifier |  | OC |  | This parameter holds the external Identifier of the individual UE, e.g., the GPSI. |  |
| External Individual Id List |  | OC |  | This parameter holds a list of external identifiers, used instead of External Individual Identifier if there is more than one. |  |
| Internal Individual Identifier |  | OC |  | This parameter holds the internal Identifier of the individual UE e.g., the SUPI. |  |
| Internal Individual Id List |  | OC |  | This parameter holds a list of internal identifiers, used instead of Internal Individual Identifier if there is more than one. |  |
| External Group Identifier |  | OC |  | This parameter holds the external identifier for a group of individual UE(s). |  |
| Internal Group Identifier |  | OC |  | The internal Identifier identifying a group of individual UE(s). |  |
| API Direction |  | OC |  | This field holds the direction to indicate if it is an API invocation from an AF or notification to an AF. |  |
| API Target Network Function |  | OC |  | This field holds the identifier of the network function that either is the destination of the API invocation or triggers the notification: e.g. PCF, UDM, AMF, SMF, UPF. |  |
| API Result Code |  | OC |  | This parameter holds the result of API Invocation. |  |
| API Name |  | M |  | This field holds the name of the API invoked. |  |
| API Operation |  | OC |  | This field holds the operation of the API invoked, together with detailed description. |  |
| API Reference |  | OC |  | This field holds the reference to the definition of the format of the API invocation, this can be a URI or refence to the standard where it's specified |  |
| API Content |  | OC |  | This field holds the actual content of the API invocation, in the format described by the API Reference |  |
| API Invoker |  | OC |  | This field holds the actual content Tenant that invoked a CAPIF API |  |

End of changes