3GPP TSG-SA5 Meeting #162	S5-253622

Goteborg, Sweden, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.299 | CR | 0828 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 32.299 IMS Call Charging Optimization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CSCN |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In offline and online charging, add the solution of charging system to handle the ACR[Stop] lost and CCR[Terminate] lost scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For ACR[Stop] lost scenario, add the description to record the timestamp of last ACR[Interim] in CDR. For CCR[Terminate] lost scenario, add the description to get the session termination timestamp based on the used service units reported in the last CCR . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operators’ revenue loss in ACR[Stop] lost scenario since incomplete CDR can not provide valid call duration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.4, 6.3.6.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 32.260 CR 0447, TS 32.298 CR 1055, TS 32.298 1056 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

6.1.3.4	CDF detected failure

The CDF closes a CDR when it detects that expected Diameter ACRs for a particular session have not been received for a period of time. In case that CDF has not received the expected ACR[Stop], the closed CDR shall record the timestamp of the last Diameter ACR CDF receives. The exact behaviour of the CDF is operator configurable.

Second change

6.3.6.X	OCS detected failure

When OCS detects that the expected Diameter CCRs have not been received, OCS shall determine the session termination timestamp based on the used service units reported in the last CCR that OCS receives.

End of change