3GPP TSG-SA5 Meeting #163	S5- 254339

# Wuhan, China, 13th Oct 2025 - 17th Oct 2025

# Source:	Ericsson Hungary

# Title:	DP Clarify use of choice

# Document for:	Endorsement

Agenda Item:	6.20.0

# 1	Decision/action requested

Endorsement

# 2	References

- 3GPP TS 32.160 Management service template

- 3GPP TS 32.156 Fixed Mobile Convergence (FMC) Model repertoire

# 3		Rationale

The usage of "choice" in NRMs and its definition of "choice" in TS 32.156 do not match.

In this discussion paper we adapt the YANG terminology where a "choice" is a possibility to select between different "cases", where each case may contain one or several items or data types. Having different terms for the "choice" and the alternative "cases" helps understanding.

Observation 1: In 32.156 clause 5.3.6 choice is defined as "represents one of a set of classes or one of a set of data types". In the NRMs (e.g. 28.622, 28.111, 28.541) choices are used mostly to indicate a choice between attributes or a set of attributes. In a smaller number of  cases it is used to choose between data types.

Observation 2: . In case of chosing between attributes, it is assumed, but never specified that only one of the cases can be selected. There is no way to specify whether it is allowed to select none of the cases, or if selecting one case is mandatory.

Observation 3: . In a smaller number of  cases choice is used to chose between data types. However, the format of defining such a choice is not defined and not uniform. Below are 2 examples of how it is documented:

| specificProblem | ... | type: string or integer |
| --- | --- | --- |
| DnnUpfInfoItem.ipv4IndexList | This attribute represents a list of Ipv4 Index supported by the UPF. This <<choice>> represents the IP Index to be sent from UDM to the SMF. (See clause 6.1.6.2.77 TS 29.503 [97]) It is a list of non-exclusive alternatives (Integer or String). | type: <<choice>> |

Observation 4: How to format the specification of a choice is not described in 32.156. Some figures are present which provide little information, as they have no relation to the actual format used in NRMs. While TS 32.160 describes the format for chosing between attribute(field)s, it provides no guidance about specifying choices between datatypes. There were complaints that specifying the format in a separate document is bad, and the current format is complicated.

Observation 5: . The second number of choice (between attributes/attribute sets) does not convey any useful information, and is sometimes mistaken as a subchoice. (E.g. CHOICE_2_3 means the third attribute of the second case, however it was misinterpreted such that you first chose between CHOICE _1,  2 and 3. Once you have chosen  CHOICE_2 you have to chose one of sub-choices . CHOICE_2_1 or CHOICE_2_2 or CHOICE_2_3.

Recommendation 1: Update the TS 32.156 definition of choice.

Recommendation 2: Update the TS 32.160 clause 5.2 W4.3.a  Attributes to indicate how to define a choice of different data types.

Recommendation 3: Update the TS 32.160 to make the second number in a CHOICE_<X>.<Y> ("Y") optional, preferably not used.

# 4	Detailed proposal

Endorse the modification of TS 32.156 and 32.160 according to recommendation 1, 2 and 3 for release-20.