3GPP TSG-SA5 Meeting #163	S5-254410

Wuhan, China, 13 - 17 October 2025

Source:	Rakuten Mobile, Inc.

Title:	Pseudo-CR on TR 28.869 NRM updates for NF Deployment LCM

Document for:	Approval

Agenda item:	6.19.6

Spec:	3GPP TR 28.869

Version:	1.6.0

Work Item:	FS_Cloud_OAM

Comments

This contribution proposes some NRM attributes for LCM of NF Deployment.

Proposed Changes

* * * First Change * * * *

#### 5.2.1.3	Potential solutions

Introduce a generic orchestration and management entity that interacts with 3GPP management system for LCM of NF Deployment instance via the new deployment management reference point.

The figure 5.2.1.3-1 below illustrates the interaction between 3GPP management system and orchestration and management entity in simplified view:

Figure 5.2.1.3-1: Lifecycle management of NF Deployment instance

The deployment management reference point includes the interfaces that 3GPP supports today with ETSI NFV MANO but not limited to it, e.g. Kubernetes based API. For the case of NFV-MANO the deployment management reference point includes the exchange of information over the interfaces that 3GPP supports with ETSI NFV-MANO (ETSI GS NFV-IFA 013 [8] and ETSI GS NFV-IFA 008 [9]).

Within the 3GPP Management system, a Provisioning MnS consumer interacts with a Provisioning MnS producer to pass NRM attributes. Within the scope of the current study, a new element named as NFDeploymentList can be added to ManagedFunction IOC (currently defined in clause 4.3.4.2 of TS 28.662) to manage LCM of NF Deployment(s). The updated ManagedFunction IOC can be seen below:

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| vnfParametersList | CM | T | T | F | T |
| peeParametersList | CM | T | T | F | T |
| priorityLabel | O | T | T | F | T |
| supportedPerfMetricGroups | O | T | F | F | T |
| supportedTraceMetrics | O | T | F | F | T |
| nfDeploymentList | CM | T | T | F | T |

The nfDeploymentList attribute can be further described in clause 4.4.1 of TS 28.662 as below:

| nfDeploymentParametersList | This attribute contains the parameter set of the NF Deployment instance(s) corresponding to an cloud-native NE. Each entry in the list contains: - nfDeploymentInstanceId: NF Deployment instance identifier corresponds to the software instance running in the cloud. This may be managed by industry SDOs such as ETSI NFV or other industry solutions. - nfDeploymentDescId (optional): Identifier of the descriptor which is a deployment template that describes a NF Deployment in terms of its deployment and operational requirements. - nfDeploymentFlavourId (optional): Identifier corresponding to the size of the deployment. This refers to the supported flavours in the NF Deployment descriptor. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| --- | --- | --- |

* * * End of Changes * * * *