3GPP TSG-SA5 Meeting #163	S5-254416

Wuhan, China, 13. – 17. October 2025

Source:	NTT DOCOMO

Title:	Pseudo-CR on Rel-20 TR 28.889 Add use case description and requirement for Network Maintenance CCL

Document for:	Approval

Agenda item:	6.20.10

Spec:	3GPP TR 28.889

Version:	0.0.0

Work Item:	FS_CCLM_Ph2

Comments

This pCR provides use-case description, requirements and solutions for WT-3 Study the need for CCL enhancements enabling automated, efficient network management.

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[X]	3GPP TS 28.567: "Management and orchestration; Management aspects of closed control loops"

[Y]	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks"

[Z]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA) "

[K]	3GPP TS 28.535: "Management and orchestration; Management services for communication service assurance; Requirements".

[Q]	3GPP TS 28.536: "Management and orchestration; Management services for communication service assurance; Stage 2 and stage 3".

* * * Next Change * * * *

## 4.X Use case#X: Closed Control Loop for Network Maintenance

### 4.X.1	Description

Network maintenance operations (e.g., software upgrades, downgrades, and patching of network functions) can significantly affect service continuity, performance, and user experience. Because of this, they need to be planned, validated, executed, and monitored with minimal disruption. CCLs can help in assisting and automating the maintenance operations, in particular when maintenance needs to be performed in complex scenarios with multiple network functions with dependencies among them.

This use case describes a scenario in which an MnS consumer requests a CCL for network maintenance, for example, a software upgrade, to achieve automation for network maintenance. The request may include the type of maintenance required  ( e.g., software upgrade, software downgrade), target version for maintenance, any constraints for maintenance (e.g., time-window when the maintenance should take place), as well as any policies required for network maintenance the order of NFs for maintenance, whether isolation of NFs is required for network maintenance work, etc.

The intent-driven MnS producer may be MnS consumer of a Network Maintenance CCL MnS producer if the intent-driven MnS producer handles a Network Maintenance Expectation as specified in 3GPP TS 28.312 [Y], clause 6.2.2.1.6.

A CCL for network maintenance may also use MDA reports in 3GPP TS 28.104 [Z], for example, for the maintenance use-cases described in clause 7.2.6, to decide and execute necessary actions for network maintenance. A CCL for network maintenance may also analyse the impact of requested maintenance work, for example, the impact of upgrading an NF software version. CCL for network maintenance may also coordinate with ConflictManagementAndCoordination Entity as specified in TS 28.567 [Y], clause 6.3.6 whether requested network maintenance cause any conflicts to existing operations. A CCL for network maintenance may work with CCLs for assurance as specified in 3GPP TS 28.535 [K] and in 3GPP TS 28.536 [Q] in order to validate the network maintenance through KPIs associated with it.

The CCL for network maintenance MnS producer reports the result of network maintenance, including the resultant maintenance and any other relevant information.

### 4.X.2	Potential requirements

REQ-MaintenanceCCL -1: The 3GPP management system should have the capability to allow MnS consumer to request a closed control loop for network maintenance

REQ-MaintenanceCCL-2: The 3GPP management system should have the capability to allow MnS consumer to get a report from the closed control loop regarding the network maintenance.

### 4.X.3	Potential solutions

#### TBD

* * * End of Changes * * * *