# 3GPP TSG SA WG5 Meeting #163	 															S5-254417

# Wuhan, CHINA 13 - 17 October 2025

Source:

Title:	Pseudo-CR on TR 28.881 Add new use case for supporting collaborative deployment of intent handling function and CCL

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR 28.881

Version:	V0.1.0

Work Item:	FS_IDMS_MN_Ph4

Comments

This contribution proposes to add use case for supporting collaborative deployment of intent handling function and CCL.

This pCR related to WT-2 and WT-5.1.

Proposed Changes

* * * First Change * * * *

4.X Issue#X: Supporting collaborative deployment of intent handling function and CCL

4.X.1 Description

In an intent-driven network management architecture, the collaboration between the intent handling function and the closed control loop (CCL) can significantly enhance the flexibility and automation capabilities of the network. The intent handling function converts the consumer's requirements into specific network operations (such as the operations of Virtual Network Functions (VNF) or Physical Network Functions (PNF)), while the closed control loop continuously monitors and dynamically adjusts, ensuring that the network service requirements are consistently achieved and met throughout the entire lifecycle.

In traditional network deployment methods, the intent handling function, VNF or PNF, and closed control loop (CCL) are typically deployed independently. Although each can meet specific requirements, this standalone deployment often leads to a lack of tight integration between these functions. They fail to quickly respond to real-time changes, often resulting in inefficiencies and resource wastage.

The introduction of collaborative deployment for the intent handling function, VNF or PNF, and closed control loop can effectively address the coordination and response latency issues inherent in standalone deployment. In the deployment process, the intent handling capability described by the intent description file, VNF packages or PNF packages, and the closed control loop capability described by the closed loop process file can be organized and deployed together. Through the tight integration of these components, network services can immediately activate the intent handling function and closed control loop after deployment, ensuring that network devices (VNF or PNF) are configured according to the intent requirements and can be optimized in real time. Meanwhile, the closed control loop ensures that these requirements are continuously met during operation and responds promptly to environmental changes.

4.X.2 Potential requirements

REQ-IDMS-IntentHandling-DEP-1: The intent driven MnS producer should enable the MnS consumer to request the deployment of the intent handling function alongside the corresponding network equipment (e.g., VNF or PNF).

REQ-IDMS-IntentHandling-DEP-2: The intent driven MnS producer should enable the MnS consumer to request the deployment of the intent handling function alongside other automation functions, such as closed control loop (CCL).

REQ-IDMS-IntentHandling-DEP-3: The intent driven MnS producer should enable the MnS consumer to request the deployment of necessary supporting files, such as the intent description file, VNF/PNF packages, and closed loop process file, to ensure proper integration.

REQ-IDMS-IntentHandling-DEP-4: The intent driven MnS producer should enable the MnS consumer to request the deployment of the intent handling function, network equipment (e.g., VNF or PNF), and closed control loop function (CCL) together.

REQ-IDMS-IntentHandling-DEP-5: The intent driven MnS producer should have the capability to allow an intent-driven MnS consumer to receive a report regarding the status and progress of the deployment of intent handling functions and related network resources.

REQ-IDMS-IntentHandling-DEP-6: The intent driven MnS producer should have the capability to allow an intent-driven MnS consumer to receive a report regarding the status and progress of the deployment of intent handling functions and related functions, such as closed-loop control.

4.X.3 Potential solution

TBD

4.X.4 Evaluation of potential solutions

TBD

* * * End of Changes * * * *