3GPP TSG-SA5 Meeting #163	S5-254463

Wuhan, China, 13. - 17. October 2025

# Source:	Ericsson

# Title:	Charging architecture considerations for CAPIF reference points

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

The group is asked to discuss and approve the proposal.

# 2	References

[1]	3GPP TR 28.891: " Study on Charging Aspects of CAPIF phase 3".

# 3	Rationale

Addition of a solution in CAPIF charging scenarios and key issues for Usage of the CAPIF reference points with all reference points listed.

# 4	Detailed proposal

* * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs"

[y]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs	"

[z]	3GPP TS 32.290: " 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)"

* * * Next Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

API	Application Program Interface

CAPIF	Common API Framework

CCF	CAPIF Core Function

CTF	Charging Trigger Function

ECUR	Event Charging with Unit Reservation

IEC	Immediate Event Charging

PEC	Post Event Charging

* * * Next Change * * * *

# x	CAPIF Reference points

## x.1	General

## x.2	CAPIF-1 and CAPIF-1e

### x.2.1	General

The CAPIF-1 and CAPIF-1e reference points (TS 23.222 [x] clauses 6.4.2 and 6.4.3), which exists between the API invoker and the CAPIF core function, is used for the API invoker within and outside the PLMN trust domain and to discover service APIs, to authenticate and to get authorization.

The CAPIF-1 and CAPIF-1e reference points supports:

-	Onboarding the new API invokers and offboarding API invokers.

-	Authenticating the API invoker based on the identity and credentials of the API invoker.

-	Mutual authentication between the API invoker and the CAPIF core function.

-	Providing authorization for the API invoker prior to accessing the service AP.

-	Providing authorization for the API invoker based on RNAA.

-	Discovering the service APIs information.

Where the main chargeable events are:

-	Onboarding the new API invokers and offboarding API invokers.

-	Discovering the service APIs information

In this case there are only two entities involved and that is the API Invoker and the CCF, and since the API Invoker may be outside the operator’s trusted domain the only place to trigger charging is the CCF.

### x.2.2	CAPIF core function acting as CTF

In this case the CTF would be in the CAPIF core function and trigger based on the CAPIF-1 and CAPIF-1e invocations. The below only shows onboarding with Post Event Charging (PEC) on CAPIF-1, other services on CAPIF-1 or CAPIF-1e, e.g. discovery, would be handled similarly, also Immediate Event Charging (IEC) and Event Charging with Unit Reservation. (ECUR) could be supported.

Figure x.2.2-1: API invoker onboarding PEC with CCF acting as CTF

Based on the flow in TS 23.222 [x] clause 8.1.

1.	CAPIF-1: The API Invoker triggers onboard API invoker request towards the CAPIF Core Function, for more details see TS 29.222 [y].

2.	CAPIF-1: After the CAPIF Core Function have performed the onboarding process it responds with an onboard API Invoker response, for more details see TS 29.222 [y].

3.	Charging Data Request [Event]: The CAPIF core function sends Charging Data Request [Event] to the CHF for reporting the charging information API invoker id and outcome of the onboarding, for more details see TS 32.290 [z].

4.	Create CDR: the CHF stores received information and creates a CDR related to the service.

5.	Charging Data Response [Event]: The CHF acknowledges by sending Charging Data Response [Event] to the CAPIF Core Function, for more details see TS 32.290 [z].

## x.3	CAPIF-2 and CAPIF-2e

### x.3.1	General

The CAPIF-2 and CAPIF-2e reference points (TS 23.222 [x] clauses 6.4.4 and 6.4.5), which exists between the API invoker and the API exposing function belonging to the same or different trust domains, is used for the API invoker to communicate with the service APIs.

The CAPIF-2 and CAPIF-2e reference points supports:

-	Authenticating the API invoker based on the identity and credentials of the API invoker.

-	Authorization verification for the API invoker upon accessing the service API.

-	Invocation of service APIs.

Where the main chargeable events are:

-	Invocation of service APIs.

In this case there are only two entities involved and that is the API Invoker and the API Provider, and since the API Invoker may be outside the operator’s trusted domain the only place to trigger charging is the API Provider.

### x.3.2	API Exposing Function in the API provider domain acting as CTF

In this case the API Exposing Function in the API provider domain would act as a CTF and trigger based on the CAPIF-2 and CAPIF-2e invocations for API provider. The below only shows invocation of service APIs on CAPIF-2 with PEC, other services on CAPIF-2 or CAPIF-2e would be handled similarly, also IEC and ECUR could be supported.

Figure x.3.2-1: Invocation of service APIs PEC with CCF acting as CTF

Based on the flow in TS 23.222 [x] clause 8.20.

1.	CAPIF-2: The API Invoker invokes an API towards the API Exposing Function in the API provider domain, for more details see TS 29.222 [y].

2.	CAPIF-2: After the execution of the service provided by the API invoked the API Exposing Function in the API provider domain responds with the result, for more details see TS 29.222 [y].

3.	Charging Data Request [Event]: The API Exposing Function in the API provider domain sends Charging Data Request [Event] to the CHF for reporting the charging information including API invoker id and result (e.g. failure or success) of the API invocation, for more details see TS 32.290 [z].

4.	Create CDR: the CHF stores received information and creates a CDR related to the service, for more details see TS 32.290 [z].

5.	Charging Data Request [Event]: The CHF acknowledges by sending Charging Data Response [Event] to the API Exposing Function in the API provider domain, for more details see TS 32.290 [z].

## x.4	CAPIF-3 and CAPIF-3e

### x.4.1	General

The CAPIF-3 and CAPIF-3e reference points (TS 23.222 [x] clauses 6.4.6 and 6.4.9), which exists between the API exposing function within the PLMN trust domain or 3rd party trust domain and the CAPIF core function within the PLMN trust domain, is used for exercising access and policy related control for service API communications initiated by the API invoker.

The CAPIF-3 and CAPIF-3e reference points supports:

-	Authenticating the API invoker based on the identity and credentials of the API invoker.

-	Providing authorization for the API invoker prior to accessing the service API.

-	Authorization verification for the API invoker upon accessing the service API.

-	Authorization verification for the API invoker based on RNAA.

-	Controlling the service API access based on PLMN operator configured policies.

-	Logging the service API invocations.

There are no real chargeable events on this reference point, for the charging of API invocations see clause x.3.

## x.5	CAPIF-4 and CAPIF-4e

### x.5.1	General

The CAPIF-4 and CAPIF-4e reference points (TS 23.222 [1] clauses 6.4.7 and 6.4.10), which exists between the API publishing function within the PLMN trust domain or 3rd party trust domain and the CAPIF core function within the PLMN trust domain, is used for publishing the service API information.

The CAPIF-4 and CAPIF-4e reference points supports:

-	Publishing the service APIs information by the API publishing function.

Where the main chargeable event is:

-	Publishing the service APIs information.

In this case there are only two entities involved and that is the API Provider and the CCF, and since the API Provider may be outside the operator’s trusted domain the only place to trigger charging is the CCF.

### x.5.2	CAPIF core function acting as CTF

In this case the CTF would be in the CCF, and trigger based on the CAPIF-4 or CAPIF-4e invocations. The below only shows Service API publish on CAPIF-4 with PEC, other services on CAPIF-4 or CAPIF-4e would be handled similarly, also IEC and ECUR could be supported.

Figure x.5.2-1: Publishing the service API PEC with CCF acting as CTF

Based on the flow in TS 23.222 [x] clause 8.3.

1.	CAPIF-4: The API Publishing Function in the API provider domain sends a service API publish request to the CAPIF Core Function, for more details see TS 29.222 [y].

2.	CAPIF-4: After the CAPIF core function have performed the publishing process the CAPIF Core Function provides a service API publish response to the API Publishing Function in the API provider domain, for more details see TS 29.222 [y].

3.	Charging Data Request [Event]: The CAPIF Core Function sends Charging Data Request [Event] to the CHF for reporting the charging information including API provider id and outcome of the publishing, for more details see TS 32.290 [z].

4.	Create CDR: the CHF stores received information and creates a CDR related to the service, for more details see TS 32.290 [z].

5.	Charging Data Request [Event]: The CHF acknowledges by sending Charging Data Response [Event] to the CAPIF Core Function, for more details see TS 32.290 [z].

## x.6	CAPIF-5 and CAPIF-5e

### x.6.1	General

The CAPIF-5 and CAPIF-5e reference points (TS 23.222 [x] clauses 6.4.8 and 6.4.11), which exists between the API management function within the PLMN trust domain or 3rd party trust domain and the CAPIF core function within the PLMN trust domain, is used for management of service API, API invoker and API provider domain function information.

The CAPIF-5 and CAPIF-5e reference points supports:

-	Accessing the service API invocation logs by the API management function.

-	Enabling the API management function to monitor the events reported due to the service APIs invocations.

-	Onboarding new API invokers by provisioning the API invoker information at the CAPIF core function, requesting explicit grant of new API invokers onboarding and confirming onboarding success.

-	Offboarding API invokers.

-	Enabling the API management function to configure policies at the CAPIF core function e.g. service API invocation throttling, blocking API invocation for certain duration.

-	Enabling the API provider domain to monitor the status of service APIs (e.g. pilot or live status, start or stop status of service API).

-	Registering API provider domain functions on the CAPIF core function.

-	Update of the registration information of API provider domain functions on the CAPIF core function.

NOTE 1:	The API invoker onboarding/offboarding over CAPIF-5 is out of the scope of the current release.

Where the main chargeable events are:

-	Accessing the service API invocation logs.

-	Onboarding new API invokers by provisioning the API invoker information at the CAPIF core function, requesting explicit grant of new API invokers onboarding and confirming onboarding success.

-	Offboarding API invokers.

-	Monitor the events reported due to the service APIs invocations.

-	Configure policies at the CAPIF core function e.g. service API invocation throttling, blocking API invocation for certain duration.

-	Monitor the status of service APIs (e.g. pilot or live status, start or stop status of service API).

-	Registering API provider domain functions.

-	Update of the registration information of API provider domain functions on the CAPIF core function.

### x.6.2	CAPIF core function acting as CTF

In this case the CTF would be in the CCF, and trigger based on the CAPIF-5 or CAPIF-5e invocations. The below only shows onboarding CAPIF-5 with PEC, other services on CAPIF-5 or CAPIF-5e would be handled similarly, also IEC and ECUR could be supported.

Figure x.5.2-1: Onboarding API invoker PEC with CCF acting as CTF

Based on the flow in TS 23.222 [x] clause 8.1.

1.	CAPIF-5: The API Invoker triggers onboard API invoker request towards the CAPIF Core Function, for more details see TS 29.222 [y].

2.	CAPIF-5: After the CAPIF Core Function have performed the onboarding process it responds with an onboard API Invoker response, for more details see TS 29.222 [y].

3.	c The CAPIF core function sends Charging Data Request [Event] to the CHF for reporting the charging information API invoker id and outcome of the onboarding, for more details see TS 32.290 [z].

4.	Create CDR: the CHF stores received information and creates a CDR related to the service, for more details see TS 32.290 [z].

5.	Charging Data Request [Event]: The CHF acknowledges by sending Charging Data Response [Event] to the CAPIF Core Function, for more details see TS 32.290 [z].

## x.7	CAPIF-6 and CAPIF-6e

### x.7.1	General

The CAPIF-6 and CAPIF-6e reference points (TS 23.222 [x] clauses 6.4.14 and 6.4.15), which exists between the CAPIF core functions instances within the same or different trust domains.

The CAPIF-6 and CAPIF-6e reference point supports:

-	Publishing the service APIs information.

-	Discovering the service APIs information.

-	Obtaining security information for the API invoker upon accessing the service API.

-	Providing authorization for the API invoker prior to accessing the service API.

Where the main chargeable events are:

-	Publishing the service APIs information.

-	Discovering the service APIs information.

### x.7.2	CAPIF core function acting as CTF

In this case the CTF would be in the CCF, and trigger based on the CAPIF-6 or CAPIF-6e invocations. The below only shows Interconnection API publish on CAPIF-6 with PEC, other services on CAPIF-6 or CAPIF-6e would be handled similarly, also IEC and ECUR could be supported.

Figure x.7.2-1: Publishing the service API PEC with CCF acting as CTF

Based on the flow in TS 23.222 [x] clause 8.25.

1.	CAPIF-6: Based on the shareable information for the service API or the service API category information, the CCF A (CAPIF Core Function A) determines to publish the service API or the service API category information to the CCF B (CAPIF Core Function B), for more details see TS 29.222 [y].

2.	CAPIF-6: After the CCF B have performed the publishing process the CCF B provides an interconnection API publish response to the CCF A, for more details see TS 29.222 [y].

3.	Charging Data Request [Event]: The CCF B sends Charging Data Request [Event] to the CHF for reporting the charging information including API provider id and outcome of the publishing, for more details see TS 32.290 [z].

4.	Create CDR: the CHF stores received information and creates a CDR related to the service, for more details see TS 32.290 [z].

5.	Charging Data Request [Event]: The CHF acknowledges by sending Charging Data Response [Event] to the CCF B, for more details see TS 32.290 [z].

## x.8	CAPIF-7 and CAPIF-7e

### 3.8.1	General

The CAPIF-7 and CAPIF-7e reference points (TS 23.222 [x] clauses 6.4.12 and 6.4.13), which exists between the API exposing functions belonging to the same or different trust domains, is used for the forwarding or routing of the API invoker's service API invocation from one API exposing function to the other API exposing function deployed in the PLMN trust domain.

The CAPIF-7 and CAPIF-7e reference point supports all the functions of CAPIF-2.

The CAPIF-7 reference point supports invocation of service APIs originated by the API invoker using CAPIF-2.

NOTE:	The aspects related to the specific service API invocation in reference point CAPIF-7 are out of scope of the present document.

For chargeable events and CTF see CAPIF-2 clause x.3

## x.9	CAPIF-8

### x.9.1	General

The CAPIF-8 reference point (TS 23.222 [1] clauses 6.4.16), which exists between the CAPIF core function and the resource owner function.

The CAPIF-8 reference point supports:

-	Providing authorization for resource access.

-	Managing and revoking the provided authorization.

NOTE:	The functionalities over CAPIF-8 is FFS and out of scope of the current release of the specification.

* * * Next Change * * * *

Annex A (informative):
PlantUML

# A.1	Flows

## A.1.1	Figure x.2.2-1

@startuml

<style>

element {

BackGroundColor: white;

}

</style>

hide footbox

participant APP as "API\nInvoker"

participant CCF as "CCF\n(CTF)"

participant CHF as "CHF"

APP -> CCF : 1. CAPIF-1 [Onboarding of API Invoker]

APP <- CCF : 2. CAPIF-1 [Onboarding response]

group Post-delivery procedure

CCF -> CHF : 3. Charging Data request [Event]

rnote over CHF: 4. Create CDR

CCF <- CHF : 5. Charging Data response [Event]

end

@enduml

## A.1.2	Figure x.3.2-1

@startuml

<style>

element {

BackGroundColor: white;

}

</style>

hide footbox

participant APP as "API\nInvoker"

participant NEF as "API\nProvider\n(CTF)"

participant CHF as "CHF"

APP -> NEF : 1. CAPIF-2 [Invocation of service API]

APP <- NEF : 2. CAPIF-2 [Invocation response]

group Post-delivery procedure

NEF -> CHF : 3. Charging Data request [Event]

rnote over CHF: 4. Create CDR

NEF <- CHF : 5. Charging Data response [Event]

end

@enduml

## A.1.3	Figure x.5.2-1

@startuml

<style>

element {

BackGroundColor: white;

}

</style>

hide footbox

participant NEF as "API\nProvider"

participant CCF as "CCF\n(CTF)"

participant CHF as "CHF"

NEF -> CCF : 1. CAPIF-4 [Service API publish request]

NEF <- CCF : 2. CAPIF-4 [Service API publish response]

group Post-delivery procedure

CCF -> CHF : 3. Charging Data request [Event]

rnote over CHF: 4. Create CDR

CCF <- CHF : 5. Charging Data response [Event]

end

@enduml

## A.1.4	Figure x.6.2-1

@startuml

<style>

element {

BackGroundColor: white;

}

</style>

hide footbox

participant APP as "API\nInvoker"

participant CCF as "CCF\n(CTF)"

participant CHF as "CHF"

APP -> CCF : 1. CAPIF-5 [Onboarding of API Invoker]

APP <- CCF : 2. CAPIF-5 [Onboarding response]

group Post-delivery procedure

CCF -> CHF : 3. Charging Data request [Event]

rnote over CHF: 4. Create CDR

CCF <- CHF : 5. Charging Data response [Event]

end

@enduml

## A.1.3	Figure x.7.2-1

@startuml

<style>

element {

BackGroundColor: white;

}

</style>

hide footbox

participant CCF_A as "CCF A"

participant CCF_B as "CCF B\n(CTF)"

CCF_A -> CCF_B : 1. CAPIF-6 [Interconnection API publish request]

CCF_A <- CCF_B : 2. CAPIF-6 [Interconnection API publish response]

group Post-delivery procedure

CCF_B -> CHF : 3. Charging Data request [Event]

rnote over CHF: 4. Create CDR

CCF_B <- CHF : 5. Charging Data response [Event]

end

@enduml

* * * End of Changes * * * *