3GPP TSG-SA5 Meeting #163	S5-254470

Wuhan, China, 13. - 17. October 2025

Source:	Huawei

Title:	Pseudo-CR on TR 28.883 Add enhanced evaluation use cases

Document for:	Approval

Agenda item:	6.20.3

Spec:	3GPP TR 28.883

Version:	0.0.0

Work Item:	FS_NDT_Ph2

Comments

It is proposed to add use cases for using NDT to perform enhanced evaluation, related to work task:

WT-4: Investigate new use cases and requirements that require enhancement of NDT in 3GPP management system.

TS 28.561 contains use cases to support evaluation of network scenarios (e.g. evaluation of high-risk network operations, evaluation of failure events including signalling storm). In the scope of Release-19, the NDT MnS producer reports the simulation/emulation output and the NDT MnS consumer is responsible for the actual evaluation step. This limits the value of NDT because the NDT MnS consumer must have the intelligence to transform the NDT report (e.g. configuration data and performance data) to the evaluation criteria (e.g. risk level).

It is proposed to enhance NDT by adding the intelligence in the NDT MnS producer to perform the evaluation step. This will add value to NDT because the NDT MnS consumer will only need to check the evaluation result, and thus will be simpler to design.

Proposed Changes

* * * First Change * * * *

## 5.X	Use case #X: Support for evaluation of high-risk network operations

### 5.X.1	Description

It is proposed to extend the existing use case “Support for evaluation of high-risk network operations” in TS 28.561 [x] as follows:

Existing use case description

Each operation for network optimization and maintenance on a mobile network may cause potential network failures, including network congestion leading to lack of ability to fulfil the expected requirements and network breakdown. High-risk operations are operations which have the potential of causing major service impact, for instance, invalid configuration or policy modification. Even other network activities such as software version upgrade, board switching, or optimization procedures on a live network may cause major network degradation and may be considered as high-risk operations. NDT can be utilized to evaluate both known high-risk network operations and those optimization and verification of network policies suspected to pose high-risk consequences to the network.

The results of NDT can give improved system awareness and insights to the consumer and can aid in finding the optimal policy or solution and in avoiding or minimizing risks.

Proposed extra text

The NDT report may include evaluation of the risk, indicating the severity of the risk and the probability of the risk.

### 5.X.2	Potential requirements

### 5.X.3	Potential solutions

### 5.X.4	Evaluation of solutions

## 5.Y	Use case #Y: Support for evaluation of failure events including signalling storm

### 5.Y.1	Description

It is proposed to extend the existing use case “Support for evaluation of failure events including signalling storm” in TS 28.561 [x] as follows:

Existing use case description

The NDT can be used to support the evaluation of failure event refers to the situation where users in the network are completely unable to get service from the network. A typical example of a failure event is signalling storm, where a large number of signalling messages suddenly surge in the mobile communication network, overloading the network's processing capacity and consequently degrading the network performance and stability. During this period, users will repeatedly try to establish the connection until reconnected, thus generating a large number of signalling messages surge suddenly, causing signalling storm. When a signalling storm occurs on the network, network Functions (NFs) such as AMF, SMF, and UDM in the 5G network might all be potential impact points. Therefore, when received the request with network object information from MnS consumer for signalling storm analysis, the NDT can execute the modelling and validation based on network object related information (network performance, KPIs, S-NSSAIs, etc.), which synchronized from the network resources, and provide the report with the modelling results to MnS consumer.

An automation function may propose a solution to a signalling storm. The NDT can be used to evaluate the appropriateness of the proposed solution (e.g.,update the configuration of network flow control parameters) based on the analysing of failure events for resolving, predicting and preventing the signalling storm issue.

NDT models network behaviors and provides information on potential impacts of network failures, including surges in signalling requests. For instance, in case of the signalling storm, the information can enable identification of optimal flow control parameters for each signalling impact point.

Proposed extra text

The NDT report may include evaluation of the failure event, indicating the severity of the failure and the probability of the failure.

### 5.Y.2	Potential requirements

### 5.Y.3	Potential solutions

### 5.Y.4	Evaluation of solutions

* * * End of Changes * * * *