3GPP TSG-SA5 Meeting #163	S5-254838

Wuhan, China, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.290 | CR | 0252 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.290 Corrections to the misalignment and editorial errors of charging scenarios |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SBI_CH, TEI19 |  |  |  |  |  | Date: |  |  | 2025-10-16 |
| Category: | F or D |  |  |  |  |  | Release: |  |  | Rel-19 or Rel-20? |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR aims to introduce several corrections, including: - Editorial error correction; - Alignment of the figures and step descriptions in the offline charging scenario and the converged charging scenario; e.g., change the request to charging data request; the request in Step 1 is for content or services, not for session establishment; align the figure and step descriptions in charging scenarios; |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Editorial error correction - Alignment of the figures and step descriptions in the offline charging scenario and the converged charging scenario |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some editorial errors and misalignments exist in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2.1, 5.1.2.2.1, 5.1.2.2.2, 5.3.2.2, 5.3.2.3, 5.3.2.4, 5.3.2.5, 5.4.8.2, 5.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-254484 |  |  |  |  |  |  |  |  |

First Change

#### 5.1.2.1	Introduction

Two basic scenarios are used:

-	Event based charging;

-	Session based charging.

Both scenarios may generate CDR files, which may then be transferred to the network operator's BD for the purpose of subscriber billing and/or inter-operator accounting.

Middle Change

##### 5.1.2.2.1	Event based charging

Figure 5.1.2.2.1.1 shows a scenario for Post Event Charging (PEC) where the NF (CTF) interacts with the CHF after the service delivery.

Figure 5.1.2.2.1.1: Post Event Charging

1)	Request for resource usage: A request for content/service is received by the NF (CTF).

2)	Content/Service Delivery: the NF (CTF) delivers the content/service.

3)	Charging Data Request [Event]: The NF (CTF) generates charging data related to the delivered content/service and sends the request for the CHF to store related charging data for CDR generation purpose.

4)	 Create CDR: the CHF stores received information and creates a CDR related to the content/service.

5)	Charging Data Response [Event]: The CHF informs the NF (CTF) on the result of the request.

Middle Change

##### 5.1.2.2.2	Session based charging

Figure 5.1.2.2.2.1 shows a scenario for Offline session based charging where the NF (CTF) interacts with the CHF.

Figure 5.1.2.2.2.1:  Offline charging

1)	Request for service delivery and start of service delivery: A request for service is received in the NF (CTF).

2)	Charging Data Request [Initial]: The NF (CTF) sends the request to inform the CHF about the service to be started.

3)	 Open CDR: the CHF opens a CDR related to the service.

4)	Charging Data Response [Initial]: The CHF informs the NF (CTF) on the result of the request and optionally provides the usage reporting triggers applicable to the service.

5)	Service Delivery: the NF (CTF) delivers the service.

6)	Usage Reporting Trigger: the NF (CTF) generates charging data related to service delivered, based on a trigger for usage reporting is met.

7)	Charging Data Request [Update]: the NF (CTF) sends the request for reporting the related charging data to the CHF.

8)	Update CDR: the CHF updates the CDR with charging data related to the service.

9)	Charging Data Response [Update]: The CHF informs the NF (CTF) on the result of the request.

10)	Service Delivery: the NF (CTF) delivers the service.

11)	 Service release: the service is released.

12)	Charging Data Request [Termination]: the NF (CTF) sends the request to the CHF, for reporting charging data related to the service termination.

13)	 Close CDR: the CHF closes the CDR with charging data related to the service termination.

14)	Charging Data Response [Termination]: The CHF informs the NF (CTF) on the result of the request.

Middle Change

#### 5.3.2.2	Event based charging

For Converged Event based Charging, he following cases are supported:

-	Immediate Event Charging (IEC);

-	Post Event Charging (PEC).

The scenario for Event based charging supported by IEC is shown in figure 5.3.2.2.1 with: Decentralized and Centralized Unit Determination, Centralized Rating configuration and user's account balance deduction before service delivery, where the NF (CTF)	may invoke converged charging service towards the CHF, prior to service delivery if needed.

Figure 5.3.2.2.1: IEC- Event based charging with  Decentralized and Centralized Unit Determination, Centralized Rating

1)	Request for resource usage: A request for content/service is received in the NF (CTF). The service is configured to be authorized by the CHF to start.

2)	Units Determination: the NF (CTF) determines the number of units depending on the service requested by the UE in "Decentralized Units determination" scenario.

3)	Charging Data Request [Event, Units]: The NF (CTF) sends the request to the CHF for the service to be granted authorization, and to allow the number of units, if determined in item 2, to be rated and accounted.

4)	Account, Rating Control: The CHF calculates the number of monetary units that represents the price and makes deduction of the calculated amount from user's account balance based on the number of units requested or on internal unit determination, if the user's credit balance is sufficient.

5)	 Create CDR: based on policies, the CHF creates a CDR related to the service.

6)	Charging Data Response [Event, Units]: The CHF grants authorization to NF (CTF) for the service to start, with a number of granted units.

7)	 Granted Units Supervision: The service starts and the NF (CTF) monitors the consumption of the granted units.

8)	Content/Service Delivery: the NF (CTF) delivers the content/service based on the number of units.

The scenario for Event based charging supported by PEC is described in figure 5.1.2.2.1.1.

Middle Change

#### 5.3.2.3	Session based charging

For Converged Session based Charging, the following cases are supported:

-	SCUR

-	ECUR

Figure 5.3.2.3.1 shows a blocking mode scenario for Session based charging (SCUR) with: Unit Reservation, Decentralized and Centralized Unit Determination, Centralized Rating configuration, user’s account deduction, where the NF (CTF) invokes a converged charging service towards the CHF.

Figure 5.3.2.3.1: SCUR - Session based charging with Decentralized and Centralized Unit Determination, Centralized Rating

1)	Request for service delivery: A request for service is received in the NF (CTF). The service is configured to be authorized by the CHF to start.

2)	Units Determination: the NF (CTF) determines the number of units depending on the service requested by the UE in "Decentralized Units determination" scenario.

3)	Charging Data Request [Initial, Quota Requested]: The NF (CTF) sends the request to the CHF for the service to be granted authorization to start, and to reserve the number of units if determined in item 2.

4)	Account, Rating, Reservation Control: the CHF rates the requests either based on the number of units requested or on internal unit determination, checks if corresponding funds can be reserved on the user's account balance. If the account has sufficient funds, the CHF performs the corresponding reservations.

5)	Open CDR: based on policies, the CHF opens a CDR related to the service.

6)	Charging Data Response [Initial, Quota Granted]: The CHF grants authorization to NF (CTF) for the service to start, with the reserved number of units.

7)	Granted Units Supervision: the NF (CTF) monitors the consumption of the granted units.

8)	Start of service delivery: the NF (CTF) starts to deliver the service based on the reserved number of units.

9)	Usage Reporting Trigger: the NF (CTF) generates charging data related to the service delivered that is not under quota management, based on a trigger for usage reporting is met.

10)	Charging Data Request [Update, Unit Used]: the NF (CTF) sends the request for reporting the related charging data, including the used units, to the CHF.

11)	Account, Rating Control: The CHF performs the reported usage process involving rating entity and user's account balance.

12)	 Update CDR: based on policies, the CHF updates the CDR with charging data related to the service.

13)	Charging Data Response [Update]: The CHF informs the NF (CTF) on the result of the request.

14)	Quota management Trigger: A trigger associated to quota management is met. Units determination is performed when applicable.

15)	Charging Data Request [Update, Unit Used, Quota Requested]: the NF (CTF) sends the request to the CHF, for more units to be granted for the service to continue, and reporting the used units. For the same rating group, the NF (CTF) can only send the next request for units after receiving the previous response from the CHF or initiate failure handling.

16)	Account, Rating, Reservation Control: The CHF performs the process related to the reported usage and the requested reservation, involving rating and user's account balance.

17)	 Update CDR: based on policies, the CHF updates the CDR with charging data related to the service.

18)	Charging Data Response [Update, Quota Granted]: The CHF grants quota to NF (CTF) for the service to continue, with the reserved number of units.

19)	Service Delivery: the NF (CTF) delivers the service based on the granted quota.

20)	Session released: the session is released.

21)	Charging Data Request [Termination, Unit Used]: the NF (CTF) sends the request to the CHF, for charging data related to the service termination with the final consumed units.

22)	Account, Rating Control: The CHF performs the service termination process involving rating and user's account balance.

23)	 Close CDR: based on policies, the CHF closes the CDR with charging data related to the service termination and the last reported units.

24)	Charging Data Response [Termination]: The CHF informs the NF (CTF) on the result of the request.

Figure 5.3.2.3.2 shows a Non-blocking mode scenario for Session based charging (SCUR) with: Unit Reservation, Decentralized and Centralized Unit Determination, Centralized Rating configuration, user’s account deduction, where the NF (CTF) invokes a converged charging service towards the CHF.

NF (CTF) may use blocking mode instead when risk of quota overdraft is more important than latency. CHF is unaware of the blocking mode status and performs the quota management independent of the blocking mode status.

Figure 5.3.2.3.2: SCUR - Session based charging with Decentralized and Centralized Unit Determination, Centralized Rating, immediate start of service delivery (Non-blocking mode)

1)	Request for service delivery and start of service delivery: A request for service is received in the NF (CTF). The NF (CTF) is configured to allow the service to be delivered.

2)	Units Determination: the NF (CTF) determines the number of units depending on the service requested, in "Decentralized Units determination" scenario.

3)	Charging Data Request [Initial, Quota Requested]: the NF (CTF) sends the request to the CHF to reserve the number of units for a rating group of the service if determined in step 2.

4)	Account, Rating, Reservation Control: the CHF rates the requests either based on the number of units requested or on internal unit determination, checks if corresponding funds can be reserved on the user's account balance. If the account has sufficient funds, the CHF performs the corresponding reservation.

5)	 Open CDR: based on policies, the CHF opens a CDR related to the service.

6)	Charging Data Response [Initial, Quota Granted]: the CHF grants the reserved number of units to NF (CTF).

7)	Granted Units Supervision: The NF (CTF) monitors the consumption of the granted units.

8)	Service delivery ongoing: the NF (CTF) continues to deliver the service.

9)	Usage reporting trigger: the NF (CTF) generates charging data related to a service delivered that is not under quota management, based on that a trigger for service usage reporting is met.

10)		Charging Data Request [Update, Unit Used]: the NF (CTF) reports the charging data related to service delivered, including the used units, to the CHF.

11)		Account, Rating Control: the CHF uses the reported charging data to rate the usage and deduct the funds corresponding to the usage on the account balance.

12)		Update CDR: based on policies, the CHF updates the CDR with charging data related to the service.

13)		Charging Data Response [Update]: The CHF informs the NF (CTF) on the result of the request.

14	)	Quota management Trigger: A trigger associated to quota management is met. Units determination is performed when applicable.

15)		Charging Data Request [Update, Unit Used, Quota Requested]: the NF (CTF) sends the request to the CHF, for more units to be granted for the service to continue, and reporting the used units.

16)		Account, Rating, Reservation Control: same as step 4, with the option to also deduct the funds corresponding to the usage on the account balance.

17)		Update CDR: based on policies, the CHF updates the CDR with charging data related to the service.

18)		Charging Data Response [Update, Quota Granted]: The CHF grants quota to NF (CTF) for the service, with the reserved number of units.

19)		Service delivery ongoing: the NF (CTF) continues to deliver the service.

20)		Service release: the NF (CTF) is requested to end the service delivery and does this.

21)		Charging Data Request [Termination, Unit Used]: the NF (CTF) sends the request to the CHF, for charging data related to the service termination with the final consumed units.

22)		Account, Rating Control: the CHF performs the service termination process which involve using the reported charging data to rate the usage and deduct the funds corresponding to the usage on the account balance.

23)		Close CDR: based on policies, the CHF closes the CDR with charging data related to the service termination and the last reported units.

24)		Charging Data Response [Termination]: The CHF informs the NF (CTF)on the result of the request.

Figure 5.3.2.3.3 shows a scenario for Session based charging (ECUR) in Decentralized and Centralized Unit Determination, Centralized Rating configuration, where the NF (CTF) invokes a converged charging service towards the CHF, prior to service delivery if needed.

Figure 5.3.2.3.3: ECUR - Session based charging with - Decentralized and Centralized Unit Determination, Centralized Rating.

1)	Request for resource usage: A request for content/service is received in the NF (CTF). The service is configured to be authorized by the CHF to start.

2)	Units Determination: the NF (CTF) determines the number of units depending on the service requested by the UE in "Decentralized Units determination" scenario.

3)	Charging Data Request [Initial, Quota Requested]: The NF (CTF) sends the request to the CHF for the service to be granted authorization to start, and to reserve the number of units if determined in item 2.

4)	Account, Rating, Reservation Control: the CHF rates the requests either based on the number of units requested or on internal unit determination, checks if corresponding funds can be reserved on the user's account balance. If the account has sufficient funds, the CHF performs the corresponding reservation.

5)	 Open CDR: based on policies, the CHF opens a CDR related to the service.

6)	Charging Data Response [Initial, Quota Granted]: The CHF grants authorization to NF (CTF) for the service to start, with the reserved number of units.

7)	 Granted Units Supervision: The service starts and the NF (CTF) monitors the consumption of the granted units.

8)	Content/Service Delivery: the NF (CTF) delivers the content/service based on the reserved number of units.

9)	Charging Data Request [Termination, Unit Used]: the NF (CTF) sends the request to the CHF, for charging data related to the delivered service with the consumed units.

10)		Account, Rating Control: The CHF performs the process for the delivered service involving rating and user's account balance.

11)		Close CDR: based on policies, the CHF closes the CDR with charging data related to the delivered service.

12)		Charging Data Response [Termination]: The CHF informs the NF (CTF) on the result of the request.

Middle Change

#### 5.3.2.4	Charging notification

The CHF can provide notifications to the NF (CTF), the NF (CTF) implicitly subscribes to these when it sends a Charging Data Request [Initial], i.e. there is no separate subscription request from the NF for notification.

Figure 5.3.2.4-1 shows a scenario for Session based charging with a notification from the CHF triggering a Charging Data Request [Update].

Figure 5.3.2.4.1: Session based charging – Notification with Re-authorization

1)	Session based charging ongoing: there is a session based charging ongoing and there have at least been a Charging Data Request [Initial] sent from the NF (CTF) to the CHF, and the CHF have opened a CDR.

2)	Event triggering notification: an event is detected in the CHF that requires a notification to be sent to the NF (CTF). In this scenario a request for triggering a Charging Data Request [Update, Quota Requested ] is sent, but also a request for Charging Data Request [Update] (without request for quota) is possible.

3)	Charging Notify Request [Re-authorization]: the CHF sends the request to the NF (CTF), for a triggering of a Charging Data Request [Update, Quota Requested] i.e. Re-authorization.

4)	Charging Notify Response: the NF (CTF) acknowledges the request by sending a response.

5)	Charging Data Request [Update, Quota Requested]: the NF (CTF) sends the request to the CHF, to be granted with more unit for the service to continue, and also for reporting the used units.

6)	Account, Rating, Reservation Control: the CHF performs the process related to the reported usage and the requested reservation, involving rating and user's account balance.

7)	 Update CDR: based on policies, the CHF updates the CDR with charging data related to the service.

8)	Charging Data Response [Update, Quota Granted]: the CHF grants quota to NF (CTF) for the service to continue, with the reserved number of units.

Figure 5.3.2.4.2 shows a scenario for Session based charging with a notification from the CHF triggering a Charging Data Request [Termination].

Figure 5.3.2.4.2: Session based charging – Notification with termination

1)	Session based charging ongoing: there is a session based charging ongoing and there have at least been a Charging Data Request [Initial] sent from the NF (CTF) to the CHF, and the CHF have opened a CDR.

2)	Event triggering notification: an event is detected in the CHF that requires a notification to be sent to the NF (CTF). In this scenario a request for triggering a Charging Data Request [Termination] is sent.

3)	Charging Notify Request [Termination]: the CHF sends the request to the NF (CTF), for a triggering of a Charging Data Request [Termination] i.e. the termination of the charging session.

4)	Charging Notify Response: the NF (CTF) acknowledges the request by sending a response.

5)	Charging Data Request [Termination]: the NF (CTF) sends the request to the CHF, for charging data related to the service termination with the final consumed units.

6)	Account, Rating Control: the CHF performs the process related to the reported usage, involving rating and user's account balance.

7)	Close CDR: based on policies, the CHF closes the CDR with charging data related to the service.

8)	Charging Data Response [Termination]: The CHF informs the NF (CTF) on the result of the request.

Middle Change

#### 5.3.2.5	Switch between quota managed and not quota managed

When converged charging is used for a service delivery it is possible to in online charging to switch from quota management to quota management suspended, and in some cases back again.

Figure 5.3.2.5.1 shows a scenario for Session based charging (SCUR) with a suspension of quota management and resume of quota management.

Figure 5.3.2.5.1: SCUR - Session based charging with suspend and resume of quota management.

1)	Request for resource usage: A request for service is received in the NF (CTF). The service is configured to be authorized by the CHF to start.

2)	Units Determination: the NF (CTF)) determines the number of units depending on the service requested by the UE in "Decentralized Units determination" scenario.

3)	Charging Data Request [Initial, Quota Requested]: The NF (CTF) sends the request to the CHF for the service to be granted authorization to start, and to reserve the number of units if determined in item 2.

4)	Account, Rating, Reservation Control: the CHF rates the requests and checks need for quota management. If not needed for the service at the moment a switch from online to offline type of charging is to be performed.

5)	 Open CDR: based on policies, the CHF opens a CDR related to the service.

6)	Charging Data Response [Initial, Result Code]: The CHF grants authorization to NF (CTF) for the service to start, with a result code indicating that quota management is suspended.

7)	Service Delivery: the NF (CTF) delivers the service without quota management.

8)	Usage Reporting Trigger: the NF (CTF) generates charging data related to the service delivered that is not under quota management, based on a trigger for usage reporting is met.

9)	Charging Data Request [Update, Unit Used, Quota Requested]: the NF (CTF) sends the request to the CHF, for units to be granted making it possible to resume the quota management. It also reports the used units with an indication that these were used with quota management suspended.

10)		Account, Rating, Reservation Control: The CHF performs the process related to the reported usage and checks if quota management should continue to be suspended or should be resumed. If needed for the service, CHF checks if corresponding funds can be reserved on the user's account balance.

11)		Update CDR: based on policies, the CHF updates the CDR with charging data related to the service.

12)		Charging Data Response [Update, Quota Granted]: The CHF grants quota to NF (CTF) for the service to continue and with this indicating that quota management is to be resumed, with the reserved number of units.

13)		Service Delivery: the NF (CTF) delivers the service based on the granted quota.

14)		Session released: the session is released.

15)		Charging Data Request [Termination]: the NF (CTF) sends the request to the CHF, for charging data related to the service termination with the final consumed units.

16)		Account, Rating Control: The CHF performs the service termination process involving rating and user's account balance.

17)	Close CDR: based on policies, the CHF closes the CDR with charging data related to the service termination and the last reported units.

18)		Charging Data Response [Termination]: The CHF informs the NF (CTF) on the result of the request.

Middle Change

#### 5.4.8.2	Quota management for inter CHFs

For the communication between consumer CHF and producer CHF, converged charging is supported.

Upon receiving Charging Data Request [Initial/Update] from the NF Consumer (CTF) with quota management, consumer CHF may:

-	send the Charging Data Request [Initial/Update] to producer CHF with or without the amount of quota, dependent on NF consumer requested, or

-	send the Charging Data Request [Initial/Update] to producer CHF with the amount of quota larger than the amount from NF Consumer (CTF), or

-	allocate the quota based on previous granted quota from the producer CHF, dependent on whether the consumer CHF has available quota.

When receiving Charging Data Response [Initial/Update] with granted quota from producer CHF, consumer CHF may send the quota to NF Consumer (CTF) with the same amount or less as granted by producer CHF.

Middle Change

### 5.5.2	Retry handling

In case a NF consumer (CTF) does not receive a Charging Data Response, it may retransmit the Charging Data Request message. The number of retries and delay between retries shall be locally configured in the NF consumer (CTF).

If the retried charging data request [Initial] is received by the same CHF, the uniqueness checking may be based on the Charging Identifier included in the charging data request. The CHF shall respond to the retried charging data request [Initial] with the original charging session identifier.

If the retried request is charging data request [Update] or charging data request [Termination], the uniqueness checking may based on the inspection of the Charging Session Identifier and Invocation Sequence Number pair.

The retried message shall have the same Invocation Sequence Number as the original of the retried message i.e. the Invocation Sequence Number shall not be incremented when the message is retried. The NF consumer (CTF) may send the retried message to an alternative CHF if the Session Failover indication is received from the CHF. The alternative CHF can be built as defined in clause 6.3.11 of 3GPP TS 23.501 [201].

In the case of a notification request time out the CHF may retry the message. The number of retries and delay between retries shall be locally configured in the CHF.

End of Change