3GPP TSG-SA5 Meeting #163	S5-255105

# Wuhan, China, 13 – 17 October 2025

# Source:	Nokia

# Title:	Discussion paper on scope of data WT in 6G Study

# Document for:	Endorsement

Agenda Item:	6.2.2

# 1	Decision/action requested

The group is requested to discuss and endorse the proposals below.

# 2	References

[1]	S5-254905 Study on 6G Management and Orchestration.

# 3	Rationale

## 3.1	Introduction

SA5 has endorsed document S5-254905 [1] as the baseline for the 6G study. Therein these three (3) options are listed for text on the scope of the study regarding WT 1.6 on data management:

- 1.6-option1.	Study the data management framework for different types of data, and coordinate with SA2 if necessary.

  - Note: the scope of different types of data are to be part of the study, including access control of the data.

- 1.6-option2a. Study the data management framework for different types of management data, and coordinate with SA2 if necessary.

- 1.6-option3a. Study the data management framework for different types of data to be used for network management and service management, and coordinate with SA2 if necessary.

- 1.6-option5: Study the data management framework, and coordinate with SA2 if necessary.

- 1.6-option5a: Study the data management framework, and coordinate with SA2 if necessary.

  - Note: the scope of data is to be part of the study.

It needs to be decided what the best option is for the study description that adequately captures the scope to be considered by SA5.

## 3.2	Required data flows:

There are several data flows expected within the 3GPP 6G network as illustrated by Figure 3.2-1. SA5 is expected to contribute to several of these data flows.

Figure 3.2-1: Potential data flows expected within the 3GPP  6G network and likely SA5 involvement.

The data flows in figure 3.2-1 illustrate that:

- Several kinds of data from different sources needs to be considered

- Several destinations for the different data need to be considered

Moreover, there may be several use cases which may have a similar flow while there can be use cases which require more than 1 flow.

## 3.3	Justifications for data management features to be studied

- Data capabilities: Data is an enabler and a key pre-requisite for many management and orchestration functionalities such as automation, optimisation, and service assurance. 3GPP SA5 has standardized methods related to data collection and processing for different kinds of management data. In 5G, the related methods and information models are scattered over several specifications. In 6G, it should be studied how a unified data handling framework may be realized, where applicable re-using the features developed in 5G.

- Data, Information, Knowledge framework: in 5G different terminologies and mechanism are used for different kinds of data, e.g., for PM data, trace data, UE data, etc. other aspects, especially information and knowledge, may also require related terminology and mechanisms, e.g. operations. It should be studied whether a common data and information framework can be developed and applied (similar to the configuration management framework with CRUD operations and NRMs). Specifically, the following should be studied:

  - Common terminology; relationships between data; versioning; etc.

  - Common data properties, including for example data name, data name identifier; data value type (like integer or some structured type), data value type identifier; data group;

  - Common data-set properties, e.g., including completeness collection time, deletion time, granularity period; scope; collector and software version of the collector; owner, sharing permissions; accuracy; links to context data

  - Common data and data-set representations;

- Common approach for data and analytics: The existing capabilities may be differently characterized as either Data, information or Knowledge and any of the different types of analytics. For example:

  - Performance measurements and KPIs can be either data, information or knowledge

  - An error (as defined in TS 28.111 e.g. by attribute alarmType) can be an example of a descriptive analytic; a fault or a root cause / root fault (as defined in TS 28.111 e.g. by attribute probableCause) can be an example of a diagnostic analytic while an example of a prescriptive analytic could a proposed action (as defined in TS 28.111 e.g. by attribute proposedRepairAction).

  - TS 28.104 contains analytics with all levels, but they are not separated but all contained in a big basket that does not provide differentiation on how they may be used.

In 5G, all analytics at the different levels (e.g. diagnostics vs. prescriptive) are mixed and it is not clear if this is a good design. Moreover, although analytics and data share properties, they are differently handled. It should be studied if a common data handling framework can be applied to analytics. The study may consider whether to differently model analytics depending on its type and whether it is needed to rebuild TS 28.104 from scratch.

- Data to be collected and managed: in 5G several PM, KPI and CM data is already defined. New data types may need to be defined in 6G including in RAN and core network specifications. It should be studied which new data or analytics needs to be defined and introduced.

## 3.5	Analysis on required features and recommendation

The data flows and the justifications of features to be studied indicate that SA5 needs to consider several data flows, data sources and data destinations to support all use cases related to management. However, SA5 does not take full responsibility for all data sources within the network - there can be data flows that are not the responsibility of SA5. The justifications also show that the usage goes beyond network and service management to include requirements on automation and exposure.

Proposal 1:	It is recommended as such to adopt a variation of options 2 or 3 which allow for data needed for different  automation and exposure requirements. The proposals are

- Simply refer to SA5 scope but show (through a note) that SA5 does not interfere with the responsibilities of other WGs, i.e.

- 1.6-option1b.       Study the data management framework for different types of data within SA5 scope, and coordinate with SA2 if necessary.

Note: Data-type definitions for each WG lies within their respective scopes. SA5 can reuse data defined by other work groups as needed.

- Use a reference to data that clearly shows it is in management scope but does not limit it to only few data defined in existing specifications, i.e.

- 1.6-option2b. Study the data management framework for different types of management-and-service data, and coordinate with SA2 if necessary.

- Reuse the term "management data" but with a note showing that it is not limited to the previous usage of the phrase i.e.

- 1.6-option2c : Study the data management framework for different types of data to be used for network and service management and exposure.

Note: In the context of 6G work, the definition of "management data" in TS28.537 does not apply

- Use a description for the scope that the data shall be used for management and exposure , i.e.,

- 1.6-option3b. Study the data management framework for different types of data to be used for network and service management and exposure, and coordinate with SA2 if necessary.

# 4	Detailed proposal

The group is asked to endorse the following proposals:

Proposal 1:	For the work task description in 6G Rel-20 study, use any of the following the text:

- 1.6-option1b.       Study the data management framework for different types of data within SA5 scope, and coordinate with SA2 if necessary.

Note: Data-type definitions for each WG lies within their respective scopes. SA5 can reuse data defined by other work groups as needed.

- 1.6-option2b. Study the data management framework for different types of management-and-service data, and coordinate with SA2 if necessary.

- 1.6-option2c: Study the data management framework for different types of management data, and coordinate with SA2 if necessary.

Note: In the context of 6G work, the definition of "management data" in TS28.537 does not apply.

- 1.6-option3b. Study the data management framework for different types of data to be used for network and service management and exposure, and coordinate with SA2 if necessary.