3GPP TSG-SA5 Meeting #164	S5-255175

Dallas, USA, 17 - 21 November 2025

# Source:	Huawei

# Title:	Rel-20 pCR 32.872 Introduce the usecase3 on Local Breakout Inter CHFs scenario

# Document for:	Approval

Agenda Item:	7.5.2

1	Decision/action requested

This is a pCR to TR 32.872 Introduce a usecase on Local Breakout Inter CHFs scenario.

2	References

[1]	3GPP TR 32.872: "Study on 5GA roaming charging reliability enhancement".

3	Rationale

This pCR proposes to introduce a new use case on the topic of Local Breakout Inter CHFs scenario.

4	Detailed proposal

Propose to incorporate the following change into the TR 32.872 [1]

First change

5.1.1.x	Use Case #3: One of the inter-CHF charging sessions terminates

In the LBO N40+N107 charging architecture, for a specific PDU session, there is a charging session between the V-SMF(CTF) and V-CHF, and there is also a charging session between the V-CHF and H-CHF. When the charging session between the V-CHF and H-CHF is terminated due to the expiry of the Unit Count Inactivity Timer, but the charging session between the V-SMF(CTF) and V-CHF remains active, and vice versa. How the charging system handles this should be studied.

Figure 5.1.1.x-1: An example of Unit Count Inactivity Timer unalignment

### The potential charging requirements for this UC are: REQ-3GPPCH-LBIC-03.

5.1.2	Potential charging requirements

REQ-3GPPCH-LBIC-03: Charging system shall support handling the association of two charging sessions.

5.x.3	Key issues

5.1.3.x	Key issue #3: Handling the association of two charging sessions

This key issue is for investigating how to support REQ-3GPPCH-LBIC-03. This investigation covers the following:

-	Identification of the handling the association of two charging sessions in LBO N40+N107 scenario;

End of change