3GPP TSG-SA5 Meeting #164	S5-255214

Dallas, USA, 17 - 21 November 2025

Source:	Huawei

Title:	pCR TR 28.884 Update use case on management data streaming based on message bus

Document for:	Approval

Agenda item:	6.20.4

Spec:	3GPP TR 28.884

Version:	0.1.0

Work Item:	FS_SBMA_Ph4

Comments

This contribution proposes to update use case on management data streaming based on message bus in SBMA for 5G.

Proposed Changes

* * * First Change * * * *

## 5.2	Use case #2: Management data streaming based on Message Bus

### 5.2.1	Description

The 5G network employs a distributed, service-oriented architecture, leading to scenarios where multiple consumers request the same management data for multiple uses. In a typical network performance data reporting scenario, an MnS producer collects and transmits performance data from network elements (e.g., gNB) to multiple MnS consumers within the network management system (e.g., real-time analytics, network optimization engines, visualization dashboards, and audit services).

The WebSocket protocol is standardized by the IETF in RFC 6455 [f]. It defines WebSocket as a protocol that enables ongoing, full-duplex, bidirectional communication between web servers and web clients over an underlying TCP connection, see [g].

Using point-to-point WebSocket communication protocol, which operates over a single TCP connection between a client and a server, a separate connection would be established between each consumer and the producer. For the case of multiple consumers request the same performance data, it causes the producer to retrieve and duplicate the relevant performance data from its cache for each connection individually. This process consumes significant CPU and memory resources of the producer and may block or delay other critical processing tasks. Moreover, the number of WebSocket connections is limited, making it difficult to handle scalability issues.

By adopting a message bus architecture, the producer establishes a single, persistent connection with the message bus and publishes the performance data only once. The message bus retrieves the appropriate data and streams it directly to the requesting consumer without involving the producer. No additional connections are initiated by the producer, and no data duplication occurs on its side for the case that multiple consumers request the same performance data. Also, there is no strict limit on the number of consumers that connect to the message bus, supporting reliable large-scale scalability. Figure 5.2.1-x is a comparison example between WebSocket and Message Bus for the case that multiple consumers request the same performance data. In this example:

- for the (a) WebSocket option, data duplication occurs N times on the producer's side for sending the data to the N consumers who request the same performance data.

- for the (b) Message Bus option, no data duplication occurs on the producer's side for sending the data to the N consumers who request the same performance data because the only one copy of data buffered on the message bus broker (i.e., a middleware that facilitates communication between different participants by acting as an intermediary for messages) can be polled by all the consumers.

Figure 5.2.1-x: Comparison example between WebSocket and Message Bus for the case that multiple consumers request the same performance data

While the message bus architecture introduces certain operational considerations, such as the need for proper message category segmentation and consumer coordination management to avoid throughput bottlenecks. These aspects are addressed through established internal mechanisms and configuration practices. Such implementation details are outside the scope of this document.

* * * End of Changes * * * *