3GPP TSG-SA5 Meeting #164	S5-255241

Dallas, USA, 17 - 21 November 2025

# Source:	Vodafone

# Title:	Discussion on 6G Data Framework

# Document for:	Discussion and Endorsement

Agenda Item:	6.2.1

# 1	Decision/action requested

SA5 is invited to discuss and endorse the following proposals:

Proposal 1: SA5 to agree that the Data Framework is included in the 6G Study.

Proposal 2: SA5 shall consider that the Data Framework is primarily intended for management and operational data, while other categories should be handled through separate interfaces.

Proposal 3: SA5 to ensure the Data Framework specification allows alternative implementations, including separate interfaces per data category, enabling vendors and operators to adopt solutions aligned with their operational models.

# 2	References

- 3GPP S5-254905 – Study on 6G Management and Orchestration

- 3GPP S5-254335 – LS on Guidance on 6G Data-Related Work Tasks

# 3	Rationale

Different types of data in 6G networks request unique handling and security requirements. Additionally, they exhibit distinct performance characteristics that must be considered independently.

In particular, management data requires dedicated lifecycle handling, access control mechanisms, and reliability guarantees. These requirements justify the use of separate interfaces and data segregation from other categories of data.

Furthermore, a fully data-agnostic framework risks blurring functional boundaries, which can lead to operational inefficiencies, complicate network management, increase exposure to potential security risks by combining unrelated data domains, imply complex and costly implementations and create scalability challenges.

Therefore, the Data Framework should either be limited to management data or, at a minimum, provide clear separation and dedicated interfaces for different categories of data. This approach ensures secure handling and scalable deployment options for both operators and vendors.

# 4	Detailed proposal

## 4.1 Background

This discussion arises from the pending item in the 6G Study on Management and Orchestration [1], where several options were proposed regarding the scope of the Data Framework study. The options included studying all types of data, only management data, or data specifically used for network and service management.

Since this topic has been pending since August 2025, the SA plenary issued a Liaison Statement [2] in September 2025 to SA2, SA5, and other relevant WGs, requesting progress and coordination on 6G data-related work tasks, using the 5G working scope as a starting point. The statement emphasizes collaboration on data security and privacy, as well as regular coordination to avoid duplicated designs and ensure system-wide consistency.

Given this context, it is critical to reach agreement in this meeting on the scope and focus of the Data Framework. Achieving consensus now will allow SA5 to progress the study and submit results to the December 2025 plenary.

This discussion paper further develops the rationale for treating management and operational data as a separate domain, emphasizing the need for dedicated handling, security, and interface separation from other data types.

## 4.2 Discussion on the scope of the 6G Study

The first point on which SA5 should reach agreement as a group in this meeting is the inclusion of some content in the 1.6 bullet of the 6G Study. Studying the Data Framework is essential to ensure coherent treatment of data-related aspects across 6G management and orchestration activities. Excluding this topic from the study would be counterproductive and could delay the development of consistent and interoperable management solutions.

Observation 1: The scope of the Data Framework must be included in the 6G Study. Excluding it would hamper coordinated progress, and the group should at least agree to include it as a dedicated section.

While there may be different views on which data types should be covered, it is critical that the study explicitly includes a dedicated section on the Data Framework. Even if full consensus on the exact data types may not be reached in this meeting, agreeing to include the topic in the 6G Study ensures that its scope is formally recognized, that SA5 can start structured discussions and align with other WGs in a timely manner.

Observation 2: If the data categorization is not agreed in this meeting, it should be included as part of the scope of the study.

Proposal 1: SA5 to agree that the Data Framework is included in the 6G Study.

Note that this proposal is compatible with 1.6-option5 and 1.6-option5a [1].

## 4.3 Discussion on Data Separation

In 6G networks, different types of data have distinct purposes, lifecycles, sensitivity levels, and performance requirements, among others. Treating all data uniformly can lead to inefficiencies, security risks, and reduced performance. This section discusses two approaches to designing the Data Framework.

### 4.3.1 Data-agnostic approach

This is one of the options proposed for the 6G Study, advocating for a single, data-agnostic framework encompassing all data types.

At first glance it may appear to offer implementation simplicity and potential cost reduction, although in reality consolidating heterogeneous data within a common framework introduces significant complexity. It requires sophisticated prioritization, queuing, and security mechanisms, increases exposure and auditing challenges, and can lead to unpredictable performance. Moreover, different data categories vary greatly in volume and latency characteristics, with some being massive and time-sensitive while others are control-oriented and low-volume. Combining them within a single framework risks resource contention, degraded management performance, and reduced overall efficiency.

Observation 3: Treating all data types uniformly within a single framework compromises operational efficiency, scalability and security governance.

### 4.3.2 Management and operational data as a distinct domain

Treating management and operational data as a distinct domain with dedicated handling ensures that each data type operates independently with its own processes, interfaces, and resources. Access is restricted to only those entities or processes that truly require it, combining functional separation with enforcement of least privilege. This approach minimizes interference, unauthorized access, and operational inefficiencies, while supporting predictable performance and robust governance.

Observation 4: Management and operational data have specific purposes, lifecycles, and sensitivity levels that differ fundamentally from other types of data.

### 4.3.3 Comparison of both options

The two approaches differ significantly. The following table summarizes the main characteristics and implications of each approach.

| Characteristic | Data-Agnostic Framework | Management and Operational Data Separation |
| --- | --- | --- |
| Access Control and Security | Complex multi-layer access control across mixed data domains, increasing exposure and risk. | Enforces isolation and least privilege, limiting access to authorized management entities. |
| Handling and Processing | Requires heterogeneous processing logic, increasing prioritization and classification complexity. | Enables dedicated processing optimized for management workflows. |
| Performance | Shared resources may cause contention and latency variation. Potential high criticality data loss due to network congestion. | Independent handling ensures stable and predictable performance. |
| Governance and Lifecycle Management | Difficult to harmonize policies across heterogeneous domains. | Domain-specific policies aligned with management frameworks, simplifying governance and auditing. |
| Scalability | Scaling must accommodate all data types, risking over- or under-provisioning. | Each domain scales independently according to its needs. |
| Implementation | Initially simpler conceptually but complex in prioritization and queuing. | Requires initial interface definition but simplifies long-term operation. |
| Cost and Operational Efficiency | Lower initial cost but higher ongoing management overhead. | Slightly higher initial investment, offset by operational efficiency. |

This comparison reflects that treating management and operational data as a distinct domain provides clear benefits in terms of functional separation, enforcement of least privilege, operational efficiency, security, and predictable performance.

Proposal 2: SA5 shall consider that the Data Framework is primarily intended for management and operational data, while other categories should be handled through separate interfaces.

Note that this proposal is compatible with 1.6-option2a [1].

Note that this proposal is not compatible with 1.6-option1 nor 1.6-option3a [1].

## 4.4 Discussion on data categories

The ongoing discussion in SA5 focuses on the alternative approaches explained in section 4.3 to design the Data Framework. Although this topic may belong to a broader scope of the 6G Study itself, narrowing the focus and clarifying how the Data Framework should handle each data category would help avoid unnecessary complexity and enable more effective progress.

A good starting point is therefore to analyse the different categories of data involved in 6G networks, as understanding their specific characteristics helps to identify where common handling may be possible and where clear separation is required.

| Data Category | Purpose | Typical Characteristics | Key Requirements | Rationale for Separation |
| --- | --- | --- | --- | --- |
| Management and Operational Data | Network configuration, monitoring, orchestration, and control. | Low volume, high criticality, continuous lifecycle. | High reliability, strict access control, full traceability. | Requires secure and predictable handling; must not be affected by traffic or data from other domains. |
| Control Plane Data | Network signalling (e.g., session setup, mobility, authentication). | Moderate volume, latency-sensitive, time-critical. | Deterministic latency, integrity, and coordination with control functions. | Security and timing constraints differ from management and user data, justifying dedicated interfaces. |
| User Plane Data | User traffic (voice, video, IoT payloads, application data). | Very high volume, highly variable, throughput oriented. | High capacity, low latency, minimal control overhead. | Should be isolated from management data to avoid contention and exposure of user traffic. |
| Sensing / Environmental / Contextual Data | Data from radio sensing, environmental awareness, or localization features. | Massive and often real-time, bursty, and time sensitive. | High bandwidth, fast processing, minimal interference. | Mixing with management data could overload shared resources and degrade network control performance. |
| Analytics and AI/ML Data | Data collection for analytics, optimization, or machine learning. | High volume (historical or batch), not real-time. | Scalable storage, flexible access, and privacy protection. | Requires decoupled handling and long-term storage mechanisms separate from operational systems. |
| Service / Application Data | Information exchanged between service management functions or exposed to verticals. | Moderate volume, multi-domain (operator–customer, network–application). | Policy control, SLA assurance, auditable access. | Distinct governance and exposure models demand separated policies and data interfaces. |

Observation 5: Each category of data presents different requirements that cannot be efficiently or securely managed through a single interface.

## 4.5. Implementation approaches

To accommodate the different options under consideration, alternative approaches for implementing the Framework can be evaluated. Two possible approaches are outlined below:

Unified Framework with Multiple Interfaces:

- A single, integrated framework supports multiple interfaces, each tailored with dedicated protections, policies, and processing rules according to the specific data types.

- This approach allows reuse of common infrastructure and coordination mechanisms, while still maintaining separation and security at the interface level.

- Operators and vendors can implement different protection levels per interface without creating entirely separate frameworks.

Observation 6: A unified framework with multiple interfaces allows differentiated protections and policies per data type while maintaining integration within a single system.

Distinct Frameworks per Data Domain:

- Separate frameworks are implemented for each data domain, where each framework corresponds to a single interface and is fully isolated from others.

- This provides maximum functional separation and dedicated handling for management and operational data, minimizing interference, simplifying governance, and improving predictability of performance.

- Each framework can scale independently and adopt domain-specific policies without being affected by the characteristics or requirements of other data categories.

Observation 7: Implementing distinct frameworks per data domain provides full isolation and dedicated handling for management and operational data, enabling independent scaling and domain-specific policies.

Both approaches are technically viable, and the choice can be guided by operator requirements and vendor implementation considerations. This decision may be in the scope of the study.

Proposal 3: SA5 to ensure the Data Framework specification allows alternative implementations, including separate interfaces per data category, enabling vendors and operators to adopt solutions aligned with their operational models.

## 4.6. Conclusion

The discussion above highlights the challenges and considerations in designing the 6G Data Framework, including the distinct requirements of different data types and the trade-offs between a data-agnostic approach and a domain-separated approach.

Given the critical importance of addressing management and operational data in a coordinated way, the minimal achievable agreement is to ensure that the Data Framework is formally included in the 6G Study. This allows SA5 to progress the work while retaining the flexibility to finalize decisions on data categorization within the study.

However, Vodafone considers that defining a narrower scope from the outset would help avoid unnecessary debate and enable more effective progress. For this reason, SA5 is encouraged to adjust the 6G Study content accordingly, emphasizing the primary focus on management and operational data.

Vodafone kindly ask to take this analysis into consideration and endorse proposals 1, 2 and 3.